3GPP TSG RAN WG4 #116-bis	R4-2513980

# Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	4.22.3

Source: 	Ericsson

Title:	Remaining issues on LP-WUS RRM requirement

Document for:	Discussion

# Introduction

In last RAN4 meeting, RAN4 successfully completed the core requirements of the LP-WUS/WUR and made the following agreement [1].

### Sub-topic 1-1 General aspects Issue 1-1-14: LP-WUR operation with eDRX Agreement: No RAN4 RRM requirements for LP-WUR operation with eDRX with PTW window in Rel-19. For LP-WUR operation with eDRX without PTW (i.e., for eDRX cycles <=10.24s) No RAN4 RRM requirements Issue 1-1-15 LP-WUR operation with RedCap Agreement: Specify LP-WUR related idle/inactive requirements including requirement on serving cell offloading, RRM relaxation and higher priority frequency layer search for Redcap UE. - Existing requirements for MR offloading, RRM relaxation and higher priority frequency layer search will be reused for Redcap UE - Confirm the MR wake up delay will apply for 2 Rx Redcap Issue 1-1-16: LP-WUR operation with EMR Agreement: When both Rel-16 EMR and Rel-19 LP-WUR are configured: When T331 is running - When Case#1 (RRM offloading) conditions are met: - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s). - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s). - When Case#3 (RRM relaxation) conditions are met: - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s). - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s). - if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ - the UE shall search for NR inter-frequency layers configured and not configured for idle mode CA/DC measurements by following the legacy measurement requirements (no relaxation) when T331 is not running, - Serving cell, intra-frequency and NR inter-frequency layers not configured for idle mode CA/DC measurements and NR inter-frequency layers configured for idle mode CA/DC measurements and mobility measurement follow corresponding agreed requirements defined in case 1 and case 3 when they are satisfied.

### Sub-topic 1-2 Detail LP-WUR requirements at RRC_IDLE/INACTIVE state Issue 1-2-1-1: Detail on LR accuracy and side conditions requirements Agreement: Use 2.5 dB as the RF impairment margin for LP-RSRP accuracy requirements. Remove [] in agreements in RAN4 114bis and the final agreements are as: For FR1: - 3.5 dB is used for core requirements for LP-RSRQ accuracy and 6 dB is used for core requirements for LP-RSRP accuracy, under the side conditions Ês/Iot = -3 dB - 3.5 dB is used for core requirements of SSB based RSRQ accuracy and 6 dB is used for core requirements for SSB based RSRP accuracy, under the side conditions Ês/Iot = -3 dB Issue 1-2-4-2-3: On how to define LR evaluation requirements Agreement: Using x1=2*x and y1=2*y for the evaluation requirement.; y = 2; For x, x= 3; Issue 1-2-9: LP-WUR status at legacy case (not at LP-WUS monitoring case/fully offloading(case 1) case/RRM relaxation (case 3) case) Agreement: - No LR measurement and evaluation requirements apply at the legacy state, i.e., for the following cases: from legacy case to LP-WUR monitoring, from legacy case to RRM measurement fully offloading (case 1), and from legacy case to RRM measurement relaxation (case 3). - At legacy state, when both MR and LR entry thresholds are configured, the UE shall evaluate both MR and LR thresholds at least once before entering relaxation / offloading Issue 1-2-11: RRM requirements for FR2 Agreement: On the SSB based LR FR2 requirement LO periodicity [s] Note 1. FR2 (Scaling factor N1). 0.32; 8. 0.64; 5. 1.28; 4. 2.56; 3. No requirement for the FR2 LP-SS based LR For SSB based FR2 LR, - the target measurement accuracy for FR2 SS-RSRP is 7.5 dB. - the target measurement accuracy for FR2 SS-RSRQ is 3.5dB. For MR-based relaxed serving and neighbour cell measurements in FR2, reuse the measurement relaxation factor of 16 as agreed for FR1. Note: No RF margin for RSRQ accuracy requirement, and 4dB RF margin for FR2 RSRP requirement. Issue 1-2-12: UE behaviro when both Rel-16 relaxation and Rel-19 LP-WUR offloading/relaxation are satisfied; or when both Rel-17 relaxation for Redcap and Rel-19 LP-WUR offloading/relaxation are satisfied Agreement: UE is allowed to follow the most relaxation requirements when only Rel-16 not at cell edge is configured, and Rel-16 not at cell edge and Rel-19 LP-WUR offloading/relaxation are both satisfied and Rel-16 EMR is not configured or Rel-16 EMR is configured and T331 is not running. ### Sub-topic 1-4 LP-WUR CONNECTED mode Issue 1-4-1: LP-WUR at CONNECTED mode Agreement: No RRM impact for connected mode in this WI. Note: This can be revisited if any issue is identified later ### Sub-topic 1-5 Others Issue 1-5-3: MR wake up delay after exiting case 1 due to offloading conditions cannot be met Agreement: When exiting case 1 due to exiting conditions of offloading is met, MR reuse the existing wake up delay.

In this contribution, we continue the discussion on the remaining issues of LP-WUS.

- Remaining issues

Transition period

In Rel-16 RRM relaxation, RAN4 specified the transition rule for UEs moving from cell-edge or low-mobility conditions to a state meeting both non-cell-edge and low-mobility criteria. When UE detects that it fulfils Rel-19 RRM measurement relaxation criteria or offloading crietia, UE will switch between:

- normal measurement,

- no neighbour cell measurement and Rel-19 MR RRM measurement relaxation,

- or offloading.

Especially, when UE exits from Rel-19 offloading mode, there is no MR to evaluate the RRM relaxation mode. UE needs to decide which status it shall enter and evaluate the criteria, RRM relaxation or normal mode when MR wakes up.

Figure 1. Transition status for Rel-19 MR RRM relaxation, offloading

Proposal 1: RAN4 to discuss transition requirement among normal measurement(Rel-15, Rel-16 measurement) and Rel-19 MR RRM measurement relaxation, offloading.

Low mobility

In Rel-16 RRM relaxation, except not in cell edge criteria, RAN4 also defined the low mobility criteria. In Rel-19 LP-WUS, RAN2 is also discussing the low mobility criteria based on LR. From our understanding, whether to introduce LR-based low mobility criteria is up to RAN2. In last meeting, RAN2 made the agreement not to consider the low mobility criteria in Rel-19. However, RAN2 also plan to continue the discussion. Therefore, RAN4 needs to wait RAN2’s final conclusion about whether considering low mobility for Rel-19.

Proposal 2: RAN4 to wait RAN2 to make the final conlusion about low mobility criteria for LP-WUR.

EMR

In last meeting, RAN4 agreed on the LP-WUR behviour with Rel-16 EMR configuration.

Issue 1-1-16: LP-WUR operation with EMR Agreement: When both Rel-16 EMR and Rel-19 LP-WUR are configured: When T331 is running - When Case#1 (RRM offloading) conditions are met: - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s). - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s). - When Case#3 (RRM relaxation) conditions are met: - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s). - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s). - if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ - the UE shall search for NR inter-frequency layers configured and not configured for idle mode CA/DC measurements by following the legacy measurement requirements (no relaxation) when T331 is not running, Serving cell, intra-frequency and NR inter-frequency layers not configured for idle mode CA/DC measurements and NR inter-frequency layers configured for idle mode CA/DC measurements and mobility measurement follow corresponding agreed requirements defined in case 1 and case 3 when they are satisfied.

However, RAN4 hasn’t discussed the UE’s behaviour when NW configures Rel-18 EMR with LP-WUR scenarios. From our understanding, when UE supports the Rel-18 EMR, UE shall follow the legacy measurement requirement to evaluate whether the measurement results are valid or not. The details are captured in the spec. as follows.

A UE supporting measValidationReportEMR-r18 and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report the results of the idle mode measurement which is specified in clause 4.4, and according to the measurement reporting requirements specified in clause 4.7.3. A UE supporting measValidationReportReselectionMeasurements-r18 shall report the results of the idle mode measurement specified in the clause 4.2.2, and according to the measurement reporting requirements specified in the clause 4.7.3.

However, when UE supports Rel-19 LP-WUS capability, the legacy IDLE mode measurement may be changed due to MR offloading. When UE is in RRM offloading mode, the MR will be turned off. In other words, UE cannot perform the measurement evaluation based on MR for EMR. To guarantee legacy EMR behaviour, at least UE shall perform the EMR carriers’ measurement following the higher priority measurement requirement in this scenario.

Proposal 3: For a UE which supports Rel-18 EMR, UE shall perform the configured Rel-18 EMR carriers’ measurement by following the higher priority frequency measurement requirements in MR offloading mode.

RedCap

In last RAN4 meeting, RAN4 agreed to specify LP-WUR requirements for RedCap UE.

Issue 1-1-15 LP-WUR operation with RedCap Agreement: Specify LP-WUR related idle/inactive requirements including requirement on serving cell offloading, RRM relaxation and higher priority frequency layer search for Redcap UE. - Existing requirements for MR offloading, RRM relaxation and higher priority frequency layer search will be reused for Redcap UE - Confirm the MR wake up delay will apply for 2 Rx Redcap

The only remaining issue is the MR wake up delay for 1Rx RedCap UE. From our understanding, the current wake up delay for 2Rx UE has left enough margin. We believe the MR wake up delay can be reused to 1Rx RedCap UE.

Proposal 4: The MR wake up delay for 1Rx RedCap UE is the same as 2Rx RedCap UE.

- Summary

In this contribution, we discussed the remaining issues on LP-WUS operation. Based on the discussions, we have made the following proposals and observations:

Proposal 1: RAN4 to discuss transition requirement among normal measurement(Rel-15, Rel-16 measurement) and Rel-19 MR RRM measurement relaxation, offloading.

Proposal 2: RAN4 to wait RAN2 to make the final conlusion about low mobility criteria for LP-WUR.

Proposal 3: For a UE which supports Rel-18 EMR, UE shall perform the configured Rel-18 EMR carriers’ measurement by following the higher priority frequency measurement requirements in MR offloading mode.

Proposal 4: The MR wake up delay for 1Rx RedCap UE is the same as 2Rx RedCap UE.

- References

[1]. R4-2508280, “WF for RRM requirements for LP-WUS/WUR”, vivo

[2]. RP-240773, “New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)”, vivo, NTT DOCOMO, Ericsson, MediaTek, Samsung, Sony