3GPP TSG RAN WG4 #116-bis	R4-2513982

# Prague, Czech Republic, 13 October – 17 October 2025

Agenda Item:	6.15.2.1

Source: 	Ericsson

Title:	Discussion on performance requirements for OD-SSB

Document for:	Discussion

# Introduction

In last meeting, there was one initial agreement about OD-SSB test case as follow.

Issue 3-1-4: Test case list Agreement RAN4 to at least define the following OD-SSB test cases. - SCell activation in Case 1 - Intra-frequency measurement(deactivated SCell) in Case 1 - L1-RSRP measurement with OD-SSB parameter update(Scenario 3B)

In this contribution, we’ll discuss the detail test case design and the related test case list[1].

# Test Case Design

General

In last meeting, RAN4 listed the following aspects need to be discussed for OD-SSB test case design.

Issue 3-1-1: Scenario - General - RAN4 to introduce test to verify FMW for deactivated Scell measurement and corresponding interruption requirement. - No new test case for A.7.7 Measurement Performance requirements for NES OD-SSB. - RAN4 should not define fast SCell activation test cases for OD-SSB. - RAN4 only choose one test case for case 2 to justify the effective periodicity of OD-SSB and AO-SSB. - RAN4 to further discuss the test case scope for scenario #2, scenario #2A and scenario #3B as follow. - Option 1: - In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A. - In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B. - The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell. - Option 2: RAN4 not to define test cases for scenario #3B. - Option 3: RAN4 to define the OD-SSB based test cases for - Case #1 and Scenario #2 - Case #2 and Scenario #2A - RAN4 to further discuss the test case scope for Case 2 as follow. FFS: RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if RAN4 agrees to define the RRM requirements for all these scenarios.

When OD-SSB feature introduced in Rel-19 NES WI, two different cases are defined for OD-SSB transmission: Case 1 – without AO-SSB; Case 2 – with AO-SSB. Furthermore, three different scenarios are agreed as follow: Scenario 2 - OD-SSB indication before SCell activation, Scenario 2a - OD-SSB indication together with SCell activation and Scenario 3B – OD-SSB configuration update after SCell activation. Therefore, the test cases shall include all three valid scenarios for both Case 1 and Case 2. RAN4 can define the test cases for OD-SSB based SCell activation with deactivated Scell measurement to test both Scenario 2 and Scenario 2a and without deactivated SCell measurement to test Scenario 2 only. At the same time, RAN4 should introduce the measurement test cases to verify the scenario 3B in CONNECTED mode.

Proposal 1: RAN4 to define the OD-SSB based test case as follows:

- In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

- In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

Currently, RAN4 defined different UE behaviours for deactivated SCell measurement, SCell activation and L1-RSRP measurement. In deactivated SCell measurement, RAN4 introduced FMW for fast OD-SSB measurement. The test case shall verify UE’s behaviour for FMW. RAN4 spent lots of time to discuss UE’s behvaiour after first OD-SSB activation. However, we don’t think such UE’s behaviour for OD-SSB re-activation before SCell activation related T2 shall be verified which will result in the test case too complex.

Proposal 2: FMW: RAN4 to define the test case for deactivated SCell measurement in both Case 1(agreed) and Case 2 to verify the FMW and corresponding interruption requirement.

Proposal 3: T2: RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

In Case 2, RAN4 agreed to define the requirement for Alt Time-C1 in which the OD-SSB transmission is overlapping with AO-SSB and the union of OD-SSB transmission and AO-SSB meets a legacy SSB periodicity. An effective measurement periodicity is applied to both SCell activation and L1-RSRP after SCell activation. Thus, RAN4 shall define the test case to verify the EMP related requirement in Case 2.

Proposal 4: EMP: RAN4 to define the test case to verify the EMP related requirement in Case 2 Alt Time-C1.

In Case 2, there are two other scenarios which are Alt Time-C2 and scenario 3. RAN4 already agreed to define requirement for Alt Time-C2 but only with OD-SSB periodicity. For scenario 3, RAN4 agreed not to introduce requirement in Rel-19.

To considering the test burden, we think it’s fine not to define the test cases for these two scenarios.

Proposal 5: In Case 2, RAN4 not to define the test case for the following scenarios.

- Alt Time-C2: the center frequency locations of always-on SSB and on-demand SSB are same, and the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern

- Scenario 3: the center frequency locations of always-on SSB and on-demand SSB are different

Furthermore, RAN4 agrees to follow the legacy SSB measurement accuracy requirements for OD-SSB. Thus, we think it’s fine not to define new test case for Measurement Performance requirements of OD-SSB.

Proposal 6: RAN4 not to define the new test case for Measurement performance requirements of OD-SSB. UE supporting OD-SSB shall also meet the legacy SSB measurement accuracy test.

Scenario – Scell activation scope

In Rel-19 NES WI, RAN4 agreed most of the SCell activation requirement based on OD-SSB. In last meeting, one of the issues for test cases design is about the SCell activation.

Issue 3-1-2: Scenario – Scell activation scope - Proposals - Proposal 1: RAN4 should focus on SCell activation for case 1 and case 2 Alt Time-C1 and multiple SCells activation. The other SCell activation scenarios should have lower priority. - Proposal 2: RAN4 only define the test cases for single SCell activation.

Considering the trade off between the test loading and the test coverage, we propose RAN4 to at least define the following SCell activation related test cases.

Proposal 7: RAN4 should at least define the SCell activation test cases as follow.

- Singel SCell activation Case 1(agreed): both known and unknown

- Singel SCell activation Case 2: both known and unknown

- Direct SCell activation: Case 1 and Case 2

- Multiple SCells activation Case 2 only

Scenario – active serving cell measurement

In Rel-19 NES WI, RAN4 defined two new requirements for OD-SSB measurement with OD-SSB update. We think RAN4 should at least define two test cases to verify UE’s behaviours for these two scenarios.

RAN4 #115 Issue 1-4-7: L1 measurement in scenario 3B Agreement - When MAC-CE indicates OD-SSB updating in scenario 3B, - UE performs the L1-RSRP measurement based on updated OD-SSB periodicity from Tfirst_SSB. - When DRX is configured, follow legacy principle to define requirements - Tfirst_SSB is the uncertainty time from the timepoint when MAC-CE for updating the transmission parameter is received till the first SSB that UE is able to process based on timeline defined in RAN1 and the addition processing time defined in RAN4. RAN4 #116 Issue 1-2-1: OD-SSB based L3 measurement when SCell is activated(scenario 3B) Agreement - For SCell measurement, UE shall follow the OD-SSB specific SMTC when OD-SSB is activated; For neighbor cell measurement, UE follows legacy SMTC regardless of status of OD-SSB.

Proposal 8: RAN4 should test L1-RSRP and L3-RSRP for OD-SSB update in scenario 3B.

General Principle

In last meeting’s WF, it lists the following principles to define the test case.

Issue 3-1-5: General principle RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed. - Both FR1 and FR2 - Test OD-SSB periodicities with 5ms, 20ms. 10ms - Only test non-DRX mode - The test without SSB time index detection - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting - SMTC and measCycleSCell as 40ms/160ms - Only NCD-SSB for OD-SSB test case

To simplify the test case setting, we prefer the following proposal:

Proposal 9: RAN4 to define the OD-SSB test cases with the following general princinples:

- Both FR1 and FR2

- Only test OD-SSB periodicities with 5ms, 10ms and 20ms

- Only test non-DRX mode

- without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

- Only NCD-SSB for OD-SSB test case

Test Configuration

RAN4 agreed that the deactivated SCell’s measurement lower bound is 10ms and the lower bound is not applied to SCell activation. Therefore, we propose to define the SCell activation test case with 5ms periodicity. When UE needs the lower bound, the test requirement shall base on 10ms for deactivated SCell measurement. In Case 2, we propose to define two types of deactivated SCell measurement. One is for OD-SSB overlapping with AO-SSB. The OD-SSB periodicity can be set as 5ms. Another test case is for the union of OD-SSB and AO-SSB meets the legacy SSB periodicity. Both OD-SSB and AO-SSB periodicity can be set as 20ms. At the same time, in case 2, two remaining scenarios are being discussed in RAN4: RAN1 Alt Time-C2 and scenario 3. We propose not to define any test case for these two scenarios.

Proposal 10: In Case 1, RAN4 to define the OD-SSB based SCell activation test case with 5ms periodicity.

Proposal 11: In Case 2, RAN4 to define the OD-SSB based SCell activation test case as follow.

- OD-SSB(5ms) is overlapping with AO-SSB(20ms)

- The union of OD-SSB(20ms) and AO-SSB(20ms) meets the legacy SSB periodicity(10ms)

# Test Case List

To tradeoff the burden of tests and the test coverage, we prefer to consider the following test cases.

Proposal 12: RAN4 to define OD-SSB test cases as follow.

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| 1a (Agreed) | SCell Activation and deactivation of known SCell in non-DRX | SCell activation Deactivated SCell measurement Interruption during deactivated SCell measurement Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation |
| 1b (Agreed) | SCell Activation and deactivation of unknown SCell in non-DRX(OD-SSB periodicity is 5ms) | SCell activation | OD-SSB periodicity 5ms | OD-SSB Case 1 OD-SSB scenario 2a Unknown SCell activation |
| 2a | SCell Activation and deactivation of known SCell in non-DRX | SCell activation Deactivated SCell measurement Interruption during deactivated SCell measurement Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - OD-SSB(5ms) is overlapping with AO-SSB(20ms) OD-SSB scenario 2 and 2a |
| 2b | SCell Activation and deactivation of known SCell in non-DRX for 640 ms SCell measurement cycle | SCell activation Deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - The union of OD-SSB and AO-SSB meets the legacy SSB periodicity OD-SSB scenario 2 and 2a |
| 3 | Multiple SCells Activation and deactivation of unknown SCells in non-DRX | Multiple SCells activation | OD-SSB periodicity 5ms AO-SSB periodicity 20ms | OD-SSB Case 2 OD-SSB scenario 2a |
| 4a | Direct SCell activation at SCell addition of known SCell | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a |
| 4b | Direct SCell activation at SCell addition of known SCell | Direct SCell activation | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 OD-SSB scenario 2a |
| 5b | PUCCH SCell Activation and deactivation of known SCell | PUCCH SCell Activation | OD-SSB periodicity 10ms | OD-SSB Case 2 OD-SSB scenario 2 and 2a Known SCell activation |
| 6a (Agreed) | Intra-frequency measurement(deactivated SCell) | Deactivated SCell measurement | OD-SSB periodicity 10ms | OD-SSB Case 1 FMW |
| 6b (Agreed) | Intra-frequency measurement(deactivated SCell) | Deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW |
| 7a | Intra-frequency measurement- SA event triggered reporting tests without gap for deactivated SCell under non DRX | L3-RSRP measurement | OD-SSB periodicity 10ms | OD-SSB scenario 3B |
| 7b | Intra-frequency measurement- SA event triggered reporting tests without gap under non DRX | L3-RSRP measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB scenario 3B |
| 8 (Agreed) | SSB based L1-RSRP measurement when DRX is not used | L1-RSRP measurement | OD-SSB periodicity 20ms -> 10ms | OD-SSB scenario 3B |

- Summary

In this contribution, we have discussed the the performance part of Rel-19 NES. Based on the discussions, we made the following proposals and observations:

Proposal 1: RAN4 to define the OD-SSB based test case as follows:

- In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A.

- In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

Proposal 2: FMW: RAN4 to define the test case for deactivated SCell measurement in both Case 1(agreed) and Case 2 to verify the FMW and corresponding interruption requirement.

Proposal 3: T2: RAN4 not to define the test case to verify T2 and UE’s behaviour for re-activating the OD-SSB transmission.

Proposal 4: EMP: RAN4 to define the test case to verify the EMP related requirement in Case 2 Alt Time-C1.

Proposal 5: In Case 2, RAN4 not to define the test case for the following scenarios.

- Alt Time-C2: the center frequency locations of always-on SSB and on-demand SSB are same, and the union of AO-SSB transmission and OD-SSB transmission has a non-periodic time domain pattern

- Scenario 3: the center frequency locations of always-on SSB and on-demand SSB are different

Proposal 6: RAN4 not to define the new test case for Measurement performance requirements of OD-SSB.

Proposal 7: RAN4 should at least define the SCell activation test cases as follow.

- Singel SCell activation Case 1(agreed): both known and unknown

- Singel SCell activation Case 2: both known and unknown

- Direct SCell activation: Case 1 and Case 2

- Multiple SCells activation Case 2 only

Proposal 8: RAN4 should test L1-RSRP and L3-RSRP for OD-SSB update in scenario 3B.

Proposal 9: RAN4 to define the OD-SSB test cases with the following general princinples:

- Both FR1 and FR2

- Only test OD-SSB periodicities with 5ms, 10ms and 20ms

- Only test non-DRX mode

- without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

Proposal 10: In Case 1, RAN4 to define the OD-SSB based SCell activation test case with 5ms periodicity.

Proposal 11: In Case 2, RAN4 to define the OD-SSB based SCell activation test case as follow.

- OD-SSB(5ms) is overlapping with AO-SSB(20ms)

- The union of OD-SSB(20ms) and AO-SSB(20ms) meets the legacy SSB periodicity(10ms)

Proposal 12: RAN4 to define OD-SSB test cases as follow.

| No. | Test category | Test scenario | Test configuration | Test purpose |
| --- | --- | --- | --- | --- |
| 1a (Agreed) | SCell Activation and deactivation of known SCell in non-DRX | SCell activation Deactivated SCell measurement Interruption during deactivated SCell measurement Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2 and 2a Known SCell activation |
| 1b (Agreed) | SCell Activation and deactivation of unknown SCell in non-DRX(OD-SSB periodicity is 5ms) | SCell activation | OD-SSB periodicity 5ms | OD-SSB Case 1 OD-SSB scenario 2a Unknown SCell activation |
| 2a | SCell Activation and deactivation of known SCell in non-DRX | SCell activation Deactivated SCell measurement Interruption during deactivated SCell measurement Interruption at OD-SSB activation/deactivation | OD-SSB periodicity 10ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - OD-SSB(5ms) is overlapping with AO-SSB(20ms) OD-SSB scenario 2 and 2a |
| 2b | SCell Activation and deactivation of known SCell in non-DRX for 640 ms SCell measurement cycle | SCell activation Deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 - The union of OD-SSB and AO-SSB meets the legacy SSB periodicity OD-SSB scenario 2 and 2a |
| 3 | Multiple SCells Activation and deactivation of unknown SCells in non-DRX | Multiple SCells activation | OD-SSB periodicity 5ms AO-SSB periodicity 20ms | OD-SSB Case 2 OD-SSB scenario 2a |
| 4a | Direct SCell activation at SCell addition of known SCell | Direct SCell activation | OD-SSB periodicity 10ms | OD-SSB Case 1 OD-SSB scenario 2a |
| 4b | Direct SCell activation at SCell addition of known SCell | Direct SCell activation | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 Alt Time-C1 OD-SSB scenario 2a |
| 5b | PUCCH SCell Activation and deactivation of known SCell | PUCCH SCell Activation | OD-SSB periodicity 10ms | OD-SSB Case 2 OD-SSB scenario 2 and 2a Known SCell activation |
| 6a (Agreed) | Intra-frequency measurement(deactivated SCell) | Deactivated SCell measurement | OD-SSB periodicity 10ms | OD-SSB Case 1 FMW |
| 6b (Agreed) | Intra-frequency measurement(deactivated SCell) | Deactivated SCell measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB Case 2 FMW |
| 7a | Intra-frequency measurement- SA event triggered reporting tests without gap for deactivated SCell under non DRX | L3-RSRP measurement | OD-SSB periodicity 10ms | OD-SSB scenario 3B |
| 7b | Intra-frequency measurement- SA event triggered reporting tests without gap under non DRX | L3-RSRP measurement | OD-SSB periodicity 20ms AO-SSB periodicity 20ms | OD-SSB scenario 3B |
| 8 (Agreed) | SSB based L1-RSRP measurement when DRX is not used | L1-RSRP measurement | OD-SSB periodicity 20ms -> 10ms | OD-SSB scenario 3B |

- References

- R4-2512327, “WF on RRM requirements for Netw_Energy_NR_enh_Part1”, Ericsson