3GPP TSG-RAN4 Meeting #116bis	R4-2513988

# Prague, Czech Republic, Oct. 13-17, 2025

Source:	Ericsson

Title:	RRM performance requirements intra-band non-collocated EN-DC/NR-CA deployment Phase2

Agenda item:	6.2.2

Document for:	Discussion

# 1	Introduction

In WID [1], it is stated:

… ### 4.2 Objective of Performance part WI NOTE: Leave empty if the WI proposal does not contain a RAN performance part. The performance part of the work item includes: - Specify the corresponding performance requirements for type 4a/4b capable FWA UE. - RRM performance requirements - PDSCH demodulation performance requirements …

In this contribution we discuss RRM performance requirements for type 4 capable FWA UE.

# 2	Discussion

The WI has had the following impact on RRM core requirements according to the WI split and the big CR [2,3]:

| Change | Sections |
| --- | --- |
| 1 | (7.6 Maximum Receive Timing Difference) 7.5.4 Minimum Requirements for NR Carrier Aggregation (MTTD) 7.6.4 Minimum Requirements for NR Carrier Aggregation (MRTD) |
| 2 | (8.1.7 Scheduling availability of UE during radio link monitoring) 8.1.7.2 Scheduling availability of UE performing radio link monitoring with a different subcarrier spacing than PDSCH/PDCCH on FR1 |
| 3 | (8.2 Interruption) 8.2.1.2.3 Interruptions at SCell addition/release 8.2.1.2.4 Interruptions at SCell activation/deactivation 8.2.2.2.1 Interruptions at SCell addition/release 8.2.2.2.2 Interruptions at SCell activation/deactivation 8.2.2.2.3 Interruptions during measurements on deactivated SCC |
| 4 | (8.3 SCell Activation and Deactivation Delay) 8.3.2 SCell Activation Delay Requirement for Deactivated SCell |
| 5 | (8.5 Link Recovery Procedures) 8.5.2 Requirements for SSB based beam failure detection 8.5.3 Requirements for CSI-RS based beam failure detection 8.5.5 Requirements for SSB based candidate beam detection 8.5.6 Requirements for CSI-RS based candidate beam detection |
| 6 | (8.5 Scheduling availability of UE performing beam failure, L1-RSRP measurement ) 8.5.7.2 Scheduling availability of UE performing beam failure detection with a different subcarrier spacing than PDSCH/PDCCH on FR1 8.5.8.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 |
| 7 | (9.2 NR intra-frequency measurements) 9.2.5.3.1 Scheduling availability of UE performing measurements in TDD bands on FR1 9.2.5.3.2 Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 (9.3 NR inter-frequency measurements) 9.3.9.3.1 Scheduling availability of UE performing measurements in TDD bands on FR1 9.3.9.3.2 Scheduling availability of UE performing measurements with a different subcarrier spacing than PDSCH/PDCCH on FR1 (9.5 L1-RSRP measurements for Reporting) 9.5.6.2 Scheduling availability of UE performing L1-RSRP measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 (9.7 Cross Link Interference measurements) 9.7.4.1 Scheduling availability of UE performing measurement on FR1 (9.8 L1-SINR measurements for Reporting) 9.8.6.2 Scheduling availability of UE performing L1-SINR measurement with a different subcarrier spacing than PDSCH/PDCCH on FR1 (9.10 CSI-RS based L3 measurements) 9.10.2.6 Scheduling availability of UE during CSI-RS based intra-frequency measurements (9.13 L1-RSRP measurements for a cell with different PCI from serving cell) 9.13.6 Scheduling availability of UE during L1-RSRP measurement |

Observation 1: The big CR summarizes the RRM core requirement changes.

We can use the RRM core changes from the WI as the basis for performance requirement development by analyzing each affected core requirement section.

Observation 2: 	Use RRM core changes from the WI to analyze development of performance requirements. The affected core requirement areas are:

- Timing: MTTD, MRTD

- Scheduling availability for CA

- Interruption

- SCell Activation and deactivation Delay

- Link Recovery Procedures

The issue becomes to figure out if we need texts cases for all these areas. The release 18 WI [4] only had a demodulation performance part, so there are no explicit RRM performance requirements tests introduced in release-18:

- Phase II:

  - Phase II work will get started after the feasibility in phase I is confirmed

  - Specify PDSCH demodulation requirements for non-collocated scenarios for intra-band non-contiguous EN-DC and NR-CA

    - Define PDSCH demodulation performance requirement based on the applicable MRTD and power imbalance values.

    - Power imbalance between the carriers is limited

Observation 3: The release 18 WI only had a demodulation performance part, so there are no explicit RRM performance requirements tests introduced in release-18.

If we follow observation 1 and check TS 38.133 we find that:

- Timing: MTTD, MRTD

  - Comment: Existing tests test multiple TRP, also separated, but only single cell (MIMO). There are no existing tests or test configurations for CA or DC.

- Scheduling availability for CA

  - Comment: There are a few scheduling availability tests in existing specification but there are no existing tests or test configurations for CA or DC in FR1

- Interruption

  - Interruptions at SCell addition/release.

    - Comment: There are no interruptions at Scell addition/release in existing specification for CA or DC in FR1

  - Interruptions during measurements on deactivated SCC

    - A4 Interruptions during measurements on deactivated NR SCC in FR1

    - A6 Interruptions during measurements on deactivated NR SCC in FR1

    - Comment: There are existing tests or test configurations for CA or DC, but not for the capabilities and signaling of Non-colocated intra band WI

- SCell Activation and deactivation Delay

  - A4 SCell Activation and Deactivation Delay

  - A6 SCell Activation and Deactivation Delay

  - Comment: There are existing tests or test configurations for CA or DC, but not for the capabilities and signaling of Non-colocated intra band WI

- Link Recovery Procedures

  - Requirements for SSB based beam failure detection

    - Comment: There are existing tests or test configurations for CA or DC, but not for the capabilities and signaling of Non-colocated intra band WI

  - Requirements for CSI-RS based beam failure detection

    - Comment: There are existing tests or test configurations for CA or DC, but not for the capabilities and signaling of Non-colocated intra band WI

  - Requirements for SSB based candidate beam detection

    - There are no candidate beam detection tests in existing specifications for CA or DC in FR1

  - Requirements for CSI-RS based candidate beam detection

    - There are no candidate beam detection tests in existing specifications for CA or DC in FR1

Looking at the list above one can conclude that in many cases there is a complete lack of tests for ENDC and CA configurations, from release-15 and onwards:

- Timing: MTTD, MRTD

- Scheduling availability for CA

- Interruptions at SCell addition/release.

Furthermore, these cases are covered as ENDC and CA cases in existing specification but not for capabilities and signaling of Non-colocated intra band WI in rel-18 and rel-19:

- Interruptions during measurements on deactivated SCC

- SCell Activation and deactivation Delay

- Link Recovery Procedures

Proposal 1: Develop tests for cases where there is a complete lack of tests for ENDC and CA configurations, from release-15 and onwards:

- Timing: MTTD, MRTD

- Scheduling availability for CA

- Interruptions at SCell addition/release.

# 3 	Summary

Observation 1: The big CR summarizes the RRM core requirement changes.

Observation 2: 	Use RRM core changes from the WI to analyze development of performance requirements. The affected core requirement areas are:

- Timing: MTTD, MRTD

- Scheduling availability for CA

- Interruption

- SCell Activation and deactivation Delay

- Link Recovery Procedures

Observation 3: The release 18 WI only had a demodulation performance part, so there are no explicit RRM performance requirements tests introduced in release-18.

Proposal 1: Develop tests for cases where there is a complete lack of tests for ENDC and CA configurations, from release-15 and onwards. These tests can contain several sets of test parameters covering ENDC and CA in general, as well as non-collocated intra band CA.

- Timing: MTTD, MRTD

- Scheduling availability for CA

- Interruptions at SCell addition/release.

# References

[1] 		RP-242083, Revised WID on Support of intra-band non-collocated EN-DC/NR-CA deployment Phase2: new receiver type(s), KDDI

[2]	R4-2420090, WF on RRM requirements for NonCol_intraB_ENDC_NR_CA_Ph2, Huawei, HiSilicon

[3]	R4-2512350, Draft big CR on RRM requirements for Support of intra-band non-collocated EN-DC/NR-CA deployment Phase 2, Huawei, HiSilicon

[4]	RP-230742, New WID: Support of intra-band non-collocated EN-DC/NR-CA deployment, KDD