3GPP TSG-RAN WG4 Meeting #116bis		R4-2513992
Prague, Czech Republic, 13 – 17 Oct 2025

Title:	Discussion on R20 PC1.5 with 1Tx for IoT NTN HPUE

Source:	OPPO

Agenda item:	7.6.3

Document for:	Approval

Introduction

In Rel-20, PC1.5 with 1Tx for NB-IoT based IoT NTN is one of the topics [1]. This paper shares our view on this aspect.

- Specify high power UE (HPUE) NB-IoT based IoT-NTN in FR1-NTN bands and the corresponding LTE NTN bands for the single uplink (UL) carrier scenario - IoT-NTN PC1.5 with 1Tx for handheld and non-handheld device types - Example bands are Band 256 and 255 - Necessary Tx requirements, at least including - Specify MPR and A-MPR for the example bands considering the regulation for the corresponding bands, if necessary - Necessary Rx requirements, at least including - Specify the RSD requirements on the example bands, if necessary - Specify the necessary signaling to support the above objectives - The band agnostic requirements for UE power class specified for TN are expected to apply also to corresponding NR NTN power classes.

Discussion

In Rel-20, for PC1.5 with 1Tx for NB-IoT based IoT NTN, the Tx and Rx requirements which is related with power class need discuss and define, as list in following Table 1:

Table 1 Tx and Rx requirements related with power class

| Tx requirements | Power class and tolerance |
| --- | --- |
|  | MPR |
|  | AMPR |
|  | ACLR |
| Rx requirements | Reference Sensitivity Degradation from PC3 to PC1.5 |

PC1.5 with 1Tx for NB-IoT based IoT NTN can adopt nominal MOP (29dBm) and +2/-3 tolerance.

In Rel-19, PC1 and PC2 with 1Tx for NB-IoT based IoT-NTN in FR1-NTN bands has been introduced in Rel-19, and the co-existence study for PC1 and PC2 with 1Tx for NB-IoT based IoT-NTN has been done, and it was agreed that:

Issue 1-3: IoT-NTN Co-ex study results discussions Agreement: - For NB-IoT based IoT-NTN PC2, it is observed that the required ACLR is less than 20 dB with 1.5km isolation distance assumed based on the simulation results. - For NB-IoT based IoT-NTN PC1, it is observed that the required ACLR is less than 26.2 dB with 1.5km isolation distance assumed based on the simulation results. Issue 1-4: IoT-NTN ACLR definition Agreement: - For PC1/PC2 NB-IoT based IoT NTN, using same ACLR definition as current PC3, i.e., adjacent channel center frequency offset is ±2.5MHz and adjacent channel measurement bandwidth is 3.84MHz

Based on the Rel-19 co-existence study results for PC1 NB-IoT based IoT-NTN, we can get the required ACLR is also less than 26.2 dB with 1.5km isolation distance assumed based on the simulation results for PC1.5 NB-IoT based IoT-NTN.

In Rel-19, for PC1/PC2 NB-IoT based IoT NTN, the required ACLR is less than 26.2 dB and 20dB based on the simulation results, separately. And reused using 37dBc ACLR as defined for PC3.

Therefore, PC1.5 with 1Tx for NB-IoT based IoT NTN can also reuse the ACLR definition as current PC3 for NB-IoT based IoT NTN.

Adopt 37dBc ACLR for PC1.5 with 1Tx for NB-IoT based IoT NTN.

RAN4 define the MPR band AMPR for PC1.5 with 1Tx for NB-IoT based IoT NTN based on following simulation assumption:

- DFT-s-OFDM 1RE (Single Tone)

- Centre Single Tone [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK]

- [2] or [3] dB post PA loss

- 0dB MPR

- Carrier leakage 28dBc

- IQ image:28 dBc

- CIM3: 60 dBc

AMPR is band specifically, two example bands n255 and n256 are given in the WID, RAN 4 needs define AMPR for band n255 with NS_02N and n256 with NS_24.

Define AMPR for example bands n255 with NS_02N and n256 with NS_24.

Since NB-IoT based IoT NTN is HD-FDD, there is no impact of Tx to Rx.

No RSD is needed for NB-IoT based IoT NTN due to no impact of Tx to Rx.

Conclusion

This paper discussed the PC1.5 with 1Tx for NB-IoT based IoT NTN HPUE, and got below observations and proposals.

Proposal 1: 	PC1.5 with 1Tx for NB-IoT based IoT NTN can adopt nominal MOP (29dBm) and +2/-3 tolerance.

Observation 1: 	In Rel-19, for PC1/PC2 NB-IoT based IoT NTN, the required ACLR is less than 26.2 dB and 20dB based on the simulation results, separately. And reused using 37dBc ACLR as defined for PC3.

Proposal 2: 	Adopt 37dBc ACLR for PC1.5 with 1Tx for NB-IoT based IoT NTN.

Proposal 3: 	RAN4 define the MPR band AMPR for PC1.5 with 1Tx for NB-IoT based IoT NTN based on following simulation assumption:

•	DFT-s-OFDM 1RE (Single Tone)

•	Centre Single Tone [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK]

•	[2] or [3] dB post PA loss

•	0dB MPR

•	Carrier leakage 28dBc

•	IQ image:28 dBc

•	CIM3: 60 dBc

Proposal 4: 	Define AMPR for example bands n255 with NS_02N and n256 with NS_24.

Proposal 5: 	No RSD is needed for NB-IoT based IoT NTN due to no impact of Tx to Rx.

2Reference

RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, RAN #109.