3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514026

Prague, CZ, 13th – 17th October 2025

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on SAN conformance requirements for NTN broadcast band

# Agenda item:	4.10.3

# Document for:	Approval

# 1 Introduction

In RAN#107 meeting, a new Work Item [1] was approved on introduction of NTN broadcast band with the following RAN4 objective. In this contribution, we want to share some initial views about NTN broadcast band from SAN conformance requirements perspective.

Specify a new SDO band for LTE-based 5G Broadcast operating in 1467 to 1492 MHz in Region 3 for GSO: - Specify related UE RF core requirements for 10MHz PMCH - UE requirements specified in TS 36.101 for LTE-based 5G Broadcast shall be considered as baseline (reuse assumption of 10 MHz UE Rx filter) - Specify related SAN RF core requirements for 10 MHz PMCH considering also applicable regulatory requirements: - Take existing SAN RF requirements specified in TS 38.108 for 10MHz as baseline - Requirements based on Resolution 761 in WRC-19 - Only consider QPSK and 16QAM modulation Note: UE only needs to support 5G broadcast for this band Note: Release independence applicability will be further discussed

# 2 Discussion

For SAN conformance requirements for NTN broadcast band 246, we make the following proposals.

Table 2.1 General requirements for NTN broadcast band

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| - 4 General test conditions and declarations |  |
| 4.1 Measurement uncertainties and test requirements | 1. Need to add max test system uncertainty with 100kHz for 10MHz occupied bandwidth in Table 4.1.2.2-1. 2. Need to add 9MHz AWGN bandwidth in Table 4.1.2.3-1. |
| 4.2 Requirement reference points | No specification impact. |
| 4.3 Satellite Access Node classes | No specification impact. |
| 4.4 Regional requirements | No specification impact. |
| 4.5 SAN configurations | No specification impact. |
| 4.6 Manufacturer declarations | No specification impact. |
| 4.7 Test configurations | The carrier bandwidth in section 4.7.3.1 needs to be updated. |
| 4.8 Applicability of requirements | No specification impact. |
| 4.9 RF channels and test models | The carrier bandwidth in section 4.9.2.2 needs to be updated. |
| 4.10 Requirements for contiguous and non-contiguous spectrum | No specification impact. |
| 4.11 Requirements for SAN capable of multi-band operation | No specification impact. |
| 4.12 Reference coordinate system | No specification impact. |
| 4.13 Format and interpretation of tests | No specification impact. |

Table 2.2 RF requirements for NTN broadcast band

| Requirements in TS 36.181 | Expected impact |
| --- | --- |
| - 6 Conducted transmitter characteristics |  |
| 6.1 General | Need to add BOG. |
| 6.2 Satellite Access Node output power | No specification impact. |
| 6.3 Output power dynamics |  |
| 6.3.1 General | No specification impact. |
| 6.3.2 RE power control dynamic range | No specification impact. |
| 6.3.3 Total power dynamic range | Need to add 10MHz SAN channel bandwidth in Table 6.3.3.5-1. |
| 6.4 Transmit ON/OFF power | No specification impact. |
| 6.5 Transmitted signal quality |  |
| 6.5.1 General | No specification impact. |
| 6.5.2 Frequency error | No specification impact. |
| 6.5.3 Modulation quality | Need to add EVM requirement for BOG in section 6.5.3.5. |
| 6.5.4 Time alignment error | No specification impact. |
| 6.5.5 DL RS power | Need to specify that this requirement is not applicable to BOG. |
| 6.6 Unwanted emissions |  |
| 6.6.1 General | No specification impact. |
| 6.6.2 Occupied bandwidth | Need to add 10MHz in Table 6.6.2.4.2-1. |
| 6.6.3 Adjacent Channel Leakage Power Ratio (ACLR) | Need to add 10MHz in Table 6.6.3.5-1. |
| 6.6.4 Out-of-band emissions | No specification impact. |
| 6.6.5 Transmitter spurious emissions | No specification impact. |
| 6.7 Transmitter intermodulation | No specification impact. |

Proposal 1: We make the proposals as Table 2.1 and 2.2 for SAN conformance requirements for NTN broadcast band 246.

# 3 Conclusion

In this contribution, we shared some views on SAN conformance requirements for NTN broadcast band and the proposals are made as follows:

Proposal 1: We make the proposals as Table 2.1 and 2.2 for SAN conformance requirements for NTN broadcast band 246.

# 4 Reference

RP-250788, New band for 5G Broadcast Utilizing Geosynchronous Satellite.