3GPP TSG-RAN WG4 Meeting #116bis	R4-2514053

Prague, Czech Republic, 13-17 October, 2025

Source:	Nokia

Title:	Discussion on FBS testing

Agenda item:	6.10.2.1

Document for:	Discussion

Introduction

In this contribution we discuss some aspects of fast L3 beam sweeping factor test cases.

Discussion

RAN4 agreed a test case list for fast L3 beam sweeping (FBS) in RAN4#115 meeting. It was also agreed FBS is controlled by a timer for L3 measurements. In RAN4#116 meeting, it was agreed that the value of the timer is 90 seconds. This means that after triggering FBS, the UE is required to continue FBS at least for the duration of the 90 second timer.

Agreement from RAN4#115 meeting: Issue 1-1-1: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” Agreement: For the quitting condition of FBS: - Introduce a new UAI indicating “overheating” issue: - RAN4 will not discuss the UE behaviour after reporting UAI - Introduce a timer-based solution: - UE is allowed to fall back to normal measurement when it has performed fast measurement for at least T1 from the time point when UE enters the FBS mode after meeting the FBS entry condition - Further discuss the candidate numbers for T1. Agreement from RAN4#116 meeting: Issue 1-1-1: Value for T1 Agreement: - A fixed value of 90 seconds.

Earlier RAN4 agreed that UE quitting FBS is not to be tested.

Agreement from RAN4#115 meeting: Issue 2-1-3: whether or not to test FBS quitting Agreement on Monday: No need to test UE not using FBS for scenario 1~4 if the FBS condition is not met.

However, considering the UE is now required to continue FBS for 90 seconds, to test the requirements properly, the test case should ensure the UE continues FBS at least for the duration of 90 seconds after the condition to trigger FBS is fulfilled. It should be discussed by RAN4 how to verify this in the test case.

Discuss how to verify in the FBS test cases for L3 measurements that the UE does fast beam sweeping at least for the duration of the 90 s timer.

Conclusion

The following proposal was made:

Proposal 1: Discuss how to verify in the FBS test cases for L3 measurements that the UE does fast beam sweeping at least for the duration of the 90 s timer.