3GPP TSG-RAN WG4 Meeting #116bis	R4-2514062

Prague, Czech, 13th – 17th October 2025

Source: 	Ericsson

Title:  	Discussion on RRM requirements for NTN for NR Phase 3

Agenda Item:	4.25.3

Document for:	Discussion

# Introduction

In the light of the WI NR Non-Terrestrial Networks (NTN) for NR Phase 3, the outcomes of the last meeting's debate, encompassing consensuses and unresolved issues, were documented in WF [1].

This contribution outlines our point of view on on the underlying impacts to RRM requirements mandated by the issues listed in the WF.

# Discussion

## Topic #1: SSB periodicity larger than 20 ms

Issue 0-B: Two different SMTC periodicities (with different offsets) in one frequency layer

Agreement (online): - When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - No requirements if the SMTC periodicity is smaller than the actual SSB periodicity of the cells associated with the SMTC - For Trs in Handover requirement: - Trs is the SMTC corresponding to the physical cell ID of the target cell - Further discuss the proposal “or the longer SMTCconfigured in the measObjectNR having the same SSB frequency and subcarrier spacing.” - For Trs in RRC Connection Release with Redirection requirement: - If physical cell ID is provided in RAN2 signalling design, Trs is the SMTC corresponding to the physical cell ID of the target cell - Otherwise, Trs is the maximum SMTC periodicity configured in the measObjectNR having the same SSB frequency and subcarrier spacing. Agreement (ad-hoc): - When two different SMTC periodicities (with different offsets) in one frequency layer are configured, - For Ttarget_cell_SMTC_period in paging interruption requirements: - Ttarget_cell_SMTC_period is the SMTC periodicity corresponding to the physical cell ID of the target cell - For Kmulti_SMTC, K_satellite, Kp, Klayer1_measurement, and Kgap in RRC Connected mode intra/inter-frequency measurement requirement: - To be confirmed in maintenance phase: - Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-th occasion of SMTC#i. - Klayer1_measurement: Calculate scaling factor Klayer1_measurement based on the union of the SMTCs

In this issue, there’re still few leftovers to be confirmed.

Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j) can be regarded as a simplification when handling the case of two SMTC periodicities and their potential collision between SMTC occasions. In our view, this approach may introduce some additional delay; however, the delay still remains within an acceptable range.

Proposal 1: Confirm Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-the occasion of SMTC#i, and it is applied for idle mode and connected mode.

For Trs in the handover requirement, the remaining question is whether the case should be considered where the SMTC of the target cell is not provided. Based on our understanding and further verification, at least in the context of handover, the target SMTC shall be provided as what we have done in SMTC2, even it isn’t explicitly indicated by a mandatory field.

Proposal 2: For handover, it is assumed that the SMTC of the target cell is always provided, not to consider the case where the SMTC of the target cell is not provided.

# Conclusion

Proposal 1: Confirm Kmulti_SMTC,i = maxj(Kmulti_SMTC,i,j), where Kmulti_SMTC,i,j is calculated in the legacy way for the j-the occasion of SMTC#i, and it is applied for idle mode and connected mode.

Proposal 2: For handover, it is assumed that the SMTC of the target cell is always provided, not to consider the case where the SMTC of the target cell is not provided.

# References

- R4-2512337, The WF on RRM requirements for Rel-19 NR NTN phase3, QC