3GPP TSG RAN WG4 Meeting #116-bis	R4-2514075

Prague, Czech, 13 – 17 October, 2025

Agenda item:			4.8.2

Source:	vivo

Title:	Remaining issues on RRM requirements for LB CA via switching

Document for:	Discussion

# Introduction

In the RAN4#116 meeting, RAN4 completed RRM core requirements for low band CA via switching WI. Following agreements were made and captured in the WF [1]. In addition, there are remaining open issues as highlighted to be solved in maintenance phase.

Issue 1-1-1: RRM requirement applicability for R19 LB CA via switching - for gap-less based L3 measurement and L1 measurement of a serving carrier which can be either FDD PCC or activated SDL SCC: - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, no delay extension is allowed. - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions. - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} are available based on the LB-CA switching pattern, the corresponding requirements do not apply. Issue 1-1-2: switching pattern applicability for dormant SCell - No requirement is defined for dormant SCell in LB-CA via switching. ### Issue 1-2: Interruption or scheduling restriction requirement Issue 1-2-1: Necessity of interruption requirement in RRM between FDD CC and SDL CC - For SDL to FDD switching and FDD to SDL switching, RAN4 to not define interruption or scheduling restriction requirement on SDL carrier and FDD carrier. Issue 1-2-2: Necessity of interruption requirement in RRM when more carriers are considered - According to the WID, RANA only defines requirements for CA with FDD PCC and one SDL SCC. Issue 1-2-3: Interruption or scheduling restriction requirement (if needed) for carrier switching between FDD CC and SDL CC - This issue is closed. ### Issue 1-3: SDL SCell related requirement #### Activated SDL SCell L3 measurement Issue 1-3-1: Requirement design of activated SDL SCell L3 measurement - if the activated SCC measurement is MG-based in LB-CA, the legacy intra-frequency identification/measurement requirement with MG can be applied. - reuse the existing measurement requirements with gap for inter-frequency measurement and inter-RAT measurement in LB-CA. - Not consider the scenario and not define the requirement for measurement without gap for inter-frequency measurement and inter-RAT measurement in LB-CA. agree on the following approach, and further update on the wording is not precluded. Further discuss on the CR drafting. - if the activated SCC measurement is MG-less in LB-CA, Kp factor can be updated for activated SCC identification/measurement requirement (intra-frequency measurement without MG) for LB-CA. - Kp factor for MG-less activated SCC identification/measurement requirement in LB-CA can be updated as following: - Kp = Ntotal / Navailable, where Navailable and Ntotal are calculated as follows: - For a window W of duration max(SMTC period, xRP_max, switch pattern periodicity), where xRP_max is the maximum xRP across all configured per-UE GAPs, periodic MUSIM gaps, and/or per-FR GAPs within the same FR as the SSB frequency layer, and switch pattern periodicity is the periodicity of the NW configured switch pattern for LB CA via switching, and starting from the beginning of any SMTC occasion: - Ntotal is the total number of SMTC occasions within the window, including those overlapped with MG and those overlapped with FDD PCC duration based on the switching pattern within the window, and - Navailable is the number of SMTC occasions that are not overlapped with any non-dropped GAP or non-dropped MUSIM gap occasions and not overlapped with FDD PCC duration based on the switching pattern within the window W. - No requirement when Navailable = 0. #### Deactivated SDL SCell L3 measurement Issue 1-3-2: Interruption requirement design of deactivated SDL SCell measurement - For R19 LB CA via switching, interruption length on PCC due to measurements on deactivated SDL is (tuning time in slot level + TSMTC_duation + retuning time in slot level), - Tuning time from PCell to SCell is equal to the RRC configured switching gap from PCell to SCell LBCA-SwitchingGap-Duration-PCelltoSCell-r19 - Returning time from SCell to PCell is equal to the RRC configured switching gap from SCell to PCell, i.e., LBCA-SwitchingGap- SCelltoPCell-r19 - Missed ACK/NACK rate - For R19 LB CA via switching, missed ACK/NACK rate due to measurements on deactivated SDL are allowed with up to X= YmeasCycleSCell, where Y=2* interruption length. #### SDL SCell activation/deactivation Issue 1-3-3: interruption or scheduling restriction during SDL SCell activation/deactivation - No requirement for interruption or scheduling restriction during SDL SCell activation/deactivation. This issue is closed. Issue 1-3-4: CQI/L1-RSRP reporting during SDL SCell activation in LB-CA - FFS this issue in maintenance stage. Issue 1-3-5: Activation of Switching Pattern - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline. Issue 1-3-5a: Deactivation of Switching Pattern - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length). - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot. #### SDL SCell L1 requirement Issue 1-3-6: BFD/CBD requirements for SDL SCell - RAN4 to define requirement for BFD/CBD for SDL SCell in LB CA via switching - follow the principle in issue 1-1-1 to define the BFD/CBD requirement for SDL SCell. - P factor in the existing BFD/CBD requirement can be updated for LB-CA via switching, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of BFD-RS or CBD-RS in W window. - Navailable is number of available BFD-RS or CBD-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage for P3. Issue 1-3-7: TCI state switch delay requirement for SDL SCell - RAN4 to define the TCI state switch delay requirement for LB-CA via switching - In TCI state switch delay requirement for SDL SCell, Tfirst-SSB_LB is time to first available SSB reception based on switching pattern after MAC CE command is decoded by the UE. Issue 1-3-8: L1-RSRP/SINR measurement requirements for SDL SCell - follow the principle in issue 1-1-1 to define the L1-RSRP/SINR requirement for SDL SCell. - P factor in the existing L1-RSRP/SINR measurement requirements can be updated for LB-CA via switching, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal / Navailable - Ntotal is total number of L1-RSRP/SINR RS of SCell in W window. - Navailable is number of available L1-RSRP/SINR RS overlapping with SCell duration according to the RRC configured switching pattern in W window. - No requirement when Navailable = 0. #### Others for SDL SCell Issue 1-3-9: BWP switching for SDL SCell - FFS this issue in maintenance stage. ### Issue 1-4: Impact on FDD PCell RRM Issue 1-4-1: RLM requirement for PCell - For RLM in LB-CA, follow principle in issue 1-1-1 to define RLM requirements for PCell in LB CA. - P factor in the existing RLM measurement requirements can be updated, where, - W window is max(TL1, xRP_max, switching pattern periodicity) - P = Ntotal/ Navailable - Ntotal is total number of RLM-RS in W window. - Navailable is number of available RLM-RS in W window. - No requirement when Navailable = 0. - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure. Issue 1-4-2: others RRM requirement of FDD PCell in LB CA via switching - The principle of agreed activated SDL SCell requirement in LB-CA can be reused for the corresponding PCell requirement in LB-CA.

In the previous RAN4 meetings, we provided our views in [2, 3, 4, 5]. In this contribution, we further provide our views on the remaining open issues for RRM core requirements for low band carrier aggregation with switching.

# Discussion

## 2.1 SDL SCell requirements

SDL SCell activation

In the last RAN4 meeting, there were discussions and agreements on the application of switching pattern, switching behavior and implementation margin.

Issue 1-3-5: Activation of Switching Pattern - Revise the previous agreement in R4-2504895 issue 1-1-3 (RAN4 #114bis) for the SCell which is in activation procedure: - For the SCell which is in activation procedure, - The RRC configured switching pattern for SDL SCell is applied. - For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

During discussions on SCell activation delay requirements in Rel-15, following agreements were made and captured in the WF [6].

- SSB based activation delay requirement Tactivation_time in FR1 - Tactivation_time is consisted of - MAC-CE message decode time - RF warm up excluding AGC settling: TBD - AGC settling: N1 SMTC periods for known and N2 SMTC periods for unknown cells - The exact value of N1 and N2 are TBD - It is FFS that the parameter value of N1 shall depend on the SCell measurement cycle in use prior to the SCell activation - PSS/SSS and SSB index acquiring: - [2] SMTC periods for known cell provided that PSS/SSS detection and PBCH decoding are not needed - [4] SMTC period for known cell provided that PBCH decoding is needed - [5] SMTC period for unknown cell provided that PSS/SSS detection and PBCH decoding are needed

It was further agreed that overall time for MAC-CE decoding, application of MAC-CE configuration and RF warm up time is 3ms.

The switching pattern is applied after SCell activation command is received and decoded. UE is not required to warm up SCell RF before switching pattern is actually applied. Therefore, during the 3ms for SCell activation time, there is still time, e.g., the time used for RF warm up for UE to prepare switching pattern related configurations if it is needed based on UE implementations. From this perspective, there is no need to have additional processing time for application of switching pattern. In other words, TLBCA is zero. If there is really need on the internal processing time for some UE implementations, a UE capability may be considered.

If it is RRC based direct SCell activation, there is no need for additional processing time since it is RRC configuration and application of RRC configuration is already included in the RRC processing time

Proposal 1: No additional time is needed for applying switching pattern, i.e., TLBCA is 0, for both MAC-CE and RRC triggered SCell activation.

When SCell is deactivated or not configured, the UE is continuing staying on PCell without switching though switching pattern is configured. If SCell activation is triggered via MAC-CE or RRC, after THARQ+3ms or TRRC_Process the UE applies the switching pattern. Then, UE should perform switching accordingly.

As depicted in Fig 1, the UE should be on PCell according to switching pattern. In this case, UE stay on the PCell and performs switching according to the configured switching pattern.

Fig 1. SCell activation: Switching pattern and switching case 1

As depicted in Fig 2-1 and Fig 2-2, the UE should be on SCell according to switching pattern. In this case, there could be different switching mechanism.

In Fig 2-1, UE switches to SCell immediately after switching pattern is applied by creating new switching gap for FDD carrier to SDL carrier. The pros are that UE can access the SCell spectrum as early as possible. The cons are that UE may stay on the SCell for a very short time depending on the timing of switching pattern application.

In Fig 2-2, UE switches to SCell at the next switching period according switching pattern after switching pattern is applied. It has less overhead due to switching compared to Fig 2-1.

Fig 2-1. SCell activation: Switching pattern and switching case 2

Fig 2-2. SCell activation: Switching pattern and switching case 2

RAN4 needs to decide which mechanism is to be used. In our view, the mechanism in Fig 2-2 is preferred. Another possible solution is to decide whether to switching immediately or switching at next switching period depending on the dwell time on SCell if UE switches to SCell immediately.

Proposal 2: If UE should be on PCell according to switching pattern after THARQ+3ms or TRRC_Process after MAC-CE or RRC based SCell activation PDSCH is received, UE performs switching according to the configured switching pattern.

Proposal 3: If UE should be on SCell according to switching pattern after THARQ+3ms or TRRC_Process after MAC-CE or RRC based SCell activation PDSCH is received, UE switches to SCell at the next switching period or UE performs switching based on dwell time on SCell.

Issue 1-3-5a: Deactivation of Switching Pattern - The switching pattern is NOT applied after UE completed the SCell deactivation (n+(THARQ+3ms+ TLBCA)/slot-length). - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - FFS in maintenance stage: If UE is on PCell after completion of SCell deactivation, UE keeps staying on PCell. If UE is on SCell after completion of SCell deactivation, UE switches to PCell immediately. The carrier switching, if applicable, shall be placed at the end of a slot.

Similar to SCell activation, there is no need to have additional processing time for not applying the switching pattern. The 3ms includes time for MAC-CE decoding, application of MAC-CE and RF retuning time (turn off or to deep sleep). Moreover, for timer based SCell deactivation, there is no MAC-CE decoding needed at all.

Proposal 4: No additional time is needed for NOT applying switching pattern, i.e., TLBCA is 0, for both MAC-CE and timer triggered SCell deactivation.

For SCell deactivation, simple mechanism can be used as depicted in Fig 3. After completion of SCell deactivation, the switching pattern is not applied anymore. The UE should switch to PCell immediately. Depending on UE is on PCell on SCell currently, UE performs switching accordingly. If UE is on PCell, UE keeps staying on PCell. If UE is on SCell, UE needs to switch to PCell immediately.

Fig 3. SCell deactivation: Switching pattern and switching cases

Proposal 5: The switching pattern is NOT applied after UE completed the SCell deactivation at n+ ((THARQ+3ms)/slot-length) for MAC-CE based SCell deactivation or at n+(3ms/slot-length) for timer triggered SCell deactivation.

Proposal 6: After completion of SCell deactivation, if UE is on PCell, UE keeps staying on PCell. If UE is on SCell, UE switches to PCell immediately.

Issue 1-3-4: CQI/L1-RSRP reporting during SDL SCell activation in LB-CA - Proposal 1 (Apple, Samsung/MTK(not consider colliding case)): - If CQI or L1-RSRP reporting is colliding with SSB/CSI-RS or SMTC of being-activated SDL SCell during the SDL SCell activation, no SCell activation requirement shall be applied; otherwise, legacy CQI and L1-RSRP reporting requirement of SCell activation can be reused for SCell activation in LB-CA. - Proposal 2 (LGE): - If the CQI or L1-RSRP reporting is colliding with SCell measurement during SDL SCell activation, the SCell measurement should be prioritized. - Proposal 3 (vivo): - UE is to drop invalid CQI reporting and prioritize valid CQI reporting when SSB reception is collided with CQI reporting, or UE doesn’t report invalid CQI during SCell activation. - Proposal 4 (HW): - During the SDL SCell activation - Starting from the slot specified in clause 4.3 of TS 38.213 (timing for secondary Cell activation/deactivation) and until Tactivation, SMTC based measurement on SDL is prioritized over transmission/reception on PCell. - During TCSI_reporting , CSI-RS based CSI measurement on SDL is prioritized over transmission/reception on PCell. - Starting from the slot specified in clause 4.3 of TS 38.213 (timing for secondary Cell activation/deactivation) and until the UE has completed a first L1-RSRP measurement, SSB/CSI-RS L1-RSRP measurement on SDL is prioritized over transmission/reception on PCell. - There are no requirements for the SCell activation which requires L1 RSRP reporting, when all L1-RSRP reporting occasions are colliding with L1-RSRP measurement occasions. - Proposal 5(ZTE): - The starting point and ending point of CSI measurement for the sake of CSI reporting should be determined. - FFS this issue in maintenance stage.

The issue should be related to SCell activation without applying switching pattern. Since it was decided switching pattern is applied during activation procedure, there should be no issue for CQI/L1-RSRP reporting. The measurement based on SSB/SMTC is preformed when UE switches to SCell, however, the CQI/L1-RSRP is reported when UE switches to PCell.

Proposal 7: There is no issue on CQI/L1-RSRP reporting during SDL SCell activation in LB-CA.

Issue 1-3-6: BFD/CBD requirements for SDL SCell - Proposal 3 (LGE): RAN4 to consider not to apply switching pattern until link recovery is completed after beam failure is declared for SDL SCell, and allow scheduling restriction/interruption for CBD evaluation after beam failure is triggered. - FFS in maintenance stage for P3.

In the last RAN4 meeting, it was agreed to define requirement for BFD/CBD for SDL SCell in LB CA via switching. In our view, it is not necessary features for FR1 operation below 1GHz. It may not be used in practical deployment. Therefore, it would be not necessarily to enhance CBD measurement further.

Proposal 8: There is no need to enhance CBD measurement requirements for SDL SCell.

## 2.2 FDD PCell requirements

Issue 1-4-1: RLM requirement for PCell - FFS in maintenance stage: RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritize to receive RS for faster link recovery. Principle of scheduling restriction or interruption requirement agreed for the SCell activation to be reused for the link recovery procedure.

For RLM requirement for PCell, further enhancement can be further considered. When there is bad radio link quality on PCell, the SCell operation is also highly impacted. There will be no uplink for ACK/NACK feedback. If SCell is scheduled by PCell, then there will also be no DL scheduling on SCell. Therefore, it is beneficial to prioritize the procedure of radio link recovery.

Then it comes to the question when the switching pattern is NOT applied and when it is applied again. Using timer T310 is one potential options. When timer T310 starts after the UE receives N310 consecutive OOS indications, the switching pattern is not applied anymore. After the UE receives N311 IS indications, timer T310 stops and the switching pattern can be applied again. With this approach, the radio link recovery procedure can be fastened. Furthermore, no interruption requirements need to be specified.

For CBD procedure, similar to SDL SCell we don’t think it is necessary feature and further enhancement is not needed.

Proposal 9: Radio link recovery procedure is enhanced for LB-CA via switching by not applying switching pattern when timer T310 starts and applying switching pattern when timer T310 stops.

Proposal 10: There is no need to enhance CBD measurement requirements for FDD PCell.

# Summary

In this contribution, we further provided views on remaining issues for RRM core requirements for low band carrier aggregation with switching. Following proposals are provided.

Proposal 1: No additional time is needed for applying switching pattern, i.e., TLBCA is 0, for both MAC-CE and RRC triggered SCell activation.

Proposal 2: If UE should be on PCell according to switching pattern after THARQ+3ms or TRRC_Process after MAC-CE or RRC based SCell activation PDSCH is received, UE performs switching according to the configured switching pattern.

Proposal 3: If UE should be on SCell according to switching pattern after THARQ+3ms or TRRC_Process after MAC-CE or RRC based SCell activation PDSCH is received, UE switches to SCell at the next switching period or UE performs switching based on dwell time on SCell.

Proposal 4: No additional time is needed for NOT applying switching pattern, i.e., TLBCA is 0, for both MAC-CE and timer triggered SCell deactivation.

Proposal 5: The switching pattern is NOT applied after UE completed the SCell deactivation at n+ ((THARQ+3ms)/slot-length) for MAC-CE based SCell deactivation or at n+(3ms/slot-length) for timer triggered SCell deactivation.

Proposal 6: After completion of SCell deactivation, if UE is on PCell, UE keeps staying on PCell. If UE is on SCell, UE switches to PCell immediately.

Proposal 7: There is no issue on CQI/L1-RSRP reporting during SDL SCell activation in LB-CA.

Proposal 8: There is no need to enhance CBD measurement requirements for SDL SCell.

Proposal 9: Radio link recovery procedure is enhanced for LB-CA via switching by not applying switching pattern when timer T310 starts and applying switching pattern when timer T310 stops.

Proposal 10: There is no need to enhance CBD measurement requirements for FDD PCell.

- References

- R4-2512119	WF on RRM requirements for NR_LBCA_Sw, Apple

- R4-2500828 Discussion on RRM requirements for low band carrier aggregation via switching, vivo

- R4-2504028 Further discussion on RRM requirements for LB CA via switching, vivo

- R4-2506454	Further discussion on RRM requirements for LB CA via switching, vivo

- R4-2510323	Further discussion on RRM requirements for LB CA via switching, vivo

- R4-1805968	WF for NR SCell activation delay requirement, Intel Corporation

- R1-2504869	LS on Low NR band carrier aggregation via switching, RAN1