# 3GPP TSG-RAN WG4 Meeting #116bis                                 R4-2514146

# Prague, CZ, 13rd – 17th, Oct. 2025

# Title: 	Simulation results on NR SBFD demodulation requirements

# Source: 	ZTE Corporation, Sanechips

# Agenda item:	6.13.4

# Document for:	Discussion

# 1 Introduction

In RAN#102, a new R19 WI on Evolution of NR duplex operation: Sub-band full duplex (SBFD) was approved, and the revised WID at RAN#104 can be found in [1].

In this contribution, we give some initial simulation results on BS demodulation requirements.

# 2 Discussion

In last meeting, RAN4 discussed how to evaluate the performance of interference cancellation for BS side.  Currently, RAN4 agreed to evaluate PUSCH performance with UL muting under inter-site gNB interference modelling. However, how to model interference modeling for performance evaluation is still open. Some potential approaches are discussed in the following. Considering the processing of baseband, our simulations are based the INR values, which can be found in the test cases.

- Agreement - Considering TDL+INR type of interference model for evaluation - Note: the RF assumption can be considered for interference modelling - The following candidate value for INR can be considered for evaluation as starting point - Option 1: based on link budget analysis Conditions; FR1. Interf1 > Interf2; INR1 <= 17.2dB, INR2 <= 7.2dB Trial simulation could try following sets: [INR1, INR2] = {[5, -5], [9, -1], [13, 3], [17, 7]} dB. Interf1 = Interf2; INR1 = INR2 <=11.2dB Trial simulation could try following sets: [INR1, INR2] = {[-1, -1], [3, 3], [7, 7], [11, 11]} dB. Note: The interference wave form could use PUSCH with 1+1 DM-RS and CP-OFDM to simplify simulation setup.. - Other options are not concluded - Interesting companies can provide the system level simulation to derive the INR value

The detail simulation assumptions as shown in following Table 2-1.

We evaluated the baseline and enhanced scenarios with different muting symbols and configurations, and different receive antennas.

The SNR corresponding to 70% maximum throughput is shown in the following.

| TC# | Duplex Mode and SCS | CHBW | MIMO | Rank | MCS | INR1 | INR2 | Channel Model | Muting symbol | Muting Configuration | Scenario | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 30 kHz SCS | 40 MHz(20MHz DL sub-band+ 20MHz UL sub-band) | 1T2R | 1 | 16 | 5 | -5 | TDLC 300-100 ULA Low | 1 symbol | [3] | Baseline | 13.0 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 12.1 |
| 2 |  |  | 1T4R |  |  | 5 | -5 |  | 1 symbol | [3] | Baseline | 8.1 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 7.2 |
| 4 |  |  | 1T2R |  |  | 5 | -5 |  | 2 symbols | [3,12] | Baseline | 12.9 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 11.1 |
| 5 |  |  | 1T4R |  |  | 5 | -5 |  | 2 symbols | [3,12] | Baseline | 7.4 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 6.1 |

| TC# | Duplex Mode and SCS | CHBW | MIMO | Rank | MCS | INR1 | INR2 | Channel Model | Muting symbol | Muting Configuration | Scenario | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 30 kHz SCS | 40 MHz(20MHz DL sub-band+ 20MHz UL sub-band) | 1T2R | 1 | 16 | 17 | 7 | TDLC 300-100 ULA Low | 1 symbol | [3] | Baseline | 21.7 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 19.5 |
| 2 |  |  | 1T4R |  |  | 17 | 7 |  | 1 symbol | [3] | Baseline | 16.8 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 11.2 |
| 4 |  |  | 1T2R |  |  | 17 | 7 |  | 2 symbols | [3,12] | Baseline | 21.1 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 19.5 |
| 5 |  |  | 1T4R |  |  | 17 | 7 |  | 2 symbols | [3,12] | Baseline | 15.01 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 10.7 |

| TC# | Duplex Mode and SCS | CHBW | MIMO | Rank | MCS | INR1 | INR2 | Channel Model | Muting symbol | Muting Configuration | Scenario | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 30 kHz SCS | 40 MHz(20MHz DL sub-band+ 20MHz UL sub-band) | 1T2R | 1 | 16 | -1 | -1 | TDLC 300-100 ULA Low | 1 symbol | [3] | Baseline | 12.0 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 10.8 |
| 2 |  |  | 1T4R |  |  | -1 | -1 |  | 1 symbol | [3] | Baseline | 7.7 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 6.2 |
| 4 |  |  | 1T2R |  |  | -1 | -1 |  | 2 symbols | [3,12] | Baseline | 11.7 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 10.8 |
| 5 |  |  | 1T4R |  |  | -1 | -1 |  | 2 symbols | [3,12] | Baseline | 6.6 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 5.7 |

| TC# | Duplex Mode and SCS | CHBW | MIMO | Rank | MCS | INR1 | INR2 | Channel Model | Muting symbol | Muting Configuration | Scenario | SNR |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 30 kHz SCS | 40 MHz(20MHz DL sub-band+ 20MHz UL sub-band) | 1T2R | 1 | 16 | 11 | 11 | TDLC 300-100 ULA Low | 1 symbol | [3] | Baseline | 13.5 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 13.0 |
| 2 |  |  | 1T4R |  |  | 11 | 11 |  | 1 symbol | [3] | Baseline | 7.2 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 6.9 |
| 4 |  |  | 1T2R |  |  | 11 | 11 |  | 2 symbols | [3,12] | Baseline | 13.5 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 13.0 |
| 5 |  |  | 1T4R |  |  | 11 | 11 |  | 2 symbols | [3,12] | Baseline | 6.7 |
|  |  |  |  |  |  |  |  |  |  |  | Enhanced | 6.5 |

Observation 1. Based on our simulation results, the significant difference is that a gap of more than 1dB gap is observed between 1 symbol and two symbols muting schemes. Also, no difference is observed between 1 symbol and two symbols muting schemes in some test cases.

Observation 2. Based on our simulation results, muting symbol scheme can achieve a better performance compared to muting disable scheme, especially when using a higher interference levels.

Observation 3. Based on our simulation results, the receiver’s ability to suppress interference is affected by the level of interference.

# 3 Conclusion

In this contribution, we provided the simulation results for SBFD demodulation performance requirements.

Observation 1. Based on our simulation results, the significant difference is that a gap of more than 1dB gap is observed between 1 symbol and two symbols muting schemes. Also, no difference is observed between 1 symbol and two symbols muting schemes in some test cases.

Observation 2. Based on our simulation results, muting symbol scheme can achieve a better performance compared to muting disable scheme, especially when using a higher interference level.

Observation 3. Based on our simulation results, the receiver’s ability to suppress interference is affected by the level of interference.

# 4 References

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD), Huawei.

- R4-2512615, Way Forward for [116][328] NR_duplex_evo_demod, Samsung.