3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514162

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on RRM core requirements for SBFD

Source: 	Huawei, HiSilicon

Agenda item:	4.19.3

Document for:	Discussion

Introduction

RRM requirements for SBFD are discussed in RAN4#116, and the outcomes are captured in WF [1]. In our view, following issues need to be further discussed.

- Counting of unavailable occasions

- RACH requirements

In this paper, we will provide our views on remaining issues in RRM core requirements in SBFD.

Discussion

Counting of unavailable occasions

For UE L1 CLI measurement, it is agreed to define requirements based on valid symbol type, and re-use NR-U approach to extend measurement period due to invalid symbol type. In a similar manner, for CSI-RS based L1 measurement, it is agreed to re-use NR-U approach to extend measurement period due to invalid symbol type and collision with UL transmission.

In NR-U approach, the measurement delay is extended by counting the number of unavailable occasions. We identify 2 issues with the counting as discussed below, which are common for both UE L1 CLI measurement and CSI-RS based L1 measurement. We will use the L1-SRS-RSRP measurement as example, and the requirements are copied below.

For the value of L1, 1> If UE does not support supportSBFD or SBFD is not configured by the network 2> L1=0 1> else if UE supports supportSBFD and SBFD is configured by the network 2> if higher layer parameter timeRestrictionForChannelMeasurement is configured 3> L1=0 2> else 3> if UE is configured to report L1-SRS-RSRP for SBFD symbols 4> When DRX is not configured, L1 is the number of occasions of the L1-SRS-RSRP resource that are overlapping with non-SBFD symbols during TL1-SRS-RSRP_measurement_period 4> When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of the L1-SRS-RSRP resource is overlapping with non-SBFD symbols during TL1-SRS-RSRP_measurement_period 3> if UE is configured to reportL1-SRS-RSRP for non-SBFD symbols 4> When DRX is not configured, L1 is the number of occasions of the L1-SRS-RSRP resource that are overlapping with SBFD symbols, during TL1-SRS-RSRP_measurement_period 4> When DRX is configured, L1 is the number of DRX cycles in which at least one occasion of theL1-SRS-RSRP resource is overlapping with SBFD symbols, during TL1-SRS-RSRP_measurement_period Table 9.x.2.5-1 Measurement period TL1_SRS_RSRP_measurement_period Configuration; TL1_SRS_measurement_period (ms). No DRX; Max(TReport, (M +L1) X TSRS). DRX cycle ≤ 320 ms; Max(TReport, Ceil(1.5 X (M +L1)) X max(TSRS, TDRX)). DRX cycle > 320 ms; (M +L1) X TDRX. NOTE 1: TSRS is SRS measurement periodicity configured SRS-PeriodicityAndOffset, and TDRX is the DRX cycle length. TReport is configured periodicity for reporting. NOTE 2: M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise..

DRX case

One issue is the delay extension in case of DRX as highlighted in yellow. The extension happens when one SRS resource has occasions on both SBFD and non-SBFD symbols. It occurs when the SRS periodicity cannot be divided by the SBFD periodicity, e.g. SRS period is 8ms and SBFD period is 5ms. In a period equal to LCM (least common multiple) of SRS period and SBFD period (40ms in the example), some of the 5 SRS resource occasions may be on SBFD symbols, and the others are on non-SBFD symbols.

Following current highlighted requirements, if there exists one invalid occasion within a DRX cycle, the whole DRX cycle would be dropped from requirement perspective. It means when DRX cycle is >= 40ms, the DRX cycle would be always dropped, and the measurement delay would extend infinitely.

The yellow highlighted requirements are re-used from NR-U. In NR-U, it is reasonable because UE does not know which measurement occasion are valid and which are not in a DRX cycle. It could happen that UE has scheduled measurement on an occasion but due to LBT the reference resource is not transmitted by the NW. It is unreasonable to require the UE to try another occasion within the same DRX cycle as it would reduce the power saving gain of DRX. However, here UE would have known the validity of measurement occasions upon configuration, and should schedule measurement on a valid occasion within a DRX cycle.

Therefore, we do not see the need to drop a DRX cycle just because there exists one invalid occasion within the DRX cycle. Instead, the DRX cycle should be dropped only when all measurement occasions are invalid. The yellow highlighted requirements should be updated as in Proposal 1.

Proposal 1: Update the measurement period requirements for UE L1 CLI measurement and CSI-RS based L1 measurement for DRX case such that L is the number of DRX cycles in which at least one all occasions of the measurement resources are overlapping with invalid symbols.

The CSI-RS based L1-RSRP and L1-SINR measurement period is extended in SBFD scenario due to 1) CSI-RS resource colliding with dynamic UL transmission, and 2) CSI-RS resource colliding with invalid symbol type. In this case, the counting can be updated such that L is the number of DRX cycles in which at least one occasion of the CSI-RS resource is overlapping with dynamic UL transmission or in which at least one all occasions of the CSI-RS resource are overlapping with invalid symbols during the measurement period.

M=1 case

Another issue is the understanding of L = 0 in case M = 1. As highlighted in cyan, there are 2 cases where L = 0 in current requirements, and they have different meaning. In the first case (non-SBFD case), the L = 0 means the measurement is performed in M samples without any further extension. In the second case (M=1), L = 0 means the measurement is not performed (it is a single sample measurement and the sample is invalid).

We suggest 2 alternatives to make the requirements clear. One is to apply the way of L counting for M = 3 to M = 1, and this means L is no longer fixed to zero when M  = 1. Another alternative is to clarify that when M = 1, L = 0 means the measurement is not performed, for example in a similar wording as SL positioning delay requirements, e.g. in clause 12A.4.5.

A UE may drop one or more SL-PRS measurement samples if the number of active slots or the number of active resources per slot for the ongoing SL-PRS measurement exceed the UE capabilities reported by UE via sl-PRS-CommonProcCapabilityPerBand specified in TS 38.355 [37]. For a single-sample measurement, the whole measurement may not be performed.

Proposal 2: Adopt one of the following alternatives to clarify the meaning of L1 = 0 when M = 1.

- Alt1: apply the way of L1 counting for M = 3 to M = 1

- Alt2: clarify that when M = 1, L1 = 0 means the measurement is not performed

RACH requirements

In RAN4#115, some companies suggested to update RACH requirements to reflect the RO type change (from legacy RO to additional RO and vice versa) at PRACH transmission re-attempt. In our view this is a mechanism introduced by RAN2 and has already been clearly captured in RAN2 spec, so we do not see the need for RAN4 to update RACH requirements.

In RAN2 MAC CR [2], in 5.1.2 it is specified that the RO selection is based on the selected RO type, and RO type selection is defined in 5.1.1 (initial selection) and 5.1.4 (switch based on attempt number). Current RAN4 requirements are referring to 5.1.2, which means the RO type switching is already considered.

###### 6.2.2.2.1.2 Correct behaviour when receiving Random Access Response The UE may stop monitoring for Random Access Response(s) and shall transmit the msg3 if the Random Access Response contains a Random Access Preamble identifier corresponding to the transmitted Random Access Preamble. The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if all received Random Access Responses contain Random Access Preamble identifiers that do not match the transmitted Random Access Preamble. ###### 6.2.2.2.1.3 Correct behaviour when not receiving Random Access Response The UE shall again perform the Random Access Resource selection procedure defined in clause 5.1.2 in TS 38.321 [7], and transmit with the calculated PRACH transmission power when the backoff time expires if no Random Access Response is received within the RA Response window defined in clause 5.1.4 in TS 38.321 [7].

We do not prefer to duplicate too much in RAN2 spec in RAN4 requirements especially when it is only about UE behaviour without performance impacts.

Proposal 3: RAN4 not to update RACH requirements for SBFD.

Conclusions

In this paper we provided our views on remaining issues in RRM core requirements in SBFD.

Proposal 1: Update the measurement period requirements for UE L1 CLI measurement and CSI-RS based L1 measurement for DRX case such that L is the number of DRX cycles in which at least one all occasions of the measurement resources are overlapping with invalid symbols.

Proposal 2: Adopt one of the following alternatives to clarify the meaning of L1 = 0 when M = 1.

- Alt1: apply the way of L1 counting for M = 3 to M = 1

- Alt2: clarify that when M = 1, L1 = 0 means the measurement is not performed

Proposal 3: RAN4 not to update RACH requirements for SBFD.

Reference

- R4-2512133, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R2-2505575, Introduction of Rel-19 Evolution of NR duplex operation (SBFD) for MAC spec,	Samsung