3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514167

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on RRM core requirements for LP-WUR

Source: 	Huawei, HiSilicon

Agenda item:	4.22.3

Document for:	Discussion

Introduction

RRM requirements related to LP-WUR operation are discussed in RAN4#116, and the outcomes are captured in WF [1]. In our view, there are some issues need to be discussed in the maintenance phase:

- Interworking with other features

- Requirements at exiting LP-WUR operation

- MR measurement at RRM relaxation mode

- LR measurement with opposite sequence

In this paper, we will provide our views on remaining issues in RRM requirements related to LP-WUR operation.

Discussion

Interworking with other features

R18 EMR

In [1] the interworking between LR operation with R16 EMR was concluded.

Issue 1-1-16: LP-WUR operation with EMR Agreement: When both Rel-16 EMR and Rel-19 LP-WUR are configured: When T331 is running - When Case#1 (RRM offloading) conditions are met: - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s). - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s). - When Case#3 (RRM relaxation) conditions are met: - If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ - The UE shall search for NR inter-frequency layers configured for idle mode CA/DC measurements by following the higher priority frequency measurement requirements (i.e., based on 60s). - Higher priority NR inter-frequency layers follow the higher priority frequency measurement requirements (i.e., based on 60s). - if Srxlev ≤ SnonIntraSearchP or Squal ≤ SnonIntraSearchQ - the UE shall search for NR inter-frequency layers configured and not configured for idle mode CA/DC measurements by following the legacy measurement requirements (no relaxation) when T331 is not running, - Serving cell, intra-frequency and NR inter-frequency layers not configured for idle mode CA/DC measurements and NR inter-frequency layers configured for idle mode CA/DC measurements and mobility measurement follow corresponding agreed requirements defined in case 1 and case 3 when they are satisfied.

There were also proposals on R18 EMR, but no conclusion on whether there is any spec impact due to interworking with it. Our view is that there is no spec impact.

According to clause 4.7.1, the measurement requirements are not changed due to R18 EMR, i.e.

- for carriers configured via measIdleCarrierListNR-r16 and/or measIdleCarrierListEUTRA-r16 the requirements for R16 EMR should apply, and T331 is applicable;

- for carriers configured via measReselectionCarrierListNR-r18 the requirements for inter-frequency reselection should apply, and T331 is not applicable.

A UE supporting measValidationReportEMR-r18 and idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16 shall report the results of the idle mode measurement which is specified in clause 4.4, and according to the measurement reporting requirements specified in clause 4.7.3. A UE supporting measValidationReportReselectionMeasurements-r18 shall report the results of the idle mode measurement specified in the clause 4.2.2, and according to the measurement reporting requirements specified in the clause 4.7.3.

Based on this understanding, the agreement in [1] is enough for interworking with R18 EMR. It is noted that R16 EMR is a subset of R18 EMR because the signalling for R16 EMR is also used in R18 EMR. When R16 EMR including T331 is configured as part of R18 EMR, the agreement in [1] applies. When R16 EMR is not configured as part of R18 EMR, the requirements for inter-frequency cell reselection defined in R19 apply. In any case, we do not see further spec change needed.

Proposal 1: RAN4 confirms that no spec change is needed for R18 EMR.

SDT

In RAN4#116 some companies proposed to discuss interworking between SDT to LR operation. On the other hand, in RAN2#131 following are agreed related to SDT.

RAN2 confirm that the available UL occasions (e.g. SR occasion, RACH occasion, CG occasion) are MR-ready. Can further check whether any spec change is needed. Confirm that SDT can be initiated while UE is monitoring LP-WUS, and there is no impact to the SDT procedure. Can check if any spec change is needed.

In our view, there is no spec impact due to interworking with SDT, and the existing SDT requirements also apply with LR operation. Before UL transmission, UE needs to obtain DL sync, and perform measurement to check whether conditions including TA validation for SDT are met. Similar to UL transmission, UE should timely wake up MR for SDT related actions. This of course may reduce the power saving gain, but UL data should be more important than power saving.

We also do not see the need to use LR measurement for SDT. UE needs to wake up MR to perform DL sync as the LR sync accuracy may be insufficient for UL transmission. Also, the SDT related thresholds are based on MR, and so far, comparing LR measurement against MR threshold is not supported in RAN2.

Proposal 2: RAN4 confirms that no spec change is needed for SDT.

R16/R17 relaxation

In RAN4#116, following is agreed related to interworking with R16 power saving based on not at cell edge.

Issue 1-2-12: UE behaviro when both Rel-16 relaxation and Rel-19 LP-WUR offloading/relaxation are satisfied; or when both Rel-17 relaxation for Redcap and Rel-19 LP-WUR offloading/relaxation are satisfied Agreement: UE is allowed to follow the most relaxation requirements when only Rel-16 not at cell edge is configured, and Rel-16 not at cell edge and Rel-19 LP-WUR offloading/relaxation are both satisfied and Rel-16 EMR is not configured or Rel-16 EMR is configured and T331 is not running.

There is no conclusion for the interworking with power saving based on low mobility, stationary, not at cell edge while stationary, and the combination of them with or without not at cell edge. We prefer to apply the agreement in [1].

The case under discussion is when both R19 LR operation and R16/R17 power saving based on low mobility are configured, and R19 criteria are met but R16/R17 low mobility criteria are not. In this case, if UE is not allowed to relax measurement, there could be some inconsistencies.

- It could happen that UE1 supports only R19 LR operation, and UE2 supports both R19 LR operation and R16/17 power saving. UE1 could enjoy the power saving from relaxation but UE2 cannot just because it supports more features than UE1.

- It could happen that NW1 configures R16/R17 power saving based on low mobility and NW2 does not. Same UE would enjoy the power saving from relaxation in NW2 but not in NW1 just because NW1 wants to enable R16/R17 UE to achieve power saving. R16/R17 power saving is R16/R17 features, so the motivation is to enable R16/R17 UE to achieve power saving rather than limit R19 LR operation.

On the other hand, we can understand that there is also an inconsistency that a UE is not allowed 3 times relaxation following R16/R17 criteria but same UE is allowed 16 times relaxation following R19 criteria. However, this can be handled by NW implementation, e.g. NW can configure the R19 thresholds high enough so that UE would already exit LR operation when it moves to cell edge.

Proposal 3: UE is allowed to follow the relaxed requirements based on R19 LR operation, when both R19 LR operation and R16/R17 power saving based on low mobility are configured, and R19 criteria are met but R16/R17 low mobility criteria are not.

Requirements at exiting LP-WUR operation

Wakeup time when exiting RRM relaxation

InRAN4#116, it is agreed to re-use the existing wakeup delay when UE exits RRM offloading.

Issue 1-5-3: MR wake up delay after exiting case 1 due to offloading conditions cannot be met Agreement: When exiting case 1 due to exiting conditions of offloading is met, MR reuse the existing wake up delay.

There is no conclusion for the case when UE exits RRM relaxation. Our view is that existing wakeup delay should be also re-used when UE exits RRM relaxation. In RAN2#131, following is agreed, and the entry condition for RRM relaxation is based on MR measurement and optionally LR measurement.

The exit condition for RRM relaxation is defined as ‘not fulfilling the entry condition’.

When LR threshold for exit condition is configured, it could happen that exiting RRM relaxation is triggered by LR measurement. In our view, this should be the typical case in real NW because LR measurement is much more frequent than MR measurement. In this case, MR would be in sleep mode in between the relaxed measurement. The measurement interval with 16 times relaxation is 20.48s with 1.28s DRX cycle, and it is up to UE implementation whether the MR goes into deep sleep mode or ultra-deep sleep mode, but in any case, the wakeup time is needed and the existing wakeup delay could be re-used. It is noted that the UE capability for the wakeup delay does not differentiate whether UE is in RRM relaxation or RRM offloading mode, and the wakeup delay after LP-WUS detection also applies when UE is in RRM relaxation mode.

Proposal 4: The existing wakeup delay is re-used when UE exits RRM relaxation mode due to exit condition being met.

MR measurement when exiting RRM offloading

In RAN4#116, there were discussions on MR measurement when exiting RRM offloading. When both RRM relaxation and RRM offloading are configured, and when exit condition for RRM offloading is met, it needs to be clarified whether MR will perform normal (1 DRX cycle) or relaxed (16 DRX cycle) measurement.

Several options were proposed in RAN4#116, and our view is that MR will perform normal measurement. In RRM offloading mode MR will not perform measurement, so when exiting RRM offloading the UE cannot determine whether MR measurement can fulfil the entry condition of RRM relaxation. MR measurement is mandatory for evaluating entry condition of RRM relaxation, and UE should exit RRM relaxation when the entry condition is not met.

In this sense, the safest way from mobility perspective is to assume MR measurement cannot fulfil the entry condition. MR will perform normal measurement, and if based on measurement it finds the entry condition of RRM relaxation is met, UE can enter RRM relaxation mode again. This may reduce the power saving gain if the entry condition can be actually met, but we do not think it is a big issue as it only impacts a small time.

Proposal 5: After exiting RRM offloading due to exit condition being met, UE should by default meet legacy (non-relaxed) requirements for MR measurement.

MR measurement at RRM relaxation mode

One issue identified in RAN4#116 during CR drafting is the measurement separation at RRM relaxation mode. In this mode, the measurement interval is 16 DRX cycles, and the question is whether the separation between 2 samples used for filtering should be half DRX cycle (same as legacy) or 8 DRX cycle (half of the measurement interval).

The LP-WUS UE shall filter the SS-RSRP and SS-RSRQ measurements of the serving cell using at least 2 measurements. Within the set of measurements used for the filtering, at least two measurements shall be spaced by either 8 DRX cycles or DRX cycle/2.

We support to use 8 DRX cycle. In our view, the rationale of this requirement is to make sure the 2 samples used for filtering have enough separation so that the filtered result can represent the radio condition across multiple measurement intervals. In this sense, it is reasonable to scale the measurement separation with the measurement interval. This is also aligned with the principle of existing requirements, e.g. when eDRX <= 10.24s is used, the measurement separation is defined as eDRX cycle/2.

Proposal 6: In RRM relaxation mode, the 2 samples used for filtering should be spaced by at least 8 DRX cycles.

LR measurement with opposite sequence

In RAN4#116, the impact of opposite binary sequence on LR measurement performance is discussed but without conclusion. Our simulation results are shown in Table 1 in Annex for all cases supported by RAN1, and we find that the performance for M=1, L=6 is particularly poor, especially in TDL-C channel. The specific of this case is that the binary sequence for the serving cell and interfering cell are opposite to each other. It can be seen that the target accuracy of 3.5dB cannot be reached even with 4 samples in TDL-C.

For M=1, L=6 (if supported), the set of LP-SS sequence is: Serving cell: [1 0 1 0 1 0] Interference cell: [0 1 0 1 0 1]

We further checked the reason for the poor performance. We used the basic RSRP calculation method in the simulation where RSRP is calculated by total power on the ON symbols minus that on the OFF symbols.

- For AWGN channel, the interference is always 6dB lower than the signal, so ideally the RSRP error would be 10*log(0.75)=-1.25dB, and we also find this is achievable when noise is small enough.

- For fading channel, when we are looking at 5% or 95%-tile of the error CDF, the interference is quite close to the signal due to the instantaneous channel fading of the service link and interference link. In this case, the RSRP error would be close to 10*log(0)=-inf.

In our view, there are 2 approaches for to address such cases with opposite sequence.

- Alt 1: define requirements for all combinations, and the requirements are based those with opposite sequence (worst case)

- Alt 2: define the requirements without considering combinations with opposite sequence, and the requirements are not applicable for those with opposite sequence

We suggest to adopt Alt 2 because the combinations with opposite sequence are in minority, but they will drag down the requirements significantly (longer delay plus degraded accuracy). In RAN#116, there were 2 concerns raised by companies with Alt 2.

- It should be made clear which requirements are not applicable. The core requirements include evaluation delay and margin (implicit accuracy). In our view, it is the accuracy performance that is impacted, and further enlarging the delay may not help, so a larger margin may be needed.

- It should be made clear that the issue only valid when the LP-SS from serving and neighbor cells are time and frequency aligned. We agree with the comment, and it is true that the interference issue occurs only when LP-SS from serving and neighbor cells are time and frequency aligned. This means NW can distribute the LP-SS from different cells in TDM or FDM manner if it wants to employ opposite binary sequences in neighboring cells and avoid the performance degradation.

Proposal 7: Capture the following note in LP-SS measurement requirements:

Note: higher margin may be required for UE to consider an entry or exit criteria fulfilled if serving cell and neighbour cell are using opposite binary sequences for LP-SS, and LP-SS are transmitted on the same time and frequency resource in the two cells.

Conclusions

In this paper we provided our views on remaining issues in RRM requirements related to LP-WUR operation.

Proposal 1: RAN4 confirms that no spec change is needed for R18 EMR.

Proposal 2: RAN4 confirms that no spec change is needed for SDT.

Proposal 3: UE is allowed to follow the relaxed requirements based on R19 LR operation, when both R19 LR operation and R16/R17 power saving based on low mobility are configured, and R19 criteria are met but R16/R17 low mobility criteria are not.

Proposal 4: The existing wakeup delay is re-used when UE exits RRM relaxation mode due to exit condition being met.

Proposal 5: After exiting RRM offloading due to exit condition being met, UE should by default meet legacy (non-relaxed) requirements for MR measurement.

Proposal 6: In RRM relaxation mode, the 2 samples used for filtering should be spaced by at least 8 DRX cycles.

Proposal 7: Capture the following note in LP-SS measurement requirements:

Note: higher margin may be required for UE to consider an entry or exit criteria fulfilled if serving cell and neighbour cell are using opposite binary sequences for LP-SS, and LP-SS are transmitted on the same time and frequency resource in the two cells.

Reference

- R4-2512139, WF on RRM requirements for NR_LPWUS, Moderator (vivo)

Annex

In the Annex, we provide our simulation results for LR measurement based on LP-SS. They are same as what we provided in RAN4#116 and reproduced here for reading convenience.

The parameters selected for simulations are as follows.

- Serving cell SINR: -3dB

  - Serving cell SNR = -2.4dB, interfering cell SNR = -8.4dB, i.e. 6dB power difference

- Sample number: 1, 2, 3, 4

- Sampling rate: 7.68MHz

- ADC bit width: 8bit

- Filter: 3th Order Butterworth with 3.96MHz bandwidth

- Interference type: LP-SS, interfering cell is fully synchronous to serving cell, binary sequence for LP-SS is independent for serving cell and interfering cell

The results in this section are for OOK based LR, i.e. without utilizing the overlaid sequence on LP-SS. Our updated simulation results for LP-SS are given in Table 1.

Table 1: Simulation results for LP-SS

| {M,L} | LP-SS Binary Sequence | Measurement Accuracy [dB] vs Number of Samples X |  |  |  |  |  |  |  | SINR(dB) | channel | Interference Source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | RSRP error |  |  |  | RSRQ error |  |  |  |  |  |  |
|  |  | X=1 | X=2 | X=3 | X=4 | X=1 | X=2 | X=3 | X=4 |  |  |  |
| {1,6} | Opposite sequence | 3.42 | 2.63 | 2.49 | 2.42 | 3.47 | 2.73 | 2.58 | 2.47 | -3 serving cell SNR = -2.4dB neighbor cell SNR = -8.4dB | AWGN | LP-SS |
| {1,8} |  | 2.93 | 2.34 | 2.29 | 2.28 | 2.66 | 2.06 | 1.98 | 1.98 |  |  |  |
| {2,12} |  | 2.76 | 2.09 | 2.03 | 2.00 | 2.43 | 1.78 | 1.72 | 1.67 |  |  |  |
| {2,16} |  | 2.35 | 1.68 | 1.63 | 1.60 | 2.19 | 1.46 | 1.42 | 1.35 |  |  |  |
| {4,16} |  | 3.10 | 2.34 | 2.26 | 2.24 | 2.64 | 1.98 | 1.88 | 1.84 |  |  |  |
| {4,32} |  | 2.33 | 1.69 | 1.66 | 1.63 | 2.24 | 1.50 | 1.45 | 1.41 |  |  |  |
| {1,6} | Opposite sequence | 30.39 | 6.31 | 4.68 | 4.07 | 30.65 | 6.46 | 5.25 | 4.22 |  | TDL-C |  |
| {1,8} |  | 4.21 | 2.57 | 2.35 | 2.17 | 3.93 | 2.33 | 2.22 | 1.99 |  |  |  |
| {2,12} |  | 3.94 | 2.23 | 1.95 | 1.94 | 3.89 | 2.10 | 1.88 | 1.74 |  |  |  |
| {2,16} |  | 3.56 | 1.95 | 1.91 | 1.72 | 3.52 | 1.91 | 1.83 | 1.69 |  |  |  |
| {4,16} |  | 4.06 | 2.53 | 2.23 | 2.10 | 3.85 | 2.28 | 2.05 | 1.87 |  |  |  |
| {4,32} |  | 3.63 | 2.11 | 1.95 | 1.73 | 3.63 | 2.09 | 1.96 | 1.81 |  |  |  |