3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514173

Prague, Czech Republic, 13 – 17 October, 2025

Title: 	Discussion on performance requirements for RedCap in NTN

Source: 	Huawei, HiSilicon

Agenda item:	6.19.3.1

Document for:	Discussion

Introduction

RRM requirements for RedCap NTN are discussed in RAN4#116, and the outcomes are captured in WF [1]. As the core part of the WI is closed, RAN4 needs to work on the performance part for the objective..

In this paper, we will provide our views on RRM performance requirements for RedCap NTN.

Discussion

One work for the performance part is to define accuracy requirements for measurements that RedCap UE would perform under NTN. Based on accuracy requirements defined in R17 for RedCap and for NTN, it is straightforward to define those listed in Proposal 1.

Proposal 1: RAN4 to define accuracy requirements for following measurements for RedCap in NTN.

- Intra- and inter-frequency RSRP

- Intra- and inter-frequency RSRQ

- Intra- and inter-frequency SINR

- L1-RSRP

- Rx-Tx for NW verified location

During core part discussion, RAN4 has made agreements for accuracy requirements for all measurements listed above, so there is no need for further discussion.

For the test cases, our view is to use the current test cases for NTN in A.14 as the starting point. For 2Rx RedCap UE supporting NTN, the core requirements are almost same as normal UE in NTN, so such a UE should pass the existing test case for NTN.

On the other hand, RAN4 can check what additional test cases specific to RedCap UE need to be defined based on TN RedCap test cases in A.16.

- One additional aspect is 1Rx UE. In A.16, test cases for 1Rx and 2Rx UEs are defined in separate clauses. RAN4 can discuss whether to re-use this approach or to define a single test case for both 1Rx and 2Rx UE. The latter can help to avoid duplication in spec, and we can define different test requirements in a single test case, even the test case is defined using the “reference” approach.

- Positioning test cases are defined for RedCap in A.16. in R18, there was a discussion on whether to define test cases for UE Rx-Tx measurement for NW verified location, and in RAN4#111 it was agreed not to define. In our view, this principle should also apply for RedCap UE.

- CG-SDT test cases are defined for RedCap in A.16. As the test case are mainly verifying UE performs measurement for RSRP change based TA validation which is not applicable for NTN, there is no need to define this test case.

- Inter-RAT LTE measurement and mobility test cases are defined for RedCap in A.16. In NTN only cell reselection requirements are defined for LTE, and in R18 it was agreed not to define test case cell reselection to TN. In our view, this principle should also apply for RedCap UE.

- Test cases for NCD-SSB are defined for RedCap in A.16. For NTN it was agreed not to define specific requirements for NCD-SSB, so those test cases should not be defined.

Finally, RAN4 may need to discuss how to test HD-FDD specific requirements. For example, the collision handling between UL and CSI-RS L1 measurement is a specific requirement for HD-FDD. In existing test case for RedCap, HD-FDD is added as a test configuration, and if UE supports both FD-FDD and HD-FDD, UE is only required to pass the test with one of them. It means the HD-FDD specific requirements will not be tested if UE pass the test with FD-FDD.

One option to resolve the issue is to require UE to pass the test with HD-FDD in test cases which involve HD-FDD specific requirements, e.g. test case for L1 measurement with CSI-RS. Another option is to define new test cases for HD-FDD specific requirements so that FD- and HD-FDD are tested if UE supports both. We are open to either option and further discussion.

Based on above, we have the following proposal for RRM test cases for RedCap in NTN.

Proposal 2: Re-use existing test cases for NTN in A.14 for RedCap in NTN

- Further discuss how to test HD-FDD specific requirements

- Further discuss whether to define separate test cases for 1Rx and 2Rx RedCap UEs

Conclusions

In this paper we provided our views on RRM performance requirements for RedCap NTN.

Proposal 1: RAN4 to define accuracy requirements for following measurements for RedCap in NTN.

- Intra- and inter-frequency RSRP

- Intra- and inter-frequency RSRQ

- Intra- and inter-frequency SINR

- L1-RSRP

- Rx-Tx for NW verified location

Proposal 2: Re-use existing test cases for NTN in A.14 for RedCap in NTN

- Further discuss how to test HD-FDD specific requirements

- Further discuss whether to define separate test cases for 1Rx and 2Rx RedCap UEs

Reference

- R4-2512146, WF on RRM requirements for NR_NTN_Ph3_Part1, CATT