# 3GPP TSG RAN WG4 Meeting #116	bis 														R4-2514185

# Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	4.19.3

Source: 	Ericsson

Title:	RRM core requirement for Rel-19 Subband full duplex evolution

Document for:	Discussion

# Introduction

The agreements on SBFD operation during RAN4#116 meeting are captured in the approved WF [1].

In this paper, we provide our views on the LS from RAN2 [2]

# Discussion

## 2.1 RRM core requirements for SBFD operation

#### Issue 2-1-1b: CSI-RS L1 measurement – puncturing in frequency domain - Agreements (Monday online session) - For UE supporting simultaneous DL reception in 2 DL subbands, - For measurement period: - For UE indicating CSI processing time scaled by 2, - Add a lower bound of 8ms on top of existing measurement period, if the CSI-RS resource is across 2 DL subbands and UE indicates CSI processing time is scaled by 2. - For UE not indicating CSI processing time scaled by 2, - Lower bound is not needed and existing measurement period applies - Further discuss the measurement accuracy requirements in Perf part of the WI - For UE indicating CSI processing time scaled by 2 - Option 1: existing accuracy requirements based on 48 RB for Case 3 apply. - Option 2: accuracy requirements based on 24 RB for Case 3 apply. - For UE not indicating CSI processing time scaled by 2 - Option 1: accuracy requirements based on 24 RB for Case 3 apply. - For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

For the CSI-RS based L1 measurement, there is a dependency on RAN1 final agreement for whether simultaneous DL reception in 2 DL subbands is a mandatory UE feature.

Based on our understanding of the latest RAN1 agreement from the R1-2506409

Agreement: - FG 60-1 to include support of PDSCH/CSI-RS reception in two DL subbands Ø Add a new component to report the number of partial PRGs supported Candidate values: {2, 4} Ø Add a new component to report CSI timeline, active CSI-RS ports counting and active CSI-RS resource counting for a single CSI-RS resource across two DL subbands ² Candidate values: {“with factor-of-two relaxation”, “without factor-of-two relaxation”} ² Add a note: “FFS: details of counting” Ø Add a new component of “maximum of one DL-UL switching point per SBFD slot for actual DL/UL transmission(s)” Ø Add a new component of “Determination of TBS of PDSCH/PUSCH in SBFD symbols”

The PDSCH/CSI-RS reception in two DL subbands is supported as basic UE feature.  RAN4 do not need to specify separate requirements.

- Proposal 1: Based on RAN1 UE feature agreement, there is no need for RAN4 to update requirements regarding UE reception for PDSCH/CSI-RS across two DL subbands.

Issue 2-1-4: Condition for SSB based measurement with basic UE operation • Agreements o For UE not supporting simultaneous DL reception in 2 DL subbands, if RAN1 agrees such basic UE operation exists in maintenance phase, RAN4 will further the corresponding requirements in maintenance phase.

As discussed in previous issue, the basic UE feature has been agreed to support PDSCH/CSI-RS reception in two DL subbands, we think there is no need to discuss side conditions for SSB based measurement.

- Proposal 2: Based on RAN1 UE feature agreement, there is no need to further discuss condition for SSB based measurement.

RAN2 reply LS in supporting simultaneous configuration of SBFD and DC

Based on RAN2’s reply supporting simultaneous configuration of SBFD and inter-band DC will be limited specification impact. For RAN4 point of view, this will need to consider both the core requirement impact as well as the test case impact if RAN4 specify requirements for support SBFD and inter-band NR-DC. Since the core part discussion is already closed, RAN4 most likely have very limited time to study whether there will be impact, we think it is better not to rush into any requirements and do not specify support simultaneous SBFD and inter-band NR-DC.

- Proposal 3: RAN4 do not specify requirements for supporting inter-band NR-DC and simultaneous SBFD.

# Summary

The following have been proposed in the current contribution:

	Proposal 1: Based on RAN1 UE feature agreement, there is no need for RAN4 to update requirements regarding UE reception for PDSCH/CSI-RS across two DL subbands.

	Proposal 2: Based on RAN1 UE feature agreement, there is no need to further discuss condition for SSB based measurement.

	Proposal 3: RAN4 do not specify requirements for supporting inter-band NR-DC and simultaneous SBFD.

# References

- R4-2512133 “WF on SBFD RRM”, Huawei, August ,2025

- R2-25064927 “Reply LS on simultaneous configuration of SBFD and DC”, ZTE, August ,2025