3GPP TSG-RAN WG4 Meeting #116bis                                                           R4-2514208

Prague, Czech Republic, 13th – 17th October, 2025

Source:	Samsung

Title:	Discussion on OD-SSB SCell operation performance

Agenda item:			6.15.2.1

Document for:	Discussion

1	Introduction

In this meeting, we will continue giving some analyses on TC design including test scope and test configuration based on WF [1] agreed in last meeting.

2	Performance requirements for NES OD-SSB

2.1 Scenario

Last meeting, several candidates for scenarios were provided, copied as below

Issue 3-1-1: Scenario - General - RAN4 to further discuss the test case scope for scenario #2, scenario #2A and scenario #3B as follow. - Option 1: - In Case 1, RAN4 should define the test cases for scenario #2, scenario #2A. - In Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B. - The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell. - Option 2: RAN4 not to define test cases for scenario #3B. - Option 3: RAN4 to define the OD-SSB based test cases for - Case #1 and Scenario #2 - Case #2 and Scenario #2A - RAN4 to further discuss the test case scope for Case 2 as follow. - FFS: RAN4 to define the test cases to verify the UE behaviour in Alt. Time C1, Alt. Time C2 and scenario 3 if RAN4 agrees to define the RRM requirements for all these scenarios.

For OD-SSB, it is possible to consider TC for scenario #3B actually no matter for case 1 or case 2, although seems that the function to test in such scenario is similar to SSB adaptation brought to some extent, the main reason is that the measurement mechanism on activated SCell for OD-SSB and SSB adaptation are both use SMTC when the OD-SSB/SSB adaptation is activated and SCell is activated. However, main differences are: 1) The two SSB-related solutions (OD-SSB and SSB adaptation) are triggered in different ways, the former one is MAC-CE based, the later one is DCI based; 2) For the periodicity of the OD-SSB, multiple candidate values can be supported to cater different aims, but for SSB adaptation, up to two additional periodicities are supported; 3) For OD-SSB, Alt Time C1 or Alt Time C2 are imposed when AO-SSB is periodically transmitted on the cell. At the same time, we agreed that RAN4 not to define the requirements when OD-SSB and AO-SSB are within different frequencies in last meeting, that is scenario 3, so there is no need to define any TC for such scenario. Overall, regarding the scenario.

Proposal 1: For the test case scope for scenario #2, scenario #2A and scenario #3B, the following aspects can be considered:

- In Case 1 and Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

- Not to define the test cases to verify the UE behaviour in scenario 3

Issue 3-1-3: Scenario – known/unknown Scell activation - Proposal 1: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown SCell. - Proposal 2: RAN4 to define the OD-SSB based SCell activation test case for known SCell only. - Proposal 3: RAN4 to define the OD-SSB based test case for both Case 1 and Case 2. One in known scenario and the other in unknown scenario such as: - Case 1 in unknown scenario - Case 2 in known scenario

In core part, we defined OD-SSB based SCell activation requirement for both known SCell and unknown SCell, it is OK to define the OD-SSB based SCell activation test case for both known SCell and unknown SCell.

Proposal 2: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown SCell.

2.2 TC list

Issue 3-1-4: Test case list RAN4 to further discuss whether the following OD-SSB test cases can be agreed. - SCell activation in Case 2 Alt Time C-1 - OD-SSB based multiple SCell activation - OD-SSB based direct SCell Activation at SCell addition in Case 1 - OD-SSB based direct SCell Activation at SCell addition in Case 2 - OD-SSB based PUCCH SCell Activation in Case 1 - OD-SSB based PUCCH SCell Activation in Case 2 - Fast SCell Activation with OD-SSB; - OD-SSB based SCell Activation with the L3 reporting during activation - Intra-frequency measurement(deactivated SCell) in Case 2 - Intra-frequency measurement in Case 1 - Intra-frequency measurement in Case 2 - Interruption at OD-SSB activation/deactivation - Interruptions during measurements on deactivated NR SCC

Proposal 3: For NES OD-SSB, the following OD-SSB test cases can be defined:

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2 Alt Time C-1

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- OD-SSB based SCell Activation with the L3 reporting during activation in Case 1

- OD-SSB based SCell Activation with the L3 reporting during activation in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- Intra-frequency measurement(deactivated SCell) in Case 2

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

Some reasons are:

- We have the following two agreements: 1) RAN4 to update the agreement of EMP for deactivated SCell measurement and SCell activation to apply to RAN1 Alt Time-C1 scenario; 2) Update RAN4 agreement to apply EMP also to L1 measurement in the active SCell after SCell activation. From this, the EMP issue need to be verified.

- FMW based the deactivated SCell measurement procedure and new interruption requirements (i.e., new interruption for scenario 2 due to OD-SSB activation/deactivation and new interruption for scenario 2 at the end of the FMW if UE does not receive SCell activation command) are defined, deactivated SCell measurement and interruption issues for case 1 and case 2 need to be verified.

- The core requirement of OD-SSB based NR intra-frequency measurement was agreed to define. Meanwhile, based on the RAN2 OD-SSB specific design, below

- RAN2 preference is to keep SMTC based L3 RRM framework and to introduce additional SMTC configuration according to SSB adaptation for L3 RRM measurement on SCell with SSB adaptation - The UE applies the OD-SSB specific SMTC when the OD-SSB is activated and SCell is activated. This decision does not impact RAN4 discussion whether both OD-SSB and AO-SSB can be measured. - When OD-SSB is activated, UE uses servingCellMO-OD to measure serving cell; when OD-SSB is deactivated, UE uses servingCellMO-AO (i.e., legacy servingCellMO) to measure serving cell.

In this sense, the L3 measurement procedure of OD-SSB based measurement should be tested, UE to perform L3 measurement based on OD-SSB specific SMTC. The intra-frequency L3 measurement contains: case 1 and case 2.

2.3 Test configuration

Issue 3-1-5: General principle RAN4 to discuss whether the following principles to design the OD-SSB test case can be agreed. - Both FR1 and FR2 - Test OD-SSB periodicities with 5ms, 20ms. 10ms - Only test non-DRX mode - The test without SSB time index detection - Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting - SMTC and measCycleSCell as 40ms/160ms - Only NCD-SSB for OD-SSB test case.

To verify new critical core requirements but with simplified the TC setting, we can agree the suggested general principle

Proposal 4: For NES OD-SSB TC, the following test configuration can be considered:

- Both FR1 and FR2

- Test OD-SSB periodicities with 5ms, 20ms. 10ms

- Only test non-DRX mode

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

- SMTC and measCycleSCell as 40ms/160ms

3	Conclusion

In this contribution, we provided our viewpoints on RRM performance requirements for NES OD-SSB. The following proposals are obtained:

Proposal 1: For the test case scope for scenario #2, scenario #2A and scenario #3B, the following aspects can be considered:

- In Case 1 and Case 2, RAN4 should define the test cases for scenario #2, scenario #2A and scenario #3B.

- The test cases should include deactivation SCell measurement, SCell activation procedure and the L1/L3 measurement for activated SCell.

- Not to define the test cases to verify the UE behaviour in scenario 3

Proposal 2: RAN4 to define the OD-SSB based SCell activation test case for both known SCell and unknown SCell.

Proposal 3: For NES OD-SSB, the following OD-SSB test cases can be defined:

- SCell activation in Case 2 Alt Time C-1

- OD-SSB based direct SCell Activation at SCell addition in Case 1

- OD-SSB based direct SCell Activation at SCell addition in Case 2 Alt Time C-1

- OD-SSB based PUCCH SCell Activation in Case 1

- OD-SSB based PUCCH SCell Activation in Case 2

- OD-SSB based SCell Activation with the L3 reporting during activation in Case 1

- OD-SSB based SCell Activation with the L3 reporting during activation in Case 2

- Intra-frequency measurement in Case 1

- Intra-frequency measurement in Case 2

- Intra-frequency measurement(deactivated SCell) in Case 2

- Interruption at OD-SSB activation/deactivation

- Interruptions during measurements on deactivated NR SCC

Proposal 4: For NES OD-SSB TC, the following test configuration can be considered:

- Both FR1 and FR2

- Test OD-SSB periodicities with 5ms, 20ms. 10ms

- Only test non-DRX mode

- The test without SSB time index detection

- Follow the legacy test methodology to use L3 event trigger reporting and L1-RSRP reporting

- SMTC and measCycleSCell as 40ms/160ms

Reference

[1] 	R4-2512327, WF on RRM requirements for Netw_Energy_NR_enh_Part1 v1