3GPP TSG-RAN WG4 Meeting # 116-bis											R4-2514219

Prague, Oct. 13th ‒ Oct. 17th, 2025

Agenda item:			5.2.2

Source:	Nokia, AT&T

Title:	TP to TR 37.719-11-11 addition of DC_30A_n14A

Document for:	Approval

# Introduction

This is a TP to TR 37.719-11-11 to add DC_30A_n14A.

************************************* Start of TP*****************************************

## 6.x.1	DC_ 30_n14

#### 6.x.1.1 	Configurations for DC

Table 6.x.1.1-1:  Inter-band EN-DC configurations of 1 LTE band + 1 NR band

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | Single UL allowed |
| --- | --- | --- |
| DC_30A_n14A | DC_30A_n14A | No |

#### 6.x.2.2	Maximum output power for DC

Table 6.x.2.2-1: Maximum output power for inter-band EN-DC of 1 LTE band + 1 NR band

| DC configuration | Power class 3 (dBm) | Tolerance (dB) |
| --- | --- | --- |
| DC_30A_n14A | 23 | +2/-3 |

#### 6.x.2.3	Spurious emission band UE co-existence for DC

Table 6.x.2.3-1: Spurious emissions for inter-band EN-DC of 1 LTE band + 1 NR band

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_ 30_n14 | Frequency range | 1884.5 | - | 1915.7 | -41 | 0.3 | 3 |
| NOTE 3: Applicable when co-existence with PHS system operating in 1884.5 - 1915.7 MHz. |  |  |  |  |  |  |  |

#### 6.x.2.4	Co-existence analysis for DC

Table 6.x.2.4-1 Band 30 and Band n14 UL/DL harmonics/harmonic mixing analysis

| UL/DL harmonics |  | 30 | UL12 | UL2 | UL33 | UL4 | UL5 | MSD type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | fLow | 2305 | 4610 | 6915 | 9220 | 11525 |  |
| n14 | fLow | fHigh | 2315 | 4630 | 6945 | 9260 | 11575 |  |
| DL1 | 758 | 768 | N/A | - | - | - | - | UL harmonic |
| DL23 | 1516 | 1536 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 2274 | 2304 | N | - | N/A | - | N/A |  |
| DL4 | 3032 | 3072 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 3790 | 3840 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | Near miss of UL1/DL3 into n14. |  |  |  |  |  |
| UL/DL harmonics |  | n14 | UL14 | UL2 | UL33 | UL4 | UL5 | MSD type |
|  |  | fLow | 788 | 1576 | 2364 | 3152 | 3940 |  |
| 30 | fLow | fHigh | 798 | 1596 | 2394 | 3192 | 3990 |  |
| DL1 | 2350 | 2360 | N/A | - | - | - | - | UL harmonic |
| DL23 | 4700 | 4720 | - | N/A | - | N/A | N/A | Harmonic mixing |
| DL34 | 7050 | 7080 | - | - | N/A | - | N/A |  |
| DL4 | 9400 | 9440 | - | N/A | N/A | N/A | N/A |  |
| DL54 | 11750 | 11800 | - | - | N/A | N/A | N/A |  |
| Analysis |  |  | No issues found. |  |  |  |  |  |
| NOTE 1: ULx means UL xth harmonic frequency, and DLy means DL yth harmonic frequency range. NOTE 2: When a collision is detected with an overlap >0Hz between the ULx with DLy frequency ranges, the ULx/DLy cell is marked “D” for direct hit. When the gap between ULx and DLy frequency range is from 0Hz to x*MinULCBW, the ULx/DLy cell is marked “N” for Near miss. NOTE 3: UL3/DL2 harmonic mixing direct hit case for PC3/5 only applies for DL>3GHz. NOTE 4: For harmonic mixing, near-miss cases only apply for UL1 and odd DLy orders. |  |  |  |  |  |  |  |  |

Based on Table 6.x.2.4-1, there is a Near miss harmonic mixing issues of UL1/DL3 into n14.

Table 6.x.2.4-2: Band 30 and Band n14 cross-band isolation analysis

| Bands | 30 |  | n14 |  |
| --- | --- | --- | --- | --- |
| Frequency limit | flow | fhigh | flow | fhigh |
| fUL (MHz) | 2305 | 2315 | 788 | 798 |
| fDL (MHz) | 2350 | 2360 | 758 | 768 |
| UL CBW (MHz)2 | Minimum CBW | Maximum CBW | Minimum CBW | Maximum CBW |
|  | 5 | 10 | 5 | 10 |
| ACLR1 range | fxULlow-maxULCBWx | fxULhigh+maxULCBWx | fyULlow-maxULCBWy | fyULhigh+maxULCBWy |
| ACLR1 (MHz) | 2295 | 2325 | 778 | 808 |
| ACLR2 range | fxULlow-2*maxULCBWx | fxULhigh+2*maxULCBWx | fyULlow-2*maxULCBWy | fyULhigh+2*maxULCBWy |
| ACLR2 (MHz) | 2285 | 2335 | 768 | 818 |
| ACLR3 range | fxULlow-3*maxULCBWx | fxULhigh+3*maxULCBWx | fyULlow-3*maxULCBWy | fyULhigh+3*maxULCBWy |
| ACLR3 (MHz) | 2275 | 2345 | 758 | 828 |
| ACLR4 range | fxULlow-4*maxULCBWx | fxULhigh+4*maxULCBWx | fyULlow-4*maxULCBWy | fyULhigh+4*maxULCBWy |
| ACLR4 (MHz) | 2265 | 2355 | 748 | 838 |
| ACLR5 range1 | fxULlow-5*maxULCBWx | fxULhigh+5*maxULCBWx | fyULlow-5*maxULCBWy | fyULhigh+5*maxULCBWy |
| ACLR5 (MHz) | 2255 | 2365 | 738 | 848 |
| Analysis | No issues found |  | No issues found |  |
| NOTE 1: Even if there is no overlap up to ACLR5, MSD beyond the ACLR5 range should be evaluated further if: - The UL aggressor band and DL aggressor band are part of the same or adjacent band group as described in table A.1-1 - If the DL band is above the UL band, it’s lower frequency edge must be below the UL lowest 2nd harmonic frequency - As an indicative threshold, if >45dB UL rejection at the DL band frequency can be guaranteed, assuming a -130dBm/Hz TX noise floor level, the transmitter noise floor related MSD should be negligible. NOTE 2: The maximum UL channel bandwidth of the band (noted maxULCBW) is used to calculate the ACLR ranges. |  |  |  |  |

Based on Table 6.x.2.4-2, there is no cross-band issue for this combination.

Table 6.x.2.4-3: Band 30 and Band n14 UL IMD products

| UE UL carriers | fx_low | fx_high | fy_low | fy_high |
| --- | --- | --- | --- | --- |
| 2nd order IMD products | ¦fy_low – fx_high¦ | ¦fy_high – fx_low¦ | ¦fy_low + fx_low¦ | ¦fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 1507 - 1527 |  | 3093 - 3113 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low – fy_high¦ | ¦2*fx_high – fy_low¦ | ¦2*fy_low – fx_high¦ | ¦2*fy_high – fx_low¦ |
| IMD frequency limits (MHz) | 3812 - 3842 |  | 709 - 739 |  |
| Two-tone 3rd order IMD products | ¦2*fx_low + fy_low¦ | ¦2*fx_high + fy_high¦ | ¦2*fy_low + fx_low¦ | ¦2*fy_high + fx_high¦ |
| IMD frequency limits (MHz) | 5398 - 5428 |  | 3881 - 3911 |  |
| Two-tone 4th order IMD products | ¦3*fx_low –1* fy_high¦ | ¦3*fx_high – 1*fy_low¦ | ¦3*fy_low – 1*fx_high¦ | ¦3*fy_high – 1*fx_low¦ |
| IMD frequency limits (MHz) | 6117 - 6157 |  | 49 - 89 |  |
| Two-tone 4th order IMD products | ¦2*fx_low –2* fy_high¦ | ¦2*fx_high –2* fy_low¦ |  |  |
| IMD frequency limits (MHz) | 3014 - 3054 |  |  |  |
| Two-tone 4th order IMD products | ¦3*fx_low +1* fy_low¦ | ¦3*fx_high + 1*fy_high¦ | ¦3*fy_low + 1*fx_low¦ | ¦3*fy_high + 1*fx_high¦ |
| IMD frequency limits (MHz) | 7703 - 7743 |  | 4669 - 4709 |  |
| Two-tone 4th order IMD products | ¦2*fx_low +2* fy_low¦ | ¦2*fx_high +2* fy_high¦ |  |  |
| IMD frequency limits (MHz) | 6186 - 6226 |  |  |  |
| Two-tone 5th order IMD products | ¦fx_low – 4*fy_high¦ | ¦fx_high – 4*fy_low¦ | ¦fy_low – 4*fx_high¦ | ¦fy_high – 4*fx_low¦ |
| IMD frequency limits (MHz) | 837 - 887 |  | 8422 - 8472 |  |
| Two-tone 5th order IMD products | ¦2*fx_low - 3*fy_high¦ | ¦2*fx_high - 3*fy_low¦ | ¦2*fy_low - 3*fx_high¦ | ¦2*fy_high -3*fx_low¦ |
| IMD frequency limits (MHz) | 2216 - 2266 |  | 5319 - 5369 |  |
| Two-tone 5th order IMD products | ¦fx_low + 4*fy_low¦ | ¦fx_high + 4*fy_high¦ | ¦fy_low + 4*fx_low¦ | ¦fy_high + 4*fx_high¦ |
| IMD frequency limits (MHz) | 5457 - 5507 |  | 10008 - 10058 |  |
| Two-tone 5th order IMD products | ¦2*fx_low + 3*fy_low¦ | ¦2*fx_high + 3*fy_high¦ | ¦2*fy_low + 3*fx_low¦ | ¦2*fy_high + 3*fx_high¦ |
| IMD frequency limits (MHz) | 6974 - 7024 |  | 8491 - 8541 |  |
| NOTE 1: For each IMD item, when two bound values before taking absolute have different signs, the relevant IMD range shall be set such that (1) the lower bound is 0 and (2) the upper bound is the bigger value of the two after taking absolute. NOTE 2: The lowest even order and lowest odd order IMD MSDs shall be considered. |  |  |  |  |

Based on Table 6.x.2.4-3, there is no IMD issue for this combination.

#### 6.x.2.5	∆TIB and ∆RIB values

The TIB,c and RIB,c values are reused from DC_30_n14 as specified in TS 38.101-3.

Table 6.x.2.5-1: ΔTIB,c

| Inter-band DC configuration | ΔTIB,c for E-UTRA band / NR band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_30_n14 | 0.3 | 0.3 |
| NOTE 1: “-” denotes ΔTIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of E-UTRA band and NR band respectively. |  |  |

Table 6.x.2.5-2: ΔRIB,c

| Inter-band DC configuration | ΔRIB,c for NR band / E-UTRA band (dB)1 |  |
| --- | --- | --- |
|  | Component band in order of bands in configuration2 |  |
| DC_30_n14 | - | 0.2 |
| NOTE 1: “-” denotes ΔRIB,c = 0. NOTE 2: The component band order in the configuration should be listed by the order of NR band and E-UTRA band respectively. |  |  |

#### 6.x.2.6	Analysis of MSD requirements

Based on the co-existence studies for DC_30_n14, there is a Near miss UL1/DL3 issue. There are no requirements for CA_n14-n30 for the Near miss case of UL1/DL3.

Table 6.x.2.6-2: Reference sensitivity exceptions (MSD) due to receiver harmonic mixing for EN-DC in NR FR1

| UL band | DL band | UL BW | SCS of UL band | UL RB Allocation | DL BW | MSD | UL/DL fc condition | UL/DL harmonic order |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (kHz) | LCRB | (MHz) | (dB) |  |  |
| 30 | n14 | 10 | 15 | 25 | 5 | 2 | NOTE 2 | UL1/DL3 Near miss |

************************************* End of TP*****************************************