3GPP TSG-RAN WG4 Meeting #116-bis			R4-2514247

Prague, Czechia, 13th - 17th October 2025

Agenda item:	4.27.3

Title:	On demodulation performance requirements of IoT-NTN TDD mode

Source:	Nordic Semiconductor ASA

Document for:	Discussion

1. Introduction

In RAN4#116 core part of the WI has been finalized, however, the demodulation performance tests for IoT-NTN TDD mode remained still undefined. In this contribution we focus on demodulation performance at the UE side.

2. Discussion

NPDSCH demodulation requirements from R17 NTN in TS 36.102 [1] subclause 8.3 are not directly reusable for TDD. This because target operation point is higher than in FDD.

Observation-1: R17 NTN demodulation requirements for UE are not directly reusable to R19 NTN IoT TDD due to different target operation point.

Below we propose the set of parameters for demodulation requirements of IoT NTN TDD.

Table 1: Common Test Parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes per component carrier | Processes | 1 |
| Maximum number of HARQ transmission |  | 4 |
| Cyclic Prefix |  | Normal |
| eutraControlRegionSize-r13 |  | N/A |
| downlinkBitmap-r13 and dl-Gap-r13 |  | Not configured |
| dl-GapNonAnchor-r13 and downlinkBitmapNonAnchor-r13 |  | Not configured |
| Unused REs or RB |  | OCNG |
| OCNG pattern |  | NB.OP.1 |

Table 2: Test Parameters of related NPDCCH and NPUSCH format 2 configurations

| Parameter | Unit | Value |
| --- | --- | --- |
| DCI format |  | DCI format N1 |
| scheduling delay field () |  | 1 |
| (ack-NACK-NumRepetitions-r13) |  | 1 |
| ACK/NACK resource field |  | 3 |
| Reference channel for NPDCCH |  | R.NB.3 NTN-TDD |
| (npdcch-Offset-USS-r13) |  | 0 |
| K_offset | ms | 8 |

Table 3: Test Parameters for NPDSCH under Standalone

| Parameter | Unit |  | Test 1, 2 |
| --- | --- | --- | --- |
| at antenna port |  | dBm/15kHz | [Y] (Note 1) |
|  |  | dBm/15kHz | [Z] (Note 2) |
| NPDCCH repetition number |  | subframe | 4 for Test 1; 16 for Test 2. |
| (npdcch-NumRepetitions-r13) |  | subframe | 4 for Test 1; 16 for Test 2. |
| (nPDCCH-startSF-USS-r13) |  |  | 48 |
| Note 1: This noise is applied to all subframes from the end of the NPDCCH to the end of the following NPDSCH transmission. Note 2: This noise is applied to all subframes from the end of the NPDSCH to the end of the following NPDCCH transmission. |  |  |  |

Table 4: Minimum performance for NPDSCH under Standalone with 1 NRS port

| Test number | Bandwidth | Carrier Type | Reference Channel | Repetition number | Propagation condition | Number of NRS ports | Antenna Configuration | Reference value |  | UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  |  | Fraction of Maximum Throughput (%) | SNR (dB) |  |
| 1 | 200kHz | Anchor | R.NB.1 NTN-TDD | 4 | NTN-TDLC5-200 | 1 | 1x1 | 70% | [-2.0] | NB1, NB2 |
| 2 | 200kHz | Non-anchor | R.NB.2 NTN-TDD | 16 | NTN-TDLA100-10 | 1 | 1x1 | 70% | [2.0] | NB1, NB2 |

Table 5: NPDSCH Reference Channel with 1Tx Antenna for UE Category NB1 and NB2 for NTN-TDD

| Parameter | Unit | Value | Value |
| --- | --- | --- | --- |
| Reference channel |  | R.NB.1 NTN-TDD | R.NB.2 NTN-TDD |
| Carrier Type |  | Anchor | Non-anchor |
| Channel bandwidth | kHz | 200 | 200 |
| Allocated subframes per Radio Frame |  | Note 1 | Note 1 |
| Modulation |  | QPSK | QPSK |
| ITBS/ISF |  | 9/3 | 6/3 |
| Target Coding Rate |  | 1/8 | 1/50 |
| Coding Rate |  | 0.12 | 0.02 |
| Information Bit Payload | Bits | 616 | 392 |
| Binary Channel Bits |  |  |  |
| For Sub-Frames 3,6,7,8 | Bits | 320 | 320 |
| For Sub-Frame 0,5 | Bits | N/A | 320 |
| For Sub-Frame 4,9 | Bits | Note 2 | 320 |
| Max. Average Throughput | Bps | Note 3 | Note 3 |
| UE Category |  | NB1, NB2 | NB1, NB2 |
| Note 1: It shall depend on the specific NPDSCH scheduling. Note 2: N/A when nf mod 2 = 0, otherwise 320. Note 3: Maximum Average Throughput equals to sum of TB(i) divided by sum of T(i), where TB(i) is the TB size of NPDSCH over ith NPDSCH scheduling period, and T(i) is the total time consisting of NPDCCH transmission duration, NPDCCH to NPDSCH scheduling delay, NPDSCH transmission duration, NPDSCH to NPUSCH format 2 scheduling delay, NPUSCH format 2 transmission duration, possible delay between NPUSCH format 2 and NPDCCH for next NPDSCH scheduling and subframes used for NPSS/NSSS/NPBCH/NB-SIB1/NB-SIB2 transmission during the ith NPDSCH scheduling period. |  |  |  |

Table 6: NPDCCH Reference Channel for Category NB1 and NB2 UE

| Parameter | Unit | Value |
| --- | --- | --- |
| Reference channel |  | R.NB.3 NTN-TDD |
| Number of NRS ports |  | 1 |
| Channel bandwidth | MHz | 0.2 |
| Aggregation level | NCCE | 2 |
| DCI Format |  | N1 |
| Payload (without CRC) | Bits | 23 |

Proposal-1: In RAN#116b agree on the set of perfomance test cases defined as depicted in the Tables 1 through 6.  Define reference SNR value by RAN4#117.

3. Conclusion

Observation-1: R17 NTN demodulation requirements for UE are not directly reusable to R19 NTN IoT TDD due to different target operation point.

Proposal-1: In RAN#116b agree on the set of perfomance test cases defined as depicted in the Tables 1 through 6.  Define reference SNR value by RAN4#117.

4. References

[1]	3GPP TS 36.102: "Evolved Universal Terrestrial Radio Access (E-UTRA); User Equipment (UE)

radio transmission and reception for satellite access".