3GPP TSG-RAN WG4 Meeting#116bis                                R4-2514277

Prague, Czech Republic, Oct. 13-17, 2025

Agenda item:	7.11.3

Source: 	ZTE Corporation, Sanechips

Title: 	Discussion on Rel-20 A-IoT coexistence study

Document for:	Approval

Introduction

In the RAN#108 meeting, a New SID on Rel-20 enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices [1] was approved. In this contribution, we would like to share our initial analysis for this SID.

- Assumption that A-IoT BS readers are deployed on the same sites as existing outdoor NR macro BSs. - FR1 licensed spectrum in FDD in-band to NR and in standalone bands, with R2D in DL spectrum and D2R in UL spectrum. This objective includes: RAN1-led - Study necessary and feasible changes to the Rel-19 air interface to support the above scenarios - Air interface for device 2b/C studied for outdoor will be reused for supporting device 2b/C in indoor scenarios - Study applicability and necessity of Device 2b and Device C to the above scenarios, assuming similar device architectures for Device 2b and Device C. - While providing clear differentiation with existing 3GPP LPWA IoT technology and the features of 6G relevant to IoT, study outcomes should include achievable cell edge data rate assuming - Maximum distance between Reader and Device of 50-500 m - Maximum transmit power between -3 dBm and +5 dBm for device C - RAN1 to recommend feasible values within this range - Maximum transmit power of -20 dBm and -10 dBm for device 2b Study outcome should include assumptions on the energy harvesting/storage used to obtain the reported data rates and this includes the possibility of capturing results using energy harvesting with energy provided by the reader or by other sources. - Define necessary further evaluation assumptions of deployment scenarios for coverage and coexistence evaluations. - Define link budget calculation for coverage. RAN4-led - Study coexistence between Device 2b/Device C and NR/LTE in the above outdoor scenarios. - Use band n8 as an example band. NOTE: Strive to minimize evaluation cases in RAN1 and RAN4.

Discussion

- 2.1. Coexistence scenarios for Rel-20 A-IoT study

- As mentioned in the SID, it is clearly stated that in-band operation and standalone operation should be considered which is similar as Rel-13NB-IoT study. As shown in Figure 2.1-1, for DL in-band operation case for A-IoT deployment, since the R2D signal in the baseband is expected to be generated with DFT-s-OFDM method in the downlink carrier which will make the R2D signal to be orthogonal with other NR DL signal. In other words, from our understanding, it is not necessary to conduct the coexistence study for the following case as shown in Figure 2.1-1.

Figure 2.1-1.The illustration of Rel-20 A-IoT in-band operation in DL spectrum

- Proposal 1: for A-IoT DL in-band operation, propose not to conduct the coexistence study due to the orthogonality between R2D signal and NR DL signal;

- As shown in Figure 2.1-2, for UL in-band operation case for A-IoT deployment, if D2R signal is still created similar as Rel-19 OOK signal or BPSK signal, then in the uplink spectrum, we need to conduct appropriate coexistence study to specify the RF requirement for D2R transmission in the uplink direction similar as coexistence study for NB-IoT uplink single tone operation with 3.75kHz SCS in Rel-13.

Figure 2.1-2.The illustration of Rel-20 A-IoT in-band operation in -UL spectrum

- Proposal 2: for A-IoT UL in-band operation, propose to conduct the coexistence study between OOK/BPSK and UL NR signal.

- As standalone A-IoT deployment as shown in Figure 2.1-3 and Figure 2.1-4, similar as other new RAT introduction, we need to conduct the appropriate coexistence study to identify the acceptable ACLR/ACS requirement for it.

Figure 2.1-3.The illustration of Rel-20 A-IoT in-band operation in DL spectrum

Figure 2.1-4.The illustration of Rel-20 A-IoT in-band operation in DL spectrum

- Proposal 3: for A-IoT standalone deployment, propose to conduct the coexistence study in both DL and UL direction.

- In the following Table 2.1-1, we summarized the necessary coexistence scenario for A-IoT outdoor deployment in Rel-20.

- Table 2.1-1. The coexistence scenario for A-IoT outdoor deployment

| Coexistence case | Aggressor system | Victim system |
| --- | --- | --- |
| 1 | A-IoT in-band UL | NR UL |
| 2 | NR UL | A-IoT in-band UL |
| 3 | A-IoT SA DL | NR DL |
| 4 | NR DL | A-IoT SA DL |
| 5 | A-IoT SA UL | NR UL |
| 6 | NR UL | A-IoT SA UL |

- 2.2. Simulation assumption

- 2.2.1.Simulation parameters

- In the following table 2.2.1-1, we provide some initial simulation assumption for coexistence study of Rel-20 A-IoT outdoor deployment.

- Table 2.2.1-1. The coexistence scenario for A-IoT outdoor deployment

|  | - IoT - standalone | A-IoT in-band/ | NR |
| --- | --- | --- | --- |
| Carrier frequency in GHz | 0.9 | 0.9 | 0.9 |
| Size of each nominal channel BW in MHz | 0.2 | 0.18 | 10 |
| Transmission bandwidth in MHz | 0.18 | 0.18 | 9 |
| Environment | Urban macro | Urban macro | Urban macro |
| Network layout | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around | 19-sites [57 sectors] with wrap-around |
| Inter-site distance in meter | 500 | 500 | 500 |
| System loading and activity | Full buffer 100% | Full buffer 100% | Full buffer 100% |
| Network location | Non co-located (at cell edge of legacy) | Co-located with NR | Co-located with A-IoT |
| DL subcarrier spacing | 15kHz | 15kHz | 15kHz |
| UL | Follow the assumption in TS 38.191 | Follow the assumption in TS 38.191 | CP-OFDM |
| DL power control | No | No | No |
| UL power control | TBD Since no DL RSRP measurement is expected for Device 2b and C | TBD Since no DL RSRP measurement is expected for Device 2b and C |  |
| Frequency reuse | 1 | 1 | 1 |
| Number of scheduled UE per cell (DL) | 1 | 1 | 1 |
| Number of scheduled UE per cell (UL) | TBD | TBD | 3 |
| BS antenna height in meter | 30 | 30 | 30 |
| BS max TX power in dBm | 43dBm/200kHz | 46 dbm | 46 |
| BS antenna gain including feeder loss in dBi | 17 | 17 | 17 |
| BS antenna pattern | Horizontal (36.942) | Horizontal (36.942) | Horizontal (36.942) |
| BS antenna front-back ratio in dB | 20 | 20 | 20 |
| UE antenna height in meter | 1.5 | 1.5 | 1.5 |
| UE TX power in dBm | - Device 2b: {-10, -20} - Device C: {-3, 0, 5} | - Device 2b: {-10, -20} Device C: {-3, 0, 5} | -40 to 23 |
| UE antenna gain in dBi | 0 | 0 | 0 |
| Building penetration loss [out door only] | n/a | n/a | n/a |
| Cell selection margin in dB | 3 | 3 | 3 |
| BS-MS min couple loss in dB | 70 | 70 | 70 |
| BS ACLR in dB | 40 to 60 | n/a | 45 |
| BS ACS in dB | 40 to 50 | n/a | 45 |
| UE ACLR in dB | 20 to 50 | n/a | 30 (ACLR1) 43 (ACLR2) |
| UE ACS in dB | 20 to 40 | n/a | 33 |
| BS noise figure in dB | 5 | 5 | 5 |
| UE noise figure in dB | Device 2b: 15 dB Device C: 13 dB | Device 2b: 15 dB Device C: 13 dB | 9 |
| BS-UE path-loss model | TR38.901 macro urban | TR38.901 macro urban | TR38.901 macro urban |
| Standard deviation of BS-UE log-normal shadow fading in dB | TR38.901 macro urban | TR38.901 macro urban | TR38.901 macro urban |
| Shadowing correlation | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 | Inter-cell 0.5 intra-cell 1 |
| Link-level performance model |  |  | As in Annex A.1 in 36.942 |
| Evaluation metrics | SINR | SINR | Throughput loss |

- 2.2.2. Antenna modelling

For non-AAS antenna, the parameters in Table 2.2.2-1 are used for 900MHz TN BS antenna pattern

Table 2.2.2-1: FR1 BS Non-AAS antenna pattern for 900MHz

| Parameter for BS | Values |  |
| --- | --- | --- |
| Antenna vertical radiation pattern (dB) | AE,V(θ″)=−min12θ″−90°θ3dB2,SLAV,θ3dB=10°,SLAV=20dB |  |
| Antenna horizontal radiation pattern (dB) | AE,H(ϕ″)=−min12ϕ″ϕ3dB2,Am,ϕ3dB=65°,Am=20dB |  |
| Combining method for 3D antenna pattern (dB) | A“θ",φ”=−min−AE,Vθ"+AE,Hφ”, Am |  |
| Maximum directional gain of an antenna GE,max | 17 dBi |  |
| Conducted power | 46 dBm |  |
| Mechanical downtilt in degrees | Urban | 10 |

For UE antennas, an omni-directional radiation pattern with antenna gain 0dBi is assumed.

- 2.2.3. ACIR modelling

ACLR modelling between NR and A-IoT system, considering the asymmetric bandwidth between NR and A-IoT, we propose to reuse the ACIR modelling in TR 36.802 for NB-IoT coexistence study in Rel-13.

- 2.2.4. Uplink power control

For uplink scenario, TPC model specified in Section 9.1 TR 36.942[8] is applied for TN with following parameters.

where:

-	Pmax = 23dBm,

-	Rmin = TBD dB,

-	CLx-ile and γ are set as following:

-	CLx-ile = 88 + 10*log10 (200/X) + 11 – Y, where X is UL transmission BW (MHz) and Y is the BS noise figure

-	γ = 1For uplink scenario,

- 2.2.5. Received power model

The received power in downlink and uplink scenarios is defined as below:

RX_PWR = TX_PWR – Path loss + G_TX + G_RX

where:

-	RX_PWR is the received power

-	TX_PWR is the transmitted power

-	G_TX is the transmitter antenna gain (directional array gain)

-	G_RX is the receiver antenna gain (directional array gain).

- 2.2.6. Performance metric

For NR, the average throughput loss and 5%-ile throughput loss should be less than 5%.

For A-IOT, the SNR loss could be considered similar as Rel-19 A-IoT coexistence study.

- 2.2.7. Throughtput~SNR mapping

Adopt Section 5.2.7 of TR 38.803[20] as the SINR-Throughput performance metrics

Conclusions

In this contribution, we shared some initial views on Rel-20 A-IoT coexistence study and proposals are made as following:

- Proposal 1: For A-IoT DL in-band operation, propose not to conduct the coexistence study due to the orthogonality between R2D signal and NR DL signal;

- Proposal 2: For A-IoT UL in-band operation, propose to conduct the coexistence study between OOK/BPSK and UL NR signal.

- Proposal 3: For A-IoT standalone deployment, propose to conduct the coexistence study in both DL and UL direction.

- Proposal 4: Propose to consider the simulation assumption as captured in section 2.2 to conduct the coexistence study in Rel-20.

References

- RP-251884, Study on enhancements for solutions for Ambient IoT (Internet of Things) in NR outdoor for active devices, approved.