# 3GPP TSG-RAN WG4 Meeting #116bis                                                        R4-2514289

# Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Introduction of 6MHz channel bandwidth: System parameters and RF requirements

Source: 	Huawei, HiSilicon

Agenda item:	7.1.5.2

Document for:	Discussion

# Introduction

In RAN 109 a new WID [1]was agreed to introduce 6 MHz channel bandwidth (CBW) on bands n5, n8, n12, n85 and n29. As 6MHz CBW has never been specified, in this tdoc we are going to discuss it is impact on the system parameters and RF requirements

# Discussion

## System parameters

Maximum transmission bandwidth configuration (NRB):

Per WID, only 15KHz SCS needs to be considered for 6MHz CBW. Therefore, multiple values for NRB can be considered at the first sight. The maximum nominal NRB value for could be 33 which means a spectrum utilization of 5.9 MHz (with a guard band of 22.5 KHz). However, such a high spectrum utilization (or lower guard band) is not recommended because of the out of band emission restrictions on the Tx. Table 1 summarizes the different spectrum utilizations and their corresponding GB and GB percentage for up to 40 MHz CBW. Among different NRB candidates (highlighted in blue) 30 is the best candidate because its guard band size remains within the ones of 5 and 7 MHz CBW.

Table 1 Maximum transmission bandwidth and guard band configurations

| CBW (MHz) | 5 | 6 | 6 | 6 | 7 | 10 | 15 | 20 | 25 | 30 | 35 | 40 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NRB | 25 | 29 | 30 | 31 | 35 | 52 | 79 | 106 | 133 | 160 | 188 | 216 |
| Max Transmission BW (MHz) | 4.5 | 5.2 | 5.4 | 5.6 | 6.30 | 9.36 | 14.22 | 19.08 | 23.94 | 28.8 | 33.84 | 38.88 |
| Guard band (KHz) | 242.5 | 382.5 | 292.5 | 202.5 | 342.5 | 312.5 | 382.5 | 452.5 | 522.5 | 592.5 | 572.5 | 552.5 |
| GB % | 4.85 | 6.4 | 4.9 | 3.4 | 4.89 | 3.13 | 2.55 | 2.26 | 2.09 | 1.98 | 1.64 | 1.38 |

Proposal 1: For a 6 MHz CBW, to ensure a reasonable Tx out of band spectrum emission an NRB of 30 and a GB of 292.5 KHz need to be used.

UE channel bandwidth per operating band:

As 6 MHz is to be used for n5, n8, n12, n85 and n29, these four bands need to be updated in TS38.101-1 Table 5.3.5-1, however their implementation needs to be optional, because the legacy UEs are already deployed without such 6MHz CBW. Therefore it is suggested to update it as:

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 6 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n5 | 15 | 34 | 5 | 64 | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n8 | 15 |  | 5 | 64 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | 34 | 5 | 64 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n29 | 15 |  | 5 | 64 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n85 | 15 | 34 | 5 | 64 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Moreover, as it was mentioned in the WID 6 MHz CBW can be early implementable from release 15.

Proposal 2: 6 MHz CBW should be optional and early implementable from release 15. Update TS38.101-1 Table 5.3.5-1 as shown below

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 6 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n5 | 15 | 34 | 5 | 64 | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n8 | 15 |  | 5 | 64 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | 34 | 5 | 64 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n29 | 15 |  | 5 | 64 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n85 | 15 | 34 | 5 | 64 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Channel spacing and channel raster:

As the same channel raster can be used (with 100KHz or 10KHz granularity), the existing channel spacing formula can be reused.

Proposal 3: For a 6 MHz CBW, reuse the existing channel spacing formula and the existing channel raster design

Synchronization raster:

The design of the release 15 sync raster can be re-used as a 6 MHz CBW would allow transmission of a 20 PRB SSB without any puncturing.

Proposal 4: Reuse release 15 sync rasters of 38.101-1 Table 5.4.3.1-1.

## RF Requirements

The summary of the impacted sections can be found in proposal 1:

Proposal 5:

Table 1:  UE RF Requirements for 6 MHz

| NR UE Tx/Rx requirement | Proposal for NR 6 MHz |
| --- | --- |
| 6.2.1 UE maximum output power | No specification impact |
| 6.2.2 MPR | NR MPR is channel bandwidth agnostic --> no specification impact |
| 6.2.3 A-MPR | Has specification impact to band n5. It requires A-MPR study. - NS_01: 6 MHz shall be added to the list of considered channel BW for the NS. - NS_15: A-MPR shall be specified for 6 MHz channel BW - NS_14 is not needed as was the case for 7MHz. - NS_100 is not dependent on the considered channel BW. Has specification impact to band n8. It requires A-MPR study. - NS_01: 6 MHz shall be added to the list of considered channel BW for the NS. - NS_43, NS_43U: A-MPR shall be specified for 6 MHz channel BW - NS_100 is not dependent on the considered channel BW. Has specification impact to band n12 and n85. It requires A-MPR study. - NS_01: 6 MHz shall be added to the list of considered channel BW for the NS. - NS_06: A-MPR shall be specified for 6 MHz channel BW for PC2 and PC1 for n85. - NS_06 is not needed for n12. Has no specification impact to band n29 |
| 6.2.4 Configured transmitted power | No specification impact |
| 6.3.1 Minimum output power | Is channel bandwidth dependent, specification change needed but re-using -40 dBm can be considered. |
| 6.3.2 Transmit OFF power | Value - 50 dBm is channel bandwidth agnostic --> 6 MHz / 15 kHz SCS shall still be added in table 6.3.2-1. |
| 6.3.3 Transmit ON/OFF time mask | No specification impact |
| 6.3.4 Power control | No specification impact |
| 6.4.1 Frequency error | No specification impact |
| 6.4.2.1 EVM | No specification impact |
| 6.4.2.2 Carrier leakage | No specification impact |
| 6.4.2.3 In-band emissions | No specification impact |
| 6.4.2.4 EVM equalizer spectrum flatness | No specification impact |
| 6.5.1 Occupied bandwidth | Specification impact, addition of 6 MHz into Table 6.5.1-1 |
| 6.5.2.2 Spectrum emission mask | Specification impact, dedicated general SEM for 6 MHz needed. |
| 6.5.2.3 Additional Spectrum emission mask | Specification impact, addition of 6MHz to Table 6.5.2.3.4-1 |
| 6.5.2.4.1 NR ACLR | Specification impact, addition of 6 MHz into Table 6.5.2.4.1-1 |
| 6.5.2.4.2 UTRA ACLR | Channel bandwidth agnostic for bands which is applicable --> no specification impact |
| 6.5.3.1 General spurious emissions | Specification impact, addition of 6 MHz into Table 6.5.3.1-1 |
| 6.5.3.2 Spurious UEtoUE co-ex | No specification impact |
| 6.5.3.3 Additional spurious emissions | NS_15: 6 MHz needs to be listed in Table 6.5.3.3.20-1 NS_43 and NS_43U: 6 MHz needs to be listed in Table 6.5.3.3.5-1 |
| 6.5.4 Transmit intermodulation | Channel bandwidth agnostic --> no specification impact |
| 7.3.2 Reference sensitivity | Channel bandwidth dependent --> specification impact -97.2 dBm for 6MHz (30 RB) @ n5 -96.2 dBm for 6MHz (30 RB) @ n8, n12 and n8 6 MHz needs to be added for n29 at Table 7.3.2-1b |
| 7.4 Max input level | Specification impact, addition of 6 MHz into Table 7.4-1: -25/-27dBm |
| 7.5 Adjacent channel selectivity | Specification impact --> 6 MHz is added into Table 7.5-1 33 dBc can be reused. |
| 7.6.2 In-band blocking | Specification impact --> 6 MHz is added into Table 7.6.2-1 |
| 7.6.3 Out of band blocking | Specification impact -->6 MHz is added into Table 7.6.3-1 as REFSENS + 6.0 dB |
| 7.6.4 Narrow band blocking | Specification impact --> 6 MHz is added into Table 7.6.4-1 |
| 7.7 Spurious response | Specification impact --> 6 MHz is added into Table 7.7-1 as REFSENS + 6 dB |
| 7.8 Intermodulation characteristics | Specification impact --> 6 MHz is added into Table 7.8.2-1 Same configuration as 5 MHz case |
| 7.9 Spurious emissions | Channel bandwidth agnostic --> no specification impact |

# Conclusions

Proposal 1: For a 6 MHz CBW, to ensure a reasonable Tx out of band spectrum emission an NRB of 30 and a GB of 292.5 KHz need to be used.

Proposal 2: 6 MHz CBW should be optional and early implementable from release 15. Update TS38.101-1 Table 5.3.5-1 as shown below

| NR Band | SCS (kHz) | UE Channel bandwidth (MHz) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5 | 6 | 10 | 15 | 20 | 25 | 30 | 35 | 40 | 45 | 50 | 60 | 70 | 80 | 90 | 100 |
| n5 | 15 | 34 | 5 | 64 | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 | 20 | 253 |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n8 | 15 |  | 5 | 64 | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 | 20 | 253 | 303 | 353 |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n12 | 15 | 34 | 5 | 64 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n29 | 15 |  | 5 | 64 | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| n85 | 15 | 34 | 5 | 64 | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 30 |  |  |  | 10 | 15 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 60 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| NOTE 4: This UE channel bandwidth is optional in this release of the specification. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Proposal 3: For a 6 MHz CBW, reuse the existing channel spacing formula and the existing channel raster design

Proposal 4: Reuse release 15 sync rasters of 38.101-1 Table 5.4.3.1-1.

Proposal 5: The summary of the impacted sections for RF requirements are given in Table 1.

# Reference

[1]	“RP-252942, New WID: UE RF enhancements for NR FR1, Phase 5, vivo.”