# 3GPP TSG RAN WG4 Meeting #116bis 														R4-2514295

Prague, Czech Republic, Oct. 13-17, 2025

Agenda Item:	4.8.2

Source: 	Ericsson

Title:	RRM requirements for low band carrier aggregation via switching

Document for:	Discussion

# Introduction

There was a good progress in the last meeting, and we were able to agree on most of the essential issues regarding low-band CA via switching and the big CR is agreed. There are few open issues which are left to FFS in the maintenance phase and we discuss them in this paper.

# Discussion

One of the remaining open issues was the activation and deactivation timing of the switching pattern. In the previous meeting, we had agreed that the RRC-configured switching pattern would become applicable after the SCell is activated. However, we later revised that agreement and decided that the pattern should be applicable from the start of the SCell activation command.

To ensure both the network and UE are aligned, we need to discuss and agree on the exact timeline from which the switching pattern becomes applicable. For this clarification, we need to consider both SCell activation methods:

– MAC CE-based SCell activation

– RRC-based direct SCell activation

For both of these activation methods, there are couple of open issues pending after last meeting, the are what is the time UE applies the switching pattern. The same is described in the WF below.

- For MAC CE based SCell activation, UE shall apply the RRC configured switching pattern at slot n+(THARQ+3ms+TLBCA)/slot-length - For RRC based direct SCell activation, UE shall apply the RRC configured switching pattern at slot n+(TRRC_Process + TLBCA)/slot-length - Option 1: TLBCA =0ms - Option 2: TLBCA =1ms - Option 3: TLBCA =4ms - In maintenance stage, RAN4 to FFS how UE performs switching according to the following two options. - For MAC CE based SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(THARQ+3ms+ TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(THARQ+3ms+ TLBCA)/slot-length. - Other options are not precluded. - For RRC based direct SCell activation, according to the switching pattern, - Option 1: UE switching to SCell immediately after n+(TRRC_Process + TLBCA)/slot-length. - Option 2: UE switching to SCell at the next switching period after n+(TRRC_Process + TLBCA)/slot-length. - Other options are not precluded. Note: if the timeline for application and for switching has ambiguity, RAN4 may revise the above application timeline.

First issue: Value of TLBCA

When the need for TLBCA was initially proposed, the main motivation was to provide the UE with additional time to configure its RF chain after the SCell activation end point. In RAN4, the SCell activation end point is defined as the CSI report transmission. The concern from the chipset vendors is that once the UE transmits the CSI report, the SCell is considered active, and the network may schedule the UE in the very next slot. This leaves limited time for the UE to apply the RRC-configured switching pattern at the RF level, as the UE must communicate the pattern from RRC to the RF module.

However, when the switching pattern is considered applicable from the SCell activation command, the UE inherently gains additional time, specifically, 3 ms + THARQ, which allows sufficient time for the RRC to communicate the switching pattern to the RF module. In the case of RRC-based direct SCell activation, we believe the RRC processing delay itself provides adequate time for the necessary internal signaling between the RRC and RF modules. From this perspective, we think that there is no need for an additional time margin (TLBCA), and therefore suggest setting TLBCA = 0 ms.

- For MAC CE based SCell activation and RRC based direct SCell activation, TLBCA = 0 ms

Second issue: When to apply the switching pattern

For simplicity, we denote both expressions as n + X1.

As per RAN1 agreement, the switching pattern has a periodicity of 40 ms, and each slot is designated as either a PCell slot or SCell slot. The pattern starts at SFN0 of the PCell.

As per option 1, UE switches to the SCell immediately after n + X1, regardless of whether the slot is designated as PCell or SCell in the switching pattern. However, if the slot following n + X1 is an SCell slot as per the switching pattern, we believe the UE should switch to the SCell immediately after n + X1. If the slot is a PCell slot, the UE should wait for the next switching period. We do not support Option 1 in its current form, as it is not aligned with the RAN1 agreement on switching pattern design.

As per option 2, UE waits to switch to the SCell until the next switching period, regardless of the slot type immediately after n + X1. We believe this option is also not fully aligned with the intended applicability of the switching pattern. If the slot after n + X1 is an SCell slot, Option 2 would require the UE to wait unnecessarily. If the slot after n + X1 is a PCell slot, the UE should indeed wait until the next SCell slot.

Since both the proposals are not inline with the switch pattern design, we suggest following.

- For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+3ms)/slot-length.

- For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+( TRRC_Process)/slot-length.

Deactivation of Switching Pattern:

Based on what we discussed above for the activation of the carrier switching, for deactivation as well, we do not think there is a need for additional margin. Regarding the switching behaviour, once the SCell is deactivated, there is no transmission from the NW to the SCell. Moreover the switching pattern shall be applicable when the SCell is active. The MAC CE deactivation command shall be the ending point for the switching pattern applicability while MAC CE activation command is the starting point.

- Upon reception of the SCell deactivation command at slot n, UE switches back to PCell at slot n+3+ THARQms.

SCell activation delay requirement:

In the last meeting, it was agreed that RAN4 will specify the SCell activation delay for two scenarios: (i) when none of the SSB/SMTC occasions overlap with the switching pattern, and (ii) when some SSB/SMTC occasions are impacted by the switching pattern. However, the specific requirements were not discussed, as expressing them is complex due to the variety of possible cases, as outlined below.

| Pattern and SSB/SMTC overlapping status | Whether to define the requirement or not |
| --- | --- |
| Some SSB overlapped, SMTC not overlapped | Define requirement |
| Some SSB and SMTC overlapped | Define requirement |
| None overlapped | Define requirement |
| All SSB overlapped, some SMTC not | FFS |
| All SMTC overlapped, some SSB not | FFS |
| All SSB and SMTC overlapped | Do not define requirement |

When only one component—either SSB or SMTC—is present during activation, the delay can be defined relatively easily. However, when both SSB and SMTC are present, the number of possible combinations increases significantly, making the requirement definition more complex. To simplify this, we propose not to define the delay for each component individually. Instead, we suggest specifying the activation timeline based on the combined delay for SSB and SMTC, ensuring the earliest possible time when both are available and not overlapping with the switching pattern.

- RAN4 to agree on the following text in the SCell activation delay requirement for the  cases of none of the SSB overlapped with the pattern or some of the SSb/SMTC occasions are overlapped with the pattern:

For the UE supporting LowBandCA-via-Switching-r19 and configured with LBCA-SwitchingPattern in FR1, the reference time for evaluating Tactivation_time is from the switching pattern activation time. Based on the content of Tactivation_time, the following applies:

-	if it contains only TFirstSSB_MAX or TFirstSSB or TFirstSSB_MAX, enhanced, it corresponds to the time to the end of the first complete SSB burst that does not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

-	if it contains TFirstSSB_MAX + TSMTC_MAX, it represents the shortest combined time at which both a valid SMTC occasion and a complete SSB burst are available, without overlapping the slots indicated by '0' in the LBCA-SwitchingPattern. The order in which the first non-overlapping SMTC occasion and the first non-overlapping complete SSB burst occur is not constrained.

-	if it contains TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced, it refers to the time to the end of two complete SSB bursts, based on the SSB periodicity of the activated SDL SCell, where both bursts do not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

RLM requirement impact on the PCell

We think couple of procedures such as RLM and CBD on the PCell needs special attention.

RLM-RS can be SSB and/or CSI-RS. In legacy, RLM was supported only on SpCell. Since the WI says Strive to minimize the RAN1 impact, we do not foresee RAN1 discussing/introducing the RLM on SCell.

Further UE is expected to perform RLM measurement on the RLM-RS at least once in 2 radio frame and send OOS indication to the higher layer if the OOS is detected.

Based on the switch pattern, the availability of RLM-RS on the PCell may be impacted. If the RLM-RS is impacted due to the switch pattern, RLM evaluation period requirements need further discussion. Since the SDL SCell do not have UL support, HARQ feedback is carried on the PCell. If the radio link of PCell fails, even if the SCell DL is good, UE communication with SCell is as good as lost. Hence, we suggest RAN4 to investigate the RLM and faster link recovery on the PCell. For example, if any of the following condition is met, UE could prioritize performing RLM and link recovery on the slots where the RLM-RS or SSB is transmitted by ignoring the pattern on those slots.

- N1 number of out of sync are triggered on PCell; or

- T310 timer is started.

Similarly, if the BFD is detected on the PCell, UE could prioritize performing CBD on the slots where the CBD-RS is transmitted by ignoring the pattern on those slots (i.e., even if the slot containing CBD-RS belonging to SCell, UE prioritize the CBD-RS reception). In the case of RLM, NW do not know when the RLM occurs at the UE and in this case we could only define the interruption requirements.

- RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritise to receive RS for for faster link recovery.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- For MAC CE based SCell activation and RRC based direct SCell activation, TLBCA = 0 ms

- For MAC CE based SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+(THARQ+3ms)/slot-length.

- For RRC based direct SCell activation procedure, if the SCell activation command is received at slot n, the UE shall switch to the SCell on the first slot that belongs to the SCell as per the switching pattern, occurring after the slot at: n+( TRRC_Process)/slot-length.

- Upon reception of the SCell deactivation command at slot n, UE switches back to PCell at slot n+3+ THARQms.

- RAN4 to agree on the following text in the SCell activation delay requirement for the  cases of none of the SSB overlapped with the pattern or some of the SSb/SMTC occasions are overlapped with the pattern:

For the UE supporting LowBandCA-via-Switching-r19 and configured with LBCA-SwitchingPattern in FR1, the reference time for evaluating Tactivation_time is from the switching pattern activation time. Based on the content of Tactivation_time, the following applies:

-	if it contains only TFirstSSB_MAX or TFirstSSB or TFirstSSB_MAX, enhanced, it corresponds to the time to the end of the first complete SSB burst that does not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

-	if it contains TFirstSSB_MAX + TSMTC_MAX, it represents the shortest combined time at which both a valid SMTC occasion and a complete SSB burst are available, without overlapping the slots indicated by '0' in the LBCA-SwitchingPattern. The order in which the first non-overlapping SMTC occasion and the first non-overlapping complete SSB burst occur is not constrained.

-	if it contains TFirstSSB_MAX, enhanced + TSMTC_MAX, enhanced, it refers to the time to the end of two complete SSB bursts, based on the SSB periodicity of the activated SDL SCell, where both bursts do not overlap with the slots indicated by '0' in the LBCA-SwitchingPattern.

- RAN4 to agree that the switch pattern is not applicable when the RLM or BFD is triggered on the PCell and UE should prioritise to receive RS for for faster link recovery.

# References

[1]		RP-243317, New WID on low band CA via switching

[2]		RP-241537

[3]		R4-2508259, WF on RRM requirements for NR_LBCA_Sw, Moderator (Apple)