3GPP TSG RAN WG4 Meeting #116bis	 R4-2514297

Prague, Czech Republic, 13 Oct. – 17 Oct.  2025

Agenda Item:	4.23.1

Source: 	Ericsson

Title:	Remaining open issues on mobility enhancements Ph4

Document for:	Discussion

# Introduction

In this contribution we discuss remaining open issues on mobility enhancements Ph4.

# Discussion

## Event triggered reporting

In last meeting, there appears to be a differing understanding across companies regarding what exactly needs to be defined as an Ecat capability. Some companies interpret the limit of 48 events as being aligned with the maximum number of reporting configuration IDs supported in LTM CSI configuration. This includes all types of reporting—periodic, aperiodic, semi-persistent, and event-triggered—and is considered sufficient. In this contribution we analyze the need for reporting criterion capability.

Given that a maximum of 8 candidate cells can be configured, and assuming an average of 6 events per candidate cell, the total number of events remains within the 48 report configurations limit. Therefore, from a configuration perspective, the number of events does not appear to be a limiting factor.

Need for a Capability Similar to Legacy Reporting Criterion:
A key question is whether we need a capability analogous to the reporting criterion used in L3 measurements. In legacy, each instance of an event—regardless of whether it shares the same or a different event identity—is treated as a separate reporting criterion. This distinction may also be relevant for LTM. For example, suppose Event LTM3 is configured for a candidate cell with 64 beams. If 10 of these beams become better than the SpCell serving beam, how many of them will the UE actually track? Without a defined capability, it is unclear whether the UE will evaluate all 10 beams or just a subset.

Clarifying Questions to Address:
To determine whether such a capability is needed in LTM, we must first answer a few key questions. In LTM, events are agreed to be evaluated at the beam level. Can the UE run multiple instances of the same event simultaneously across different beams, either within the same candidate cell or across different candidate cells? If no explicit capability is defined, will different UEs implement this differently and decide independently how many event instances to track in parallel? Is there a risk that some UEs may by default track only one or a few event instances per event type, potentially missing other beams that satisfy the event condition?

Potential Limitations on UE Side:

- Measurement Capability: The number of RS (Reference Signals) the UE can measure is already defined by an existing capability, so this is not a limiting factor.

- Number of Comparisons: The UE may not be able to perform comparisons across multiple beams simultaneously for event evaluation. Will this be a bottleneck?

- TTT Timers: If the UE has a limit on the number of TTT timers it can run per event or across all the events at any given time, it may not be able to track multiple beams concurrently. Is this a potential constraint?

If the answers to the above two questions, regarding comparisons and TTT timers, are negative, then there may be no need to define a reporting criterion capability analogous to L3 measurements. In that case, it would be helpful to clarify in the specification that there is no limit on the number of event instances a UE can run in parallel.

- As long as the number of RS configured is within the UE’s measurement capability and the number of reporting configurations remains within the 48, RAN4 to discuss whether there should be any defined maximum or minimum number of parallel L1-triggered event instances that a UE can evaluate and run for each configured event or across all reporting configurations.

# Summary and Conclusion

In this contribution we analyzed potential RAN4 RRM requirements impact and made following observations and proposals.

- As long as the number of RS configured is within the UE’s measurement capability and the number of reporting configurations remains within the 48, RAN4 to discuss whether there should be any defined maximum or minimum number of parallel L1-triggered event instances that a UE can evaluate and run for each configured event or across all reporting configurations.

# References

[1] 	RP-241515, New WID: NR mobility enhancements Phase 4

[2] 	R4-2420101, WF on NR mobility enhancements Phase 4