3GPP TSG RAN WG4 Meeting #116	bis 														R4-2514298

Prague, Czech Republic, Oct. 13-17, 2025

Source:	Ericsson

Title:	Performance requirements for low-band carrier aggregation via switching

Agenda item:	6.3.2

Document for:	Discussion

# 1	Introduction

There was a good progress in the last meeting, and we were able to agree on most of the essential issues regarding low-band CA via switching and the big CR is agreed. In this contribution we discuss and propose performance requirements for low-band carrier aggregation via switching and related test cases.

# 2	Discussion

The core requirements for low-band CA are completed in last meeting and the following requirements are introduced.

- Active TCI state switch delay requirements for Low band CA via switching

- L3 measurement delay requirements for PCell, and activated SCell

- Interruption requirements due to deactivated SCell measurement

- BFD/CBD requirements for SDL SCell and PCell for low band carrier aggregation via switching

- SCell activation for SDL SCell supporting LB CA via switching

- PL-RS switching delay for R19 LB-CA

- RLM requirements for UE supporting low band CA via switching

- L1-RSRP/SINR measurement requirement

We think this feature is identified from the existing issue in the field and suppose to solve the important market demand. We think all of the core requirements shall be properly tested for PCell and SCell. Based on the core requirements defined, we list the respective test cases below and also list if the requirement is to be tested for the PCell or SCell or both.

|  | Applicable to PCell/SCell | In the same test or different test |
| --- | --- | --- |
| Active TCI state switch delay requirements for Low band CA via switching | Both PCell and SCell | In the same test |
| L3 measurement delay requirements for PCell, and activated SCell | L3 measurement delay of PCell L3 measurement delay of activated SCell | In the same test |
| Interruption requirement due to deactivated SCell measurement | Deactivated SCell measurement and interruption on the PCell | Same test |
| BFD/CBD requirements for SDL SCell and PCell for low band carrier aggregation via switching | BFD/CBD on SCell BFD/CBD on PCell | Different test cases for PCell and SCell |
| SCell activation for SDL SCell supporting LB CA via switching | SCell |  |
| PL-RS switching delay for R19 LB-CA | PCell |  |
| RLM requirements for UE supporting low band CA via switching | PCell |  |
| L1-RSRP/SINR measurement requirement | L1-RSRP PCell L1-RSRP SCell L1-SINR PCell L1-SINR SCell | Different tests |

- RAN4 to further discuss the test case list based on the below table.

|  | Applicable to PCell/SCell | In the same test or different test |
| --- | --- | --- |
| Active TCI state switch delay requirements for Low band CA via switching | Both PCell and SCell | In the same test |
| L3 measurement delay requirements for PCell, and activated SCell | L3 measurement delay of PCell L3 measurement delay of activated SCell | In the same test |
| Interruption requirement due to deactivated SCell measurement | Deactivated SCell measurement and interruption on the PCell | Same test |
| BFD/CBD requirements for SDL SCell and PCell for low band carrier aggregation via switching | BFD/CBD on SCell BFD/CBD on PCell | Different test cases for PCell and SCell |
| SCell activation for SDL SCell supporting LB CA via switching | SCell |  |
| PL-RS switching delay for R19 LB-CA | PCell |  |
| RLM requirements for UE supporting low band CA via switching | PCell |  |
| L1-RSRP/SINR measurement requirement | L1-RSRP PCell L1-RSRP SCell L1-SINR PCell L1-SINR SCell | Different tests |

Test configurations:

Based on the WID description, we can derive the following test configuration

-	The cell 1 is on n5 and cell 2 is on n29

-	MRTD = 3µs

-	SCS 15KHz on both carriers

## 2.x	Timing

In existing RRM specification TS 38.133 baseline we have the following tests:

… A.6 NR standalone tests with all NR cells in FR1 … A.6.4 Timing A.6.4.1 UE transmit timing A.6.4.1.1 NR UE Transmit Timing Test for FR1 A.6.4.1.2 NR UE Transmit Timing Test for two TRPs in FR1 … A.6.4.2 UE timer accuracy … A.6.4.3 Timing advance … A.6.4.3.1 SA FR1 timing advance adjustment accuracy …

The WI for low band carrier aggregation via switching has the following deployment considerations [1]:

… - The carrier frequency for all cases is <1 GHz - Co-located and synchronized network deployment for both carriers - Both carriers are in a single TAG - SCS 15KHz on both carriers …

Based on this one can view low band carrier aggregation via switching as an instantiation of NR standalone tests with all NR cells in FR1, when it comes to RRM timing performance requirements.

Observation 1:	 Low band carrier aggregation via switching as an instantiation of NR standalone tests with all NR cells in FR1.

Proposal 2: For timing test cases, existing NR standalone tests with all NR cells in FR1 are sufficient to cover also Low band carrier aggregation via switching.

# 3 	Summary

In this contribution we analyzed the performance requirements of low-band CA via switching and made the following observations and proposals.

- RAN4 to further discuss the test case list based on the below table.

|  | Applicable to PCell/SCell | In the same test or different test |
| --- | --- | --- |
| Active TCI state switch delay requirements for Low band CA via switching | Both PCell and SCell | In the same test |
| L3 measurement delay requirements for PCell, and activated SCell | L3 measurement delay of PCell L3 measurement delay of activated SCell | In the same test |
| Interruption requirement due to deactivated SCell measurement | Deactivated SCell measurement and interruption on the PCell | Same test |
| BFD/CBD requirements for SDL SCell and PCell for low band carrier aggregation via switching | BFD/CBD on SCell BFD/CBD on PCell | Different test cases for PCell and SCell |
| SCell activation for SDL SCell supporting LB CA via switching | SCell |  |
| PL-RS switching delay for R19 LB-CA | PCell |  |
| RLM requirements for UE supporting low band CA via switching | PCell |  |
| L1-RSRP/SINR measurement requirement | L1-RSRP PCell L1-RSRP SCell L1-SINR PCell L1-SINR SCell | Different tests |

Proposal 2: For timing test cases, existing NR standalone tests with all NR cells in FR1 are sufficient to cover also Low band carrier aggregation via switching.

# References

[1] 		RP-251315, Revised WID on Low NR band carrier aggregation via switching, TELUS