3GPP TSG RAN4 Meeting #116bis		R4-2514347
Prague, Czech Republic, 13th – 18th October, 2025

Source:	Nokia

Title:	Requirements for NTN UEs on Satellite switching in Ku band operation

Agenda item:	4.2.7.4

Document for:	Discussion

Introduction

In the last RAN4 meeting, the discussions for the implementation of RRM requirements related to Ku Band have LED to the implementation of this new feature in the standard. One of the main difference of the new feature is that in the Ku Band,  both FR1-NTN and FR2-NTN numerology might be used, at the discretion of the network.

Currently there is just one open issue in the core part of the requirements for the Ku Band operation in NTN [1]. In this document we provide our view related to this issue.

Discussion

Currently there is a single open issue in the implementation of the Ku band operation in NTN according to the WF [1].

Issue 1-2-2: soft satellite switching with re-sync Agreement: - Agree to introduce RRM requirements for soft satellite switching with re-sync for the VSAT UE with electronic beam steering: Tsoft_switch = max(t-service-t-serviceStart, Tsat_beam + Tsearch + T∆ + Tmargin) + Tprocessing - The requirements in this clause do not apply if the time span from the last slot of SI transmission within SI modification period where the broadcasting of the last updated value for t-ServiceStart-r18 [and ssb-TimeOffset-r18] is acquired by the UE for the first time to the first slot corresponding to t-ServiceStart-r18 is is less than [1] s. - Further refinement on the wording is not precluced. - Further discussion on the value of [1] is not precluded. - Extend the period of the scheduling restriction by two sided TRX_switch, TRX_switch = 1 symbol.

There are two aspects to be discussed:

- The refinement of the wording of the “warning period” of the soft satellite switching that enables the UE to implement the feature correctly.

- The confirmation of the value of 1 s.

First of all, we need to discuss the impact of the warning period. It states that if the UE has not received the RRC information of the soft satellite switching ahead none of the requirements are applicable. One aspect is that the definition of the “the broadcasting of the last updated value for t-ServiceStart-r18” might lead to ambiguous interpretation. Besides, the point of the requirement shall not be the acquisition of the information by the UE, but the availability of the information to this UE. Instead of indicating the “last slot of SI transmission within the modification period where the broadcasting of the last updated value of t-serviceStart-r18…” it is more clear to say, “If the network has transmitted the value of t-serviceStart-r18 and not modified since the last SI modification period that is at least [x] before t-serviceStart-r18”

Rephrase the requirements: “The requirements in this clause are not applicable if the network has transmitted the value of t-serviceStart-r18 and not modified since the last SI modification period that is at least [x] before t-serviceStart-r18”.

For the second part we would like to have a value that is more in line with the processing times required in Ku band, in order to have an harmonic specification. For that,  we propose to halve the current value considered:

The warning period shall be 0.5 seconds for the applicability of soft satellite switching requirements.

Conclusion

In this paper we provided our views on the NTN operation in the KU band. We focused our discussion in open issues of the previous WF, and based on our analysis we provided the following list of proposals:

Proposal 1: Rephrase the requirements: “The requirements in this clause are not applicable if the network has transmitted the value of t-serviceStart-r18 and not modified since the last SI modification period that is at least [x] before t-serviceStart-r18”.

Proposal 2: The warning period shall be 0.5 seconds for the applicability of soft satellite switching requirements.

- References

- R4-2512121, WF on RRM requirements for NR_NTN_Ku_bands, Apple, 3GPP TSG-RAN WG4 Meeting #116