# 3GPP TSG-RAN WG4 Meeting # 116-bis					R4-2514370

# Prague, Czech Republic , October 13 - 17, 2025

Source:		Ericsson

Title:			VSAT RF requirement for HD-FDD

Agenda item:		7.6.4.1

Document for:		Approval

# Introduction

In this paper, we present our overview on HD-FDD Vsat terminal RF impact.

# Discussion

In WID[1], the HD-FDD operation mode to be specified:

HD-FDD for NTN Core Part

- Specify RF and RRM requirements to support HD-FDD VSATs in both Ka and Ku bands with FR1 and FR2 numerologies

- Phase 1: Ku band with FR1 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

- Phase 2:  Ku band and Ka band with FR2 numerology

  - Introduce a single set of requirements for HD-FDD for VSAT types 2, 3, 5, 6 with electronically steered antenna

  - Introduce requirements for HD-FDD Mobile VSAT with small antennas (e.g., 20x20cm) with a new VSAT type

- Phase 1 work will commence immediately.  Phase 2 work will commence upon completion of Phase 1 and RAN1/RAN2 availability

- Note :Leverage RedCap NTN HD-FDD solution to NTN HD-FDD VSAT as applicable.

According to WID, RAN4 will focus on phase 1 for Ku band with FR1 numerology, the band corresponding to this is n247, n248.

Similar to the HD-FDD mode for RedCap device, replacing the duplexer with single Tx/Rx filter in VSAT UE will reduce the insertion loss and by this, antenna gain can be reduced and this leads the antenna size reduction. According to the WID justification, the 3 dB post front end improvement can be benefit from the HD-FDD mode and such can be discussed so the impact on the RF requirement can be identified.

As the reduced insertion loss can be compensated by reduced the antenna gain. The Tx min peak EIPR can be kept the same. With the replacing the duplexer, the attenuation on unwanted emission may be compromised but UE need to meet the same unwanted emission.

Tx min peak power could be kept.

As the receiver antenna gain will be changed so the EIS may be impacted. This relates to the G/T delta value compared with the 6 dB in TR 38.863.

EIS=10*log10(k)+30+10*log10(BW)+SNR+IM-G/T

BW is channel bandwidth. (candidate value: 50MHz)

IM is implementation margin. 2dB can be assumed.

SNR is the required SNR for demodulation. -1dB can be assumed.

G/T: 6 dB.

K: is Boltzmann constant, 1.38*10-23 J/K

However, according to 38.863, the DL link budget should be met (-121.78 dBm for GSO and -115.68 dBm for LEO) and therefore, RAN4 should discuss whether G/T could be changed comparing current EIS specification.

EIS may be impacted but DL link budget may limit further relaxation.

# Conclusions

In this contribution, we present our our overview on HD-FDD Vsat terminal RF impact.with below observations:

Observation 1 Tx min peak power could be kept.

Observation 2 EIS may be impacted with DL link budget limit further relaxation.

# References

- RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2