3GPP TSG-RAN WG4 Meeting #116bis	R4-2514391

Prague, Czech Republic, Oct 13th – 17th, 2025

Agenda item:	6.13.3.1

Source: 	Qualcomm Incorporated

Title: 		RRM performance requirements for L1 CLI measurements

Document for:	Discussion

# Introduction

In Release 19, one of the main objectives of the 3GPP work item on the Evolution of NR Duplex Operation: Sub-band Full Duplex (SBFD) [1] is to specify SBFD operation in TDD bands. RAN4 completed the core part during the last meeting. In this paper, we discuss some of the performance aspects of the work-item.

# Measurement accuracy requirements

Side conditions for L1-SRS-RSRP

To ensure consistency and realism in L1-SRS-RSRP test cases, RAN4 needs to define appropriate side conditions, including:

- Time offset conditions

- SRS bandwidth (BW)

- SRS Es/IoT

For the time-offset conditions, a key issue is whether to include the 3 µs cell phase synchronization error. Since RAN4 has already agreed that CLI requirements apply to both intra-cell and inter-cell scenarios, we believe this synchronization error must be included when defining the time offset for SRS arrival.

Proposal 1: The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

- Terror_SRS_RSRP = TC × NTA_offset + 4.67µs for FR1

- Terror_SRS_RSRP = TC × NTA_offset + 3.67µs for FR2

For SRS BW, we propose to reuse the Release 16 CLI assumption for SRS bandwidth, which specifies a bandwidth of 48 PRBs.

Proposal 2: Use an SRS BW of 48 PRB, which is consistent with R16 CLI assumptions.

The side conditions for SRS Es/IoT need to be re-evaluated in the context of SBFD. In Release 16, the assumptions were based on dynamic TDD, where DL reception was not protected from UL interference. However, in SBFD, UL transmissions occur in a dedicated UL subband, and UE-side filtering significantly reduces leakage into DL subbands. As a result, using an SRS SNR of 1 dB (as in R16) may result in negligible DL degradation, making it difficult to assess measurement performance. To ensure meaningful evaluation of SRS RSRP accuracy, we propose using a higher SNR.

Simulation results have already been shared for 1 dB, 2 dB, and 3 dB. Among these, we recommend using 3 dB as the side condition.

Proposal 3: Use SRS Es/Iot side conditions of 3db to determine measurement accuracy requirements, based on simulation results.

L1-CLI-RSSI

There is no fundamental physical layer difference between Release 16 CLI-RSSI measurements and the current L1-CLI-RSSI measurements. Additionally, RAN4 has already agreed that the number of samples used to determine measurement accuracy is one, consistent with the assumptions in Release 16.

Given this alignment, we believe it is appropriate to reuse the Release 16 CLI-RSSI measurement accuracy requirements for L1-CLI-RSSI.

Proposal 4: Re-use the R16 CLI-RSSI measurement accuracy requirements for L1-CLI-RSSI.

# Simulation results

We provided our simulation results for L1-SRS-RSRP measurement accuracy requirements in R4-2511640.

# Conclusion

Proposal 1: The time difference between UE’s DL reference timing in the serving cell and SRS arrival time is no larger than Terror_SRS_RSRP, where

- Terror_SRS_RSRP = TC × NTA_offset + 4.67µs for FR1

- Terror_SRS_RSRP = TC × NTA_offset + 3.67µs for FR2

Proposal 2: Use an SRS BW of 48 PRB, which is consistent with R16 CLI assumptions.

Proposal 3: Use SRS Es/Iot side conditions of 3db to determine measurement accuracy requirements, based on simulation results.

Proposal 4: Re-use the R16 CLI-RSSI measurement accuracy requirements for L1-CLI-RSSI.

# References

[1] 	RP-241614, “Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#104