3GPP TSG RAN WG4 Meeting #116-bis	R4-2514394

Prague, CZ 13th – 17th October 2025

Agenda Item:	6.7.3

Source: 	Ericsson

Title:	Discussions on Remaining Issues of ATG CA RRM Performance

Document for:	Discussion

# Introduction

In our last meeting, we discussed the ATG CA test cases and reached an agreement on most of the TCs. The approved WF for RAN4#116 can be found in [1]. This contribution focuses on addressing the remaining issues related to the performance of the ATG CA enhancement, particularly concerning the test scope.

# Discussion

In this contribution, we discuss our views on the performance requirements based on the agreements regarding core requirements.

## Test Scope

In last meeting, we agreed on the following test cases.

Issue 2-1-3: Test scope Agreement: - Introduced test cases: - Interruptions during measurements on deactivated NR SCC in FR1 - test together with Event triggered reporting on SCC with deactivated SCell test under non-DRX, with measurement cycle of no smaller than 640ms - If it is feasible to test both neighbour cell measurement skipping and not skipping in this test case, - not introduce the test case for “Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle”. - If not feasible, - introduce one more test case for “Event triggered reporting on SCC with deactivated SCell test under non-DRX for 160 ms SCell measurement cycle” - Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot - Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification) - SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification) - SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification) - Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification) - Direct SCell activation at handover with known SCell in FR1 - Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle - Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle - SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle - TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX - Only consider periodic TRS for this test case - This test case is only applicable to case 4 UE - Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG

In addition to the previously agreed test cases, we believe it's essential to include DCI-based and Timer-based Active BWP Switch specifically for ATG case 4, which is not covered in A.19.4.3.1.1. In case 4, PCell and SCell share a common Rx beam that makes BWP switching more timing-critical compared to legacy NR cases. Because both carriers are tied to the same beamforming weight, any SCell BWP switch requires coordinated retuning of the receive path, and this may momentarily impact PCell reception.

Moreover, during a BWP switch, the UE needs to meet the TBWPswitchDelay requirement defined in clause 8.6D.2 (or TdormantBWPswitchDelay = TBWPswitchDelay + X (+Z) for dormant BWPs) while ensuring that any PCell interruption caused by the SCell’s BWP transition remains within the interruption bounds defined in clause 8.2D.1.2.7. When dormant BWPs are involved, activation delay may increase further due to the additional X/Z slot offsets required by 8.6D.2, raising the risk of missed timing requirements. Since Rel-19 ATG introduces case 4, it is necessary to test this scenario to ensure reliable operation.

RAN4 should agree on Test Case ‘DCI-based and Timer-based Active BWP Switch’ for Case 4.

The purpose of this test is to verify the requirements for the DL BWP switch delay and the associated interruption behaviour for a co-located SCell in ATG Case-4, as the PCell and SCell share a common RX beam, making BWP switching more timing-critical, even when the UE supports multiple receive chains. If Proposal 1 is agreed, this same test shall additionally be used to verify that Pre-MG activation/deactivation is completed within 5 ms, as specified in clause 8.19D.4.

‘DCI-based and Timer-based Active BWP Switch’ test should include a test to ensure that UE shall be able to finish pre-configured activation or deactivation within 5 ms (as specified in clause 8.19D.4) after the completion of the active BWP switch.

As the Core part of this WI is finalized and most of the TCs have already been agreed, we should split the test cases and plan for the performance CR from the next meeting.

RAN4 should agree on the work split for the agreed TCs.

# Summary and Conclusion

In this contribution, we have analyzed RAN4 RRM performance aspects for ATG UE DL CA and made the following Proposals.

Proposal 1	RAN4 should discuss Test Case ‘DCI-based and Timer-based Active BWP Switch’ for Case 4.

Proposal 2	‘DCI-based and Timer-based Active BWP Switch’ test should include a test to ensure that UE shall be able to finish pre-configured activation or deactivation within 5 ms (as specified in clause 8.19D.4) after the completion of the active BWP switch.

Proposal 3	RAN4 should agree on the work split for the agreed TCs.

# References

- R4-2512122, WF on RRM requirements for NR_ATG_enh, ZTE, CMCC

- R4-2508263, WF on RRM requirements for NR_ATG_enh, CMCC

- RP-240804, New WID on Enhancements for Air-to-ground network for NR.

- R4-2502591, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420237, WF on RRM requirements for NR_ATG_enh, CMCC

- R4-2420338, WF on NR ATG UE RF, CMCC