3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514398

Prague, CZ, October 13 – 17, 2025

Agenda Item:	6.16.3

Source:	MediaTek Inc.

Title:	Discussion on the RRM performance requirements for LP-WUS

Document for:	Discussion

# Introduction

In this paper, we further discuss the issues related to RRM perf part for R19 LPWUS and provide our view. The achieved agreements in the last meeting are captured in the WF [1].

# Discussion

## Testability

The RRM core requirements for LPWUS are defined in Idle mode and are focused on two scenarios when RRM measurements are fully offloaded to LR (Case#1) and RRM measurements are relaxed (Case#3). These requirements include:

- Serving cell measurements by LR (Case#1)

- Serving and neighbour cell measurement relaxation by MR (Case#3)

- Evaluation requirements for Entry/Exit to different cases

One of the testability challenges in IDLE mode is that UE does not report the measurement results to the NW and therefore, it is not easy to verify whether UE is meeting the requirements. In fact, NW does not know whether UE is operating under Case#1 or Case#3 either. Testing the evaluation requirements for LR/MR switching conditions between the cases (i.e., entry/exit conditions) cannot be verified without a feedback from the UE side. Although the triggering of the switching can be configured based on the power level of the TE, the test cannot verify whether the UE can meet the evaluation requirement for the switching.

Proposal 1: Not to define test cases for the evaluation requirements for Entry/Exit between different cases (Case#1 and Case#3).

Similarly testing serving cell measurements by LR in Idle mode (i.e., Case#1), cannot be verified if UE does not report these measurements back. Therefore, RAN4 not to define test cases for RRM measurements in Case#1.

Proposal 2: RAN4 not to define test cases for RRM measurements requirements in Case#1.

On the other hand, for serving and neighbour cell measurement relaxation by MR in Idle mode (Case#3), it is noticed that in Rel-16 UE power saving WI, a few test cases were introduced to verify the cell reselection performance when UE satisfies a particular relaxed measurement criterion. Therefore, RAN4 to discuss whether the same principle can be applied for Case#3 (RRM relaxation).

Proposal 3: RAN4 to discuss whether Rel-16 test cases for RRM relaxation based on cell reselection can be applied for Case#3.

Even if such test cases are defined for Case#3, it is still not possible to verify whether UE is performing the measurements according to the new relaxation requirements defined in R19 LPWUS or without relaxation requirements. In both cases UE will pass the test (i.e., with or without relaxation) as the reported measurements are considered valid. Therefore, in our view, the test case will be redundant and does not actually prove whether the UE is following the requirements.

Proposal 4: Defining TCs for LPWUS cannot verify whether UE meet the new RRM relaxation requirements in R19 LPWUS or the legacy requirements without RRM relaxation (in both cases UE will pass the test). The TCs are redundant.

# Summary

In this contribution, the following conclusions are provided:

Proposal 1: Not to define test cases for the evaluation requirements for Entry/Exit between different cases (Case#1 and Case#3).

Proposal 2: RAN4 not to define test cases for RRM measurements requirements in Case#1.

Proposal 3: RAN4 to discuss whether Rel-16 test cases for RRM relaxation based on cell reselection can be applied for Case#3.

Proposal 4: Defining TCs for LPWUS cannot verify whether UE meet the new RRM relaxation requirements in R19 LPWUS or the legacy requirements without RRM relaxation (in both cases UE will pass the test). The TCs are redundant.

# References

- R4-2512139, WF on RRM requirements for NR_LPWUS, vivo.