3GPP TSG-RAN WG4 Meeting #116-bis	R4-2514403

Prague, CZ, October 13 – 17, 2025

Agenda Item:	6.10.2.2

Source:	MediaTek Inc.

Title:	Discussion on perf part for FR2-1 SSB based L3 measurement delay reduction by optimizing CSSF

Document for:	Discussion

# Introduction

In this paper we discuss the performance part for R19 RRM phase 5 WI, specifically for CSSF optimization for L3 measurement delay reduction. The progress in the last meeting of RAN4 is captured in the WF [1].

# Discussion

## Solution#1 (1 CC per band)

RAN4 previously agreed on the TC list to be defined for CSSF optimization based on solution 1 as shown below:

RAN4#115 agreement: TC index; Scenario; CA/DC Operation mode; Configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; Nokia. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; Ericsson. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode: FR1+FR2 CA and EN-DC, UE is only required to pass one of them; - Non-DRX, - Without SSB time index detection - CA mode: FR1 PCC+ 2 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and indicates to measure one FR2 SCC - EN-DC: LTE PCC + FR2 PSCC + 1 FR2 intra-band SCCs, network configures MOs on both FR2 SCCs and UE by-default to measure one FR2 PSCC; ZTE. Note: network indication is up to the other RAN2/4 core part discussion. In case network indication for a specific SCC measurement is replaced by other new solution(e.g., SCC selected by UE implementation), use the new solution for CA mode test.

Issue 2-2-2: sub-TC for CSSF optimization solution 1 with network indication - Option 1(Apple): for CSSF enhancement solution 1, if the network indication for a specific SCC measurement is replaced by “SCC selection by UE implementation”, remove the TCs related with network indication in last meeting. - Option 2 (Nokia): To define sub-test to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per FR2 band. - [Way forward] - Continue discussion in RAN4#116bis.

For issue 2-2-2, some companies proposed to introduce a sub-test to verify the measurement delay with enhanced CSSF in case network indicates the carrier to be measured per FR2 band. According to the last meeting agreement in RAN4, UE follow the new RRC network indication (if present) to measure the indicated SCC in an FR2-1 band, if

•	Neither PCC nor PSCC is in the same FR2-1 band, and

•	The indicated SCC to be measured shall be one of the SCCs on which UE is configured to report SSB based measurements in the same FR2-1 band.

From the TCs list agreed previously in RAN4#115, testing UE measurement for the CC indicated by the NW is already covered by the CA operation mode. Therefore, we don’t see the need to introduce additional sub-test to verify this enhancement to other TCs.

Proposal 1: Not need to introduce additional sub-test to verify measurement delay with enhanced CSSF based on network indication.

## Solution#3 (3 searchers)

RAN4 previously agreed on the TC list to be defined for CSSF optimization based on solution 3 as shown below:

RAN4#115 agreement: TC index; Aspects; CA/DC mode; Detailed configurations and scopes; Company. 1; Aspect 1: SSB based Intra-frequency measurement without MG; 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them.; Without DRX and without SSB index reporting; Ericsson. 2; Aspect 2: SSB based Inter-frequency measurement without MG; 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them.; Without DRX and without SSB index reporting; OPPO. 3; Aspect 3: Inter-RAT SSB measurement without MG; 1 TC per operation mode: (1)FR1 only CA: inter-band FR1 CA with 3CCs on 3bands; (2)FR1 only EN-DC: LTE PCC + FR1 PSCC + inter-band FR1 SCC + inter-band FR1 SCC; (3)FR1+FR2 CA: FR1 PCC + FR1 inter-band SCC + FR2 SCC. UE is only required to pass one of them.; Without DRX and without SSB index reporting; Huawei.

The TC#3 in the above list need to be removed because RAN4 agreed in the last meeting (captured below) that the 3 searcher solution for CSSF enhancement does not apply to Inter-RAT measurement without MG. This means, the Inter-RAT measurement requirements without MG are based on 2 searchers assumption same as legacy. Therefore, there is no need to verify this scenario.

RAN4#116 agreement: Issue 1-2-1: applicability limitation for solution 3 Agreement: For UE supporting [new 3-searcher capability] the Enhanced CSSFoutside_gap,i scaling factor does not apply for the E-UTRA inter-RAT MOs without MG (for the scenario of NR SA) that are being measured outside of MG. FFS: For UE supporting [new 3-searcher capability], whether or not the Enhanced CSSFoutside_gap,i scaling factor applies for the inter-frequency MOs without MG that are being measured outside of MG. This part can be discussed in maintenance stage.

Proposal 2: Remove TC#3 for solution 3 since RAN4 agreed that 3 searcher solution for CSSF enhancement does not apply to Inter-RAT measurement without MG.

# Summary

In this contribution, the following observations and proposals are concluded:

Proposal 1: Not need to introduce additional sub-test to verify measurement delay with enhanced CSSF based on network indication.

Proposal 2: Remove TC#3 for solution 3 since RAN4 agreed that 3 searcher solution for CSSF enhancement does not apply to Inter-RAT measurement without MG.

# References

- R4-2512124, WF on RRM requirements for NR_RRM_Ph5_Part1, Apple.