3GPP TSG-RAN4 Meeting #116bis	R4-2514417

Prague, Czech Republic, 13th – 17th , October 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-1 | CR | draftCR | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Corrections for device RF requirements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | Ambient_IoT_Solutions-Core |  |  |  |  |  | Date: |  |  | 2025-10-13 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | The test configuration and FRC for D2R measurement are missing. There’re errors in the R2D FRC as well as the requirements for D2R channel bandwdith. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add test configuration and FRC for D2R. Correct the error for R2D FRC and D2R channel bandwidth requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Errors remain in the spec. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.3.2, Annex A, Annex B |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

### 5.3.2	D2R Channel bandwidth

#### 5.3.2.1	General

The D2R channel bandwidth supports a single AIoT RF carrier in the D2R link at the device. From a BS perspective, different device channel bandwidths may be supported within the same spectrum for backscattering from devices connected to the BS.

From a device perspective, the device is allocated with its own device channel bandwidth. The device does not need to be aware of the BS channel bandwidth.

#### 5.3.2.2	D2R Transmission bandwidth

The transmission bandwidth for each D2R channel bandwidth is specified in table 5.3.2.3-1.

#### 5.3.2.3	D2R channel bandwidth per operating band

The requirements  specified in table 5.3.2.3-1 for device and in table 5.3.2.3-2 for BS apply to all the operating bands in Table 5.2-1.

Table 5.3.2.3-1: Device D2R channel bandwidth

| Device D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal D2R transmission Bandwidth without SFO (kHz) | Nominal Small frequency shift without SFO (kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 17 | 25 | 42 | 75 | 141 | 273 | 534 | 1065 |  |
| 30 |  | 33 | 50 | 83 | 149 | 281 | 545 | 1073 |  |
| 60 |  |  | 66 | 99 | 165 | 297 | 561 | 1089 |  |
| 120 |  |  |  | 132 | 198 | 330 | 594 | 1122 |  |
| 240 |  |  |  |  | 264 | 396 | 660 | 1188 |  |
| 480 |  |  |  |  |  | 528 | 792 | 1320 |  |
| 960 |  |  |  |  |  |  | 1056 | 1584 |  |
| 2880 |  |  |  |  |  |  |  |  | 3168 |

Table 5.3.2.3-2: BS D2R channel bandwidth

| BS D2R channel bandwidth (kHz) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Nominal D2R transmission Bandwidth without SFO (kHz) | Nominal Small frequency shift without SFO (kHz) |  |  |  |  |  |  |  |  |
|  | 3.75 | 7.5 | 15 | 30 | 60 | 120 | 240 | 480 | 720 |
| 15 | 19 | 28 | 46 | 83 | 156 | 303 | 596 | 1183 |  |
| 30 |  | 37 | 55 | 92 | 165 | 312 | 605 | 1192 |  |
| 60 |  |  | 74 | 110 | 184 | 330 | 624 | 1210 |  |
| 120 |  |  |  | 147 | 220 | 367 | 660 | 1247 |  |
| 240 |  |  |  |  | 294 | 440 | 734 | 1320 |  |
| 480 |  |  |  |  |  | 587 | 880 | 1467 |  |
| 960 |  |  |  |  |  |  | 1174 | 1760 |  |
| 2880 |  |  |  |  |  |  |  |  | 3520 |

<<< NEXT CHANGES >>>

Annex A (normative):
The test configuration for D2R measurement

The test configuration in Table A.1-1 is defined to be used for measuring the D2R backscatter power for the requirements defined in section 6.1.1 and 6.2.1.

The CW frequency, D2R small frequency shift (SFS) factor and the D2R nominal bit duration are denoted as FCW, RSFS and Tb, respectively.

Table A.1-1: Test configuration for D2R measurement

| Measurement parameter | Value |
| --- | --- |
| CW frequency (FCW) | According to TS 38.192 |
| Filter passband range for D2R upper sideband | FCW+RSFS-1Tb*0.9 to FCW+RSFS+1Tb*1.1 |
| Filter passband range for D2R lower sideband | FCW-RSFS+1Tb*1.1 to FCW-RSFS-1Tb*0.9 |

<<< NEXT CHANGES >>>

## B.2	R2D reference measurement channels

### B.2.1	Fixed Reference Channels for reference sensitivity level (OOK)

Table B.2.1-1: Fixed Reference Channels for reference sensitivity level (OOK)

| Component | Parameter |  |  | Unit |  | Value |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| General | PRB |  |  | RBs |  | 1 |  | 2 | 3 |  | 4 |
|  | SCS |  |  | kHz |  | 15 |  | 15 | 15 |  | 15 |
| SIP | Bit length |  |  | Bits |  | 8 |  | 8 | 8 |  | 8 |
|  | Mapping to OFDM |  |  | Chips/Symbol |  | 4 |  | 4 | 4 |  | 4 |
| CAP | Bit length |  |  | Bits |  | 4 |  | 4 | 4 |  | 4 |
|  | M |  |  | Chips/Symbol |  | 6 |  | 6 | 6 |  | 6 |
| PRDCH | TBS |  |  | Bits |  | Depending on the size of the MAC PDU of A-IoT CFA paging message |  |  |  |  |  |
|  | CRC |  |  | Bits |  | 16 |  | 16 | 16 |  | 16 |
|  | M |  |  | Chips/Symbol |  | 6 |  | 6 | 6 |  | 6 |
| Postamble | Bit length |  |  | Bits |  | 4 |  | 4 | 4 |  | 4 |
|  | M |  |  | Chips/Symbol |  | 6 |  | 6 | 6 |  | 6 |
| Padding | Padding for last OFDM symbol |  |  | Chips |  | Depending on the TBS |  |  |  |  |  |

## B.3	D2R reference measurement channels

### B.3.1	Fixed Reference Channels for transmitter characteristics

Table B.3.1-1: Fixed Reference Channels for [backscatter power, SEM and] spurious emissions

| Parameter | Unit | Value |
| --- | --- | --- |
| Transmission BW | kHz | 15 |
| TBS | Bits | Depending on the length of AIoT device ID |
| CRC | Bits | 16 |
| FEC code rate |  | 1/3 |
| Block repetition number |  | 1 |
| Preamble length | Bits | 31 |
| Midamble length | Bits | 31 |
| Interval for midamble insersion | Bits | 48 |
| Additional midamble insertion |  | No |
| Small frequency shift | kHz | 480 |
| Modulation |  | BPSK/OOK (NOTE1) |
| NOTE 1: The modulation scheme used is up to device implementation. |  |  |

<<< END OF CHANGES >>>