# 3GPP TSG-RAN WG4 Meeting # 116-Bis 	R4-2514421

Prague, CZ, Oct. 2025

Title:	Reply LS on Beam Correspondence Initial Access

Source:	Qualcomm, Sony, Ericsson, Lenovo

Agenda item:	10.3

Document for:	Approval

# 1.	Introduction

The LS from RAN5 [1] seeks clarifications on the Rel-18 initial access beam correspondence core requirement in the form of three questions. Our view is provided below

# 2.	Discussion

## 2.1	RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

In general, BL (beam lock) and other test-related functions (‘test modes’) exist to facilitate testing in some way (RAN5 domain). UE core requirements (RAN4 domain) on the other hand are meant to establish design goals for field performance for UEs. For relevance to field performance, it is preferred for UEs to not rely on behavior-modifying test modes (like BL) for compliance.

In the context of the Rel-18 IABC (initial access and RRC_inactive beam correspondence), RAN4 have discussed BL extensively as part of testability of the IABC requirement. While there was no technical consensus among participating companies on the need for BL, a compromise was reached where RAN4 agreed to a set of requirements based on existence of a BL function for verification [2]. In our understanding of both intent and wording in the LS or the core requirement, there is no mandate from RAN4 for the UE to implement such a function for compliance.

Observation 1: There is no mandate from RAN4 for the UE to implement a test function like beam lock for compliance.

Unfortunately, the meaning conveyed by the words in the core requirement seems ambiguous to some companies, so it is useful to establish common understanding on the lack of mandate on test function implementation:

Proposal 1: RAN4 to clarify whether the wording in any normative document pertaining to Rel-18 initial access beam correspondence requirement carries a mandate to implement the beam lock test function as precondition for compliance.

Moreover, it is useful to revisit the intent of beam correspondence. As defined in RAN4, the connected-mode beam correspondence is a requirement on the statistics of the UE’s UL power across all tested directions (typically the whole sphere for PC3 and others). The UE’s UL power is the maximum UL EIRP in the direction of the DL during the time interval when that UL is expected.

Observation 2: In RAN4, the beam correspondence requirement is based on maximum UL EIRP in the direction of the DL during the time interval when that UL is expected.

Translated to the Rel-18 initial access beam correspondence, the requirement applies to the highest MSG1 EIRP measured over as many RACH occasions as required to satisfy the requirement. In short, the UE simply must demonstrate ‘enough MSG1 EIRP’ in all tested directions. There is no requirement on what the EIRP may be in untested directions.

Observation 3: The Rel-18 initial access beam correspondence requirements apply to a UE’s MSG1 EIRP during RACH occasions in each direction of test, and for each tested direction there is no implied requirement on the EIRP in other directions.

This requirement is consistent with the needs of field operation, where a UE that manages to register its MSG1 at the gNB with sufficient SINR during a RACH occasion can expect to receive a MSG2 response back. Note that if a UE meets the wording of the RAN4 requirement (‘enough power in the direction of the DL’), it is not important how the UE achieves this (for example, a UE that uses multiple beam candidates to send out its MSG1 or ‘spraying UE’).

Observation 4: The wording in the RAN4 core requirement is consistent with ensuring a certain expectation in field operation, and no new behaviour like beam lock is needed to guarantee good field performance.

Beyond the technical arguments above, one must also consider where RAN4 is headed, as we start discussing 6G. If RAN4 ties compliance with core requirements to implementation of a test function, it implies that a UE that can meet the core requirements without the test function is non-compliant. This is not RAN4 intent (even if some companies may want this) and is a dangerous precedent.

Observation 5: If RAN4 ties compliance with core requirements to implementation of a test function, a worrisome implication for the future of RAN4 is that ‘Even if a UE that can meet core requirements without an available test function, it is non-compliant.’

Proposal 2: Discuss if OK for RAN4 to enforce ‘Even if a UE that can meet core requirements without an available test function, it is non-compliant.’

Proposal 3: RAN4 confirm that the Rel-18 initial access beam correspondence core requirements apply to the max msg1 EIRP in the direction of test over all available RACH occasions and does not include implementation of a test function.

Consequently:

Answer 1: Yes, either procedure can be used, since the Rel-18 initial access beam correspondence core requirements apply to the max msg1 EIRP only in the direction of test over all evaluated RACH occasions. RAN4 confirm that core requirements do not mandate a UE to implement an IABC-specific BL function for compliance with the IABC requirements.

## 2.2	Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams? Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

The first question contains a double negative ‘will (BL) prevent UE to not autonomously choose’ which when straightened out presumably means ‘will (BL) allow UE to autonomously choose …beams’. This seems consistent with the second question, ‘Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?’….

There can be UE implementations whose native behavior an IABC BL may ‘muzzle’. For example, if a UE gets restless about its choice of UL beam due to lack of RAR, it may start to speculatively use beams other than what is chosen as optimal based on DL measurements. For such a UE, an IABC will freeze the UL beam in some configuration determined by arbitrary timing of the BL trigger or arbitrary implementation of the freeze. This example implementation therefore will be prevented from ‘autonomously choosing other beams even if UE’s behaviour is such’.

Answer 2: Yes, BL can prevent a UE from autonomously choosing other beams if UE’s behaviour is such.

## 2.3	Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

As explained in 2.1, There is no RAN4 mandate to use IABC BL for core requirement compliance.

Answer3: Yes. There is no RAN4 mandate for a UE to use IABC BL for IABC core requirement compliance. From a RAN4 perspective, there is no conflict if a UE chooses to meet core requirements without a test mode. It is up to RAN5 to develop the requisite test procedures.

# 3. 	Conclusions

Towards constructing a reply to RAN5 with meaningful content, it is recommended that RAN4 discussed the aspects below:

Observation 1: There is no mandate from RAN4 for the UE to implement a test function like beam lock for compliance.

Proposal 1: RAN4 to clarify whether the wording in any normative document pertaining to Rel-18 initial access beam correspondence requirement carries a mandate to implement the beam lock test function as precondition for compliance.

Observation 2: In RAN4, the beam correspondence requirement is based on maximum UL EIRP in the direction of the DL during the time interval when that UL is expected.

Observation 3: The Rel-18 initial access beam correspondence requirements apply to a UE’s MSG1 EIRP during RACH occasions in each direction of test, and for each tested direction there is no implied requirement on the EIRP in other directions.

Observation 4: The wording in the RAN4 core requirement is consistent with ensuring a certain expectation in field operation, and no new behaviour like beam lock is needed to guarantee good field performance.

Observation 5: If RAN4 ties compliance with core requirements to implementation of a test function, a worrisome implication for the future of RAN4 is that ‘Even if a UE that can meet core requirements without an available test function, it is non-compliant.’

Proposal 2: Discuss if OK for RAN4 to enforce ‘Even if a UE that can meet core requirements without an available test function, it is non-compliant.’

Proposal 3: RAN4 confirm that the Rel-18 initial access beam correspondence core requirements apply to the max msg1 EIRP in the direction of test over all available RACH occasions and does not include implementation of a test function.

# 4. 	References

- R5-253653, ‘LS on Beam Correspondence Initial Access’, RAN5, RAN5#107, May. 2025

- R4-2317764, ‘LS on testability issues of beam correspondence requirement for initial access’, RAN4#108Bis, Oct. ‘23

- R4-2511345, ‘Reply LS on Beam Correspondence Initial Access‘, Qualcomm et al., RAN4#116, Aug. ‘25

# 3GPP TSG-RAN WG4 Meeting # 116-Bis 	R4-2514421

Prague, CZ, Oct. 2025

Title:	[Draft] Reply LS on Beam Correspondence Initial Access

Response to:	(R5-253653) LS on Beam Correspondence Initial Access

Source:	RAN4

To:	RAN5

Contact Person:

Name	: Sumant Iyer

E-mail Address	: sumanti _at_ qti_dot_qualcomm_dot_com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN4 would like to thank RAN5 for their questions, which are clarified thusly:

- RAN5 is looking to develop test procedure with and without UE Beamlock test function, can either procedure be used to verify RAN4 core requirements?

Answer1: Yes, either procedure can be used, since the Rel-18 initial access beam correspondence core requirements apply to the max msg1 EIRP only in the direction of test over all evaluated RACH occasions. RAN4 confirm that core requirements do not mandate a UE to implement an IABC-specific BL function for compliance with the IABC requirements.

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE using UE Beamlock test function in the IDLE mode, will it prevent UE to not autonomously choose Uplink beams? Will using beam lock prevent UE from autonomously choosing other beams even if UE’s behaviour is such?

Answer2: Yes, BL can prevent a UE from autonomously choosing other beams if UE’s behaviour is such.

- Regarding the verification of spherical coverage requirement for initial access and RRC_INACTIVE without using UE Beamlock test function in the IDLE mode, would this be aligned to verifying RAN4 core requirements when the requirements were defined?

Answer3: Yes. There is no RAN4 mandate for a UE to use IABC BL for IABC core requirement compliance. From a RAN4 perspective, there is no conflict if a UE chooses to meet core requirements without a test mode. It is up to RAN5 to develop the requisite test procedures.

2. Date of Next TSG WG RAN4 Meetings:

TSG-RAN WG4 Meeting #116-Bis	Oct. 2025         		Prague, CZ

TSG-RAN WG4 Meeting #117	Nov. 2023         		Dallas, TX, US