3GPP TSG-RAN4 Meeting #116bis	R4-2514460

# Prague, Czech Republic 13th – 17th October, 2025

Source: 	Qualcomm Incorporated

Title: 	AI-ML based CSI Prediction: Simulation Results

Agenda item:	6.12.7

Document for:	Discussion

# Introduction

In this paper, we present simulation results for AI-ML CSI prediction.

# Simulation Framework

During the last meeting, RAN4 agreed to a diverse set of options to simulate the performance of AI-ML based CSI prediction. Many companies did not submit SGCS2 and SGCS3 results during the last meeting. Hence, companies were requested to submit results in terms of SGCS and throughput during RAN4 116 [1]-[3]. This contribution focuses on those options and contains SGCS centric simulation results. The simulation results have been re-submitted from our last meeting’s contribution [5].

Table 1 shows the set of simulation assumptions that were used to generate result for this contribution.

Table 1: Updated simulation assumptions for AIML based CSI prediction [6]

| Parameter | Value |
| --- | --- |
| Duplex, Waveform | FDD, TDD, OFDM |
| Carrier frequency | 2GHz for FDD 4GHz for TDD |
| Subcarrier spacing | 15kHz for FDD, 30kHz for TDD |
| Bandwidth | 10MHz for FDD, 40MHz for TDD |
| Symbol | 1 symbol for step-1 and step-2 simulation |
| Number of subbands | 13 subbands, 4RB as a bundle for FDD 14 subbands, 8RB as a bundle for TDD |
| gNB TX antennas | 16(N1,N2)=(4,2) as defined in Rel-18 CSI prediction requirements |
| UE RX antennas | 4 |
| Channel model | TDLA30 with XP medium correlation as baseline |
| Doppler spread | 20Hz, 50Hz |
| Channel estimation for CSI prediction | Step-1: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., accuracy of AI/ML output CSI, etc.). Step-2: - Ideal DL channel estimation, for the purpose of calibration and/or comparing intermediate results (e.g., to check SGCS after CSI feedback). Step-3: - Practical DL channel estimation, use a common CSI-RS configuration as in demodulation test (may need to define/choose a CSI-RS configuration) |
| CSI-RS configuration | For 16Tx Number of CSI-RS ports (X); 16. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0); (5). - For 32Tx Number of CSI-RS ports (X); 32. CDM Type; CDM4 (FD2, TD2). Density (ρ); 1. First subcarrier index in the PRB used for CSI-RS (k0, k1, k2, k3); (2, 4, 6, 8). First OFDM symbol in the PRB used for CSI-RS (l0, l1); (5, 12). |
| Rank per UE | Step-1: Rank 1 and 2 to do quick check Step-2 and Step-3: Rank 2 as baseline |
| CSI feedback assumption | Step-1: check the CSI prediction (outputs Raw channel) performance, do not need to take CSI feedback into account |
|  | Step-2 and Step-3: take CSI feedback into account - Use Rel-18 eTypeII-Doppler, N4 = 1， N4>1 can be further discussed in the future if needed |
| Test setup(timeline) | Only for step-3 For FDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 4 slots Note: If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-4), this reported PMI cannot be applied at the gNB downlink before slot#(n+4) - Test setup for FDD test setup with Periodic CSI-RS configuration is captured in Table2 For TDD test setup with Periodic CSI-RS configuration - CSI-RS periodicity: 5 ms - CSI feedback periodicity: 5 ms - Scheduling delay (from CSI feedback to apply in scheduling): 6 slots Note : If the UE reports in an available uplink reporting instance at slot#n based on PMI estimation using a CSI-RS resource set in which the last CSI-RS resource is transmitted at a downlink slot not later than slot#(n-6), this reported PMI cannot be applied at the gNB downlink before slot#(n+6). - Test setup for TDD test setup with Periodic CSI-RS configuration is captured in Table3 FFS test setup for aperiodic CSI-RS configuration |
| Beam steer approach | For step-3, randomization of the dual-cluster beam directions should be used as specified in AnnexB.2.3.2.3A in 38.101-4. The value of relative power ratio (p) shall be fixed as 1 during the test |
| SNR assumption | Only for step-3 Claimed by companies for step-3 first round simulation (TP as KPI) FFS: other suggestions |
| CSI feedback overhead | PC7 as baseline |
| Baseline | FFS - Random PMI with Rel-15 Type I single panel codebook if needed (e.g., for Step-3 or performance requirements) |
| KPI | Step-1: - check SGCS1 before CSI feedback - SGCS1 is calculated by comparing the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2), Step-2: - check SGCS2 and SGCS3 - SGCS2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2) Step-3: - check SGCS1 and SGCS3, under practical channel estimation, corresponding to 90% of maximum TP with follow PMI using AI/ML-based CSI prediction - SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel - SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth - Average of SGCS for intermediate results over all subbands per layer (e.g. for rank 2) - check TPs after CSI feedback for both AI based CSI prediction and Random PMI with Rel-15 Type I single panel codebook, as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction γ1_AI/ML: AI/ML based CSI prediction is compared with Rel-15 Type I random PMI - check TPs after CSI feedback for both AI based CSI prediction and Rel-16 eType II codebook(sample and hold), as well as the SNR value corresponding to 90% of maximum TP of the AI based CSI prediction γ2_AI/ML: AI/ML based CSI prediction is compared with Rel-16 eType II CB(sample and hold) - Companies are also encouraged to provide the TP curve with corresponding SNRs. - The test parameters for TPs simulation can be referred as 38.101-4 section of 6.3 - FDD with 16Tx4Rx: Table 6.3.3.1.8-1 - FDD with 32Tx4Rx: Table 6.3.3.1.4-1: - TDD with 16Tx4Rx: Table 6.3.3.2.8-1 - TDD with 32Tx4Rx: Table 6.3.3.2.4-1: |
| Model input type | Raw channel matrix |
| Model out type | Raw channel matrix |
| Observation window | Observation window (number/distance): 5/5ms as baseline Optional for 10/5ms |
| Prediction window | Prediction window (number/distance between prediction instances/distance from the last observation instance to the 1st prediction instance): 1/5ms/5ms |
| Training data and test data | Use following combinations: Training dataset (TDLA30); Test on (TDLA30). 20Hz Doppler spreads; 20Hz Doppler spreads. 50Hz Doppler spreads; 50Hz Doppler spreads. Mixed Doppler spreads (20Hz, 50Hz, 100Hz); 20Hz Doppler spreads. 50Hz Doppler spreads. |
| Companies can provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models Companies are encouraged to clarify whether the SNR dataset for model training is per SNR or with an SNR range when providing the results of step-3 simulation |  |
| Clarifications on the SGCS calculation: SGCS1: Compares the SVD of model output (before feedback) with the SVD of the ground truth Raw channel SGCS2: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the ground-truth raw channel’s SVD. SGCS3: Compares the PMI feedback-derived based on Rel-18 eType II-Doppler CB with the model output against the PMI -derived based on Rel-18 eType II-Doppler CB with the ground truth. |  |

Table2: Test setup for FDD test setup with Periodic  CSI-RS configuration (based on R.PDSCH.1-24.1 FDD)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | MCS13 | MCS17 | MCS19 |  |  |
| Channel bandwidth | MHz | 10 | 10 | 10 |  |  |
| Subcarrier spacing | kHz | 15 | 15 | 15 |  |  |
| Number of allocated resource blocks | PRBs | 52 | 52 | 52 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 16 | 16 | 16 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 17 | 19 |  |  |
| Modulation |  | 16QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.43 | 0.50 |  |  |
| Number of MIMO layer |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot | Bits | 24072 | 32264 | 37896 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For CSI Slots i, if mod (i, 5) = {0}, i={0,…,19} |  | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot | CBs | 3 | 4 | 5 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data An example for FDD test setup with Periodic CSI-RS configuration |  |  |  |  |  |  |

Table3: Test setup for TDD test setup with Periodic  CSI-RS configuration (based on R.PDSCH.2-38.1 TDD)

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | MCS13 | MCS17 | MCS19 |  |  |
| Channel bandwidth | MHz | 40 | 40 | 40 |  |  |
| Subcarrier spacing | kHz | 30 | 30 | 30 |  |  |
| Number of allocated resource blocks | PRBs | 106 | 106 | 106 |  |  |
| Number of consecutive PDSCH symbols |  | 12 | 12 | 12 |  |  |
| Allocated slots per 2 frames | Slots | 24 | 24 | 24 |  |  |
| MCS table |  | 64QAM | 64QAM | 64QAM |  |  |
| MCS index |  | 13 | 17 | 19 |  |  |
| Modulation |  | 16QAM | 64QAM | 64QAM |  |  |
| Target Coding Rate |  | 0.48 | 0.43 | 0.50 |  |  |
| Number of MIMO layer |  | 2 | 2 | 2 |  |  |
| Number of DMRS REs (Note 3) |  | 24 | 24 | 24 |  |  |
| Overhead for TBS determination |  | 0 | 0 | 0 |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {1,2,3,4,5,6} for i from {1,…,39} |  | 49176 | 65576 | 77896 |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0} for i from {0,…,39} | Bits | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot i | Bits | 24 | 24 | 24 |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot i, if mod(i, 10) = {7,8,9} for i from {0,…,39} | CBs | N/A | N/A | N/A |  |  |
| For CSI-RS Slot i, if mod(i, 10) = {0} for i from {0,…,39} | CBs | N/A | N/A | N/A |  |  |
| For Non CSI-RS Slot | CBs | 6 | 8 | 10 |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: Number of DMRS REs includes the overhead of the DM-RS CDM groups without data An example for TDD test setup with Periodic CSI-RS configuration |  |  |  |  |  |  |

# Simulation Results

## Results

Table 2: SGCS results at step 1, 2 and 3 with rank 2

| Prediction algorithm | Doppler during training (Hz) | Doppler during test (Hz) | Layer | Rank | Step 1 SGCS1 | Step 2 SGCS2 | Step 2 SGCS3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample and hold | -- | 20 | 0 | 2 | 0.7805 | 0.7102 | 0.7157 |
| Sample and hold | -- | 20 | 1 | 2 | 0.6245 | 0.5363 | 0.5427 |
| AI-ML20 | 20 | 20 | 0 | 2 | 0.9938 | 0.8957 | 0.9435 |
| AI-ML20 | 20 | 20 | 1 | 2 | 0.9892 | 0.8351 | 0.9071 |
| AI-ML Mix | 20+50+100 | 20 | 0 | 2 | 0.9926 | 0.8947 | 0.9410 |
| AI-ML Mix | 20+50+100 | 20 | 1 | 2 | 0.9866 | 0.8332 | 0.9029 |
| Sample and hold | -- | 50 | 0 | 2 | 0.3127 | 0.2970 | 0.2959 |
| Sample and hold | -- | 50 | 1 | 2 | 0.1734 | 0.1662 | 0.1694 |
| AI-ML50 | 50 | 50 | 0 | 2 | 0.9572 | 0.8653 | 0.8931 |
| AI-ML50 | 50 | 50 | 1 | 2 | 0.9173 | 0.7788 | 0.8167 |
| AI-ML Mix | 20+50+100 | 50 | 0 | 2 | 0.9590 | 0.8668 | 0.8945 |
| AI-ML Mix | 20+50+100 | 50 | 1 | 2 | 0.9197 | 0.7806 | 0.8187 |

Table 3: SGCS results at step 1, 2 and 3 with rank 1

| Prediction Algorithm | Doppler during training (Hz) | Doppler during test (Hz) | Layer | Rank | Step 1 SGCS1 | Step 2 SGCS2 | Step 2 SGCS3 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sample and hold | -- | 20 | 0 | 1 | 0.7805 | 0.7102 | 0.7157 |
| AI-ML20 | 20 | 20 | 0 | 1 | 0.9938 | 0.8957 | 0.9435 |
| AI-ML Mix | 20+50+100 | 20 | 0 | 1 | 0.9926 | 0.8947 | 0.9410 |
| Sample and hold | -- | 50 | 0 | 1 | 0.3127 | 0.2970 | 0.2959 |
| AI-ML50 | 50 | 50 | 0 | 1 | 0.9572 | 0.8653 | 0.8931 |
| AI-ML Mix | 20+50+100 | 50 | 0 | 1 | 0.9590 | 0.8668 | 0.8945 |

Table 2 and table 3 show the following things:

- The SGCS of AI-ML based CSI prediction in step 1 and step 2 are significantly higher than those of “sample and hold” methods in 20 and 50Hz.

- The SGCS performances of AI-ML model, trained based on the mixed dataset of 20, 50 and 100Hz, are almost identical to those of AI-ML model, trained based on individual datasets.

Observation 1: For 25Hz and 50Hz Doppler frequencies, the SGCS obtained with AI-ML based CSI prediction in both step 1 and step 2 are significantly higher than those obtained with “sample and hold” approach.

Observation 2: The SGCS performances of AI-ML model, trained based on the mixed dataset of 20, 50 and 100Hz, are almost identical to those of AI-ML model, trained based on individual datasets.

## Complexity

RAN4 made the following observation during the last meeting

Way forward for next step simulation for CSI prediction: …………. - Companies can also provide the model complexity information (e.g., flops, backbone) of their AI/ML CSI prediction models.

The number of parameters of our AI-ML CSI prediction model to generate the results of this contribution is in the order of several millions.

Observation 3: The number of parameters of our AI-ML CSI prediction model, that got used to generate the results of this contribution, is in the order of several millions.

# Conclusions

Observation 1: For 25Hz and 50Hz Doppler frequencies, the SGCS obtained with AI-ML based CSI prediction in both step 1 and step 2 are significantly higher than those obtained with “sample and hold” approach.

Observation 2: The SGCS performances of AI-ML model, trained based on the mixed dataset of 20, 50 and 100Hz, are almost identical to those of AI-ML model, trained based on individual datasets.

Observation 3: The number of parameters of our AI-ML CSI prediction model, that got used to generate the results of this contribution, is in the order of several millions.

# References

- R4-2505105, WF on requirements for AI/ML air interface normative work, 3GPP TSG RAN WG4 #114bis..

- R4-2505229, “Updated simulation assumptions for AIML CSI prediction”, RAN4 #114bis.

- R4-2508102, “WF on updated simulation assumptions for AI based CSI prediction”, RAN4 #115

- R4-2505513, “CSI Prediction: Simulation Results”, Qualcomm Incorporated, RAN4 #115.

- R4-2511573, “CSI Prediction: Simulation Results”, Qualcomm Incorporated, RAN4 #116.

- R4-2511796, “WF on updated simulation assumptions for AI based CSI prediction final”, RAN4 #116bis