# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514469

# Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Discussion on Tx spurious emission co-existence simulation results

Agenda Item:	6.8.3.1

Source: 	Huawei, HiSilicon

Document for:	Discussion

# 1	Introduction

In RAN4#116 and the subsequent ad-hoc offline meetings simulation assumptions [1] were agreed for the co-existence simulation calibration.

In this paper we present our results for the BS-BS and BS-UE simulations, discuss potential issues and propose the next steps.

# 2	Discussion

## 2.1	BS – BS

### 2.1.1	Introduction

The BS to BS simulation is based on 2 macro networks with a 500m ISD and 100% grid shift between networks. Key points for the simulation are as follows:

- The victim BS height is outside the range for the UMa pathloss model, as macro BS are intended to be above rooftop height and as such should be in LOS with each other. Therefore, it was agreed to use FSPL model for the pathloss. This implies no fading.

- The BS locations are fixed with respect to each other.

- With fixed BS-BS distances and no fading, the number of different pathloss values is limited making the CCDF curve quite blocky.

- It was agreed during the off-line meeting that for the calibration phase the aggressor antenna pattern used would be that of the fundamental 2 GHz antenna. The 6.425 GHz array performance of the antenna is easily derived and it should be discussed if this is more appropriate when considering the final results.

- It was agreed that 20% of UE’s are indoor, whilst this makes no difference to the pathloss calculations (as that is between BS) indoor UE’s are given a random height (outdoor UEs are assumed fixed at 1.5m above ground level) which will affect the antenna direction, as such indoor UE’s were placed.

### 2.1.2	Simulation set-up

19 sites with 3 sectors per site are set-up as in figure 2.1.2-1. The simulation has no wrap around so data is collected from the middle site only. Each sector has a UE dropped in it and the beam is electrically steered towards it.

Figure 2.1.2-1 BS-BS macro layout

### 2.1.3	Calibration results

The power arriving at each of the victim sectors is the sum of the aggressor networks 19x3 = 57 base stations. As its not possible to add path losses each BS was attributed the existing requirement (i.e., -52 dBm) output power so the result can be compared to the desensitisation power level requirements currently used.

For example, for a desensitisation of 0.8 dB a signal 7dB above noise floor is required

Pco-existence_UL=-174+60+6-7 = -115dBm/MHz

As such result of -115 dBm would be equivalent to the existing requirement.

The interferer level for each path is calculated as the sum of the BS output (-52 dBm), the pathloss and the aggressor and victim antenna gain.

Pco-existence_UL=n=157PBS+Gant_aggn+Gant_victn-PLn

The simulation parameters of path loss and antenna gain can be checked against known performance.

### 2.1.3.1	Pathloss

As discussed, the pathloss is not very statistically complex as the FSPL and the distances are calculable. The CCDF is seen in Figure 2.1.3.1-1.

Figure 2.1.3.1-1. BS-BS pathloss CCDF

The extremes can e checked with a manual calculation:

Dmin = 288m, FSPL = 97.8dB, from Figure 2.2.1.1-1 PLmin = 97.8dB

Dmax = 1258m, FSPL = 110.6dB, from Figure 2.2.1.1-1 PLmax = 110.6dB

This checks out exactly.

### 2.1.3.2	Antenna gain

The antenna gain CCDF and the aggressor and victim antenna gain plots can be seen in figure 2.1.3.2-1

Figure 2.1.3.2-1. BS-BS antenna gain CCDF

Again checking the extremes:

Aggressor antenna (2 GHz) – Directivity max = 26.4dBi, Figure 2.2.1.2-1 Gmax = 24.6dBi

Victim antenna (6.425 GHz) – Directivity max = 35.2dBi, Figure 2.2.1.2-1 Gmax = 31.5dBi

The maximum antenna gain is not seen in the statistical results. However, this is to be expected as with mechanical tilt the BS are not aligned in elevation and they are also due to the grid shift they not aligned in azimuth, some loss of gain therefore would be expected.

The minimum gain in azimuth and elevation is limited to -30 dBi so the lower limit of -60 dBi is as expected.

### 2.1.3.3	Interference level

The total interference CCDF curves are shown in figure 2.1.3.3-1.

Figure 2.1.3.3-1. Total interferer level CCDF’s

The 99% levels are extracted in table 2.1.3.3-1.

Table 2.1.3.3-1. BS-BS 99% CCDF results

|  | Fundamental aggressor response |  | Harmonic aggressor response |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  | rho=1 | rho=0 | rho=1 | rho=0 |  |
| 99% | -117 | -113 | -125 | -123.5 | dBm |

Using out of band harmonic array response there is 8.5 to 10 dB margin over the current requirement depending on the correlation assumption.

### 2.1.3.4	Issues

Whilst carrying out the calibration some issues were identified:

- The element description is not consistent with the array geometry. The array has a separation between elements of 0.5λ horizontally and 0.7λ vertically. The maximum element size is hence 0.5x0.7λ. An element of this size cannot have a beam width of 90°x65° as it is not large enough (65° is generally used for 0.9λ, 90° for 0.7λ and 130° for 0.5λ). The estimation of the element pattern with respect to the separation has been discussed during many different simulation tasks and generally these figures have been settled on. The error does not have too much effect on the correlated results where the array performance dominates but can affect the non-correlated results where the element pattern is important.

## 2.2.	BS-UE

### 2.2.1	Introduction

The BS to UE simulation is based on 2 micro networks placed within a macro 19 site 3 sector layout with 500m ISD. There are 3 micro sites per Macro sector.

Key points for the simulation are as follows:

- Micro BS are randomly located along one of 3 circles placed within the macro cell. Micro BS must be no closer than 10 m to each other.

- It was agreed during the off-line meeting that for the calibration phase the aggressor antenna pattern used would be that of the fundamental 2 GHz antenna. The 6.425 GHz array performance of the antenna is easily derived and it should be discussed if this is more appropriate when considering the final results.

### 2.2.2	Simulation set-up

The micro sites are set up as in figure 2.2.2-1, there are 171 micro BS per network. The macro BS and its associated UE is shown in figure 2.2.1-1 but has no part in the simulation. The Micro BS are dropped randomly on the edge of the shown circle and is nominally pointing at the centre of the circle. The AAS steering is used to steer the beam directly at the assigned UE.

As with the BS-BS simulation no wrap around is implemented in the simulator as data is taken only from the centre site. In this case there are 3 sectors with 3 micro BS per sector so 9 data points are collected per drop.

Figure 2.2.2-1 BS-UE micro layout

### 2.2.3	Calibration results

The power arriving at each of the victim UE’s is the sum of the aggressor networks 19x3x3 = 171 base stations. As it’s not possible to add path losses each BS was attributed the existing requirement (i.e., -52 dBm) output power so the result can be compared to the desensitisation power level requirements currently used. The DL desensitisation requirement is different to the UL, in this case 3 dB is the target.

The emission requirement is hence:

Pco-existence_DL=-174+60+9-0 = -105 dBm/MHz

(note UE NF is 9dB @ 6.4GHz)

As such result of -105 dBm would be equivalent to the existing requirement.

The interferer level for each path is calculated as the sum of the BS output (i.e., -52dBm), the pathloss and the aggressor and victim antenna gain.

Pco-existence_DL=n=1171PBS+Gant_aggn-PLn

The simulation parameters of path loss and antenna gain can be checked against known performance.

### 2.2.3.1	Pathloss

The pathloss is based on the UMi propagation model, the model contains a number of random factors including shadow fading and penetration loss.

Figure 2.2.3.1-1. BS-UE pathloss CCDF

The UMi model is far more complex than the FSPL model so is harder to check than the BS-BS case but will be checked in the calibration procedure.

### 2.2.3.2	Antenna gain

The antenna gain CCDF and the aggressor and victim antenna gain plots can be seen in figure 2.2.3.2-1.

Figure 2.2.3.2-1. Antenna gain CCDF

Checking the extreme gain:

Aggressor antenna (2 GHz) – Directivity max = 24.7 dBi, Figure 2.2.3.2-1 Gmax = 24.5dBi

Unlike the BS-BS case the maximum antenna gain can be seen in the CCDF curve, this is reasonable as both the intended target UE and the victim UE are potentially in the same location.

The minimum gain in azimuth and elevation is limited to -30 dBi so the lower limit of -60 dBi is as expected.

### 2.2.1.3	Interference level

The total interference CCDF curves are shown in figure 2.2.1.3-1.

Figure 2.2.3.3-1. Total interferer level CCDF’s

The 99% levels are extracted in table 2.2.3.3-1.

Table 2.2.3.3-1. BS-UE 99% CCDF results

|  | Fundamental aggressor response |  | Harmonic aggressor response |  | Unit |
| --- | --- | --- | --- | --- | --- |
|  | rho=1 | rho=0 | rho=1 | rho=0 |  |
| 99% | -121.5 | -128.5 | -129 | -128 | dBm |

As with the BS-BS the harmonic response is the correct approach. There is at least 23 dB margin over the requirement in these cases.

### 2.2.3.4	Issues

Whilst carrying out the calibration some potential issues were identified:

- The element description is not consistent with the array geometry. The array has a separation between elements of 0.5λ horizontally and 0.7λ vertically. The maximum element size is hence 0.5x0.7λ. An element of this size cannot have a beam width of 90°x65° as it is not large enough (65° is generally used for 0.9λ, 90° for 0.7λ and 130° for 0.5λ). The estimation of the element pattern with respect to the separation has been discussed during many different simulation tasks and generally these figures have been settled on. The error does not have too much effect on the correlated results where the array performance dominates but can affect the non-correlated results where the element pattern is important.

- The indoor distance d2d_in is described as the minimum of 2 uniformly random variable between 0 and 25m. The max value of d2d_in is hence 25m. However, the minimum distance between the BS and the UE, d2d is 3m. As d2d=d2d_in+d2d_out this could result in a negative value for d2d_out. Clearly that is not possible. As a solution our simulation sets d2d_in as the minimum of 2 uniformly random variable between 0 and min(d2d,25) this prevents d2d_out from becoming negative.

- In TR 36.873 the UMi model applicability range has a minimum limit for d2d of 10 m, the minimum d2d in the simulation assumptions d2d min is 3m. It can be noted that the applicability range limit is not present in TR 38.922 so this may not be an issue.

## 2.4	Simulation summary

The CCDF calibration results have been added to the results collection spreadsheet (albeit in a slightly different format; to be further resolved).

Whilst the results with the aggressor antenna using the fundamental pattern have been presented for calibration we think that the correct approach should be to use the harmonic response, whilst it is difficult to predict what the element pattern will be at the harmonic frequencies it is clear that the antenna elements will be sparse when a 2 GHz antenna is considered at 6.425 GHz and the harmonic response is as valid as the fundamental response in that it is based on the number of elements and their physical separation. It can be noted that in 38.922 the concept of design frequency and excitation frequency being separate has already been introduced.

Proposal 1: The array response should be based on the out of band harmonic array radiation pattern not the fundamental.

Another issue is how we use the CCDF curves to derive an acceptable BS interfere level. This analysis uses a -52 dBm interference level and we can use a percentage output from the CCDF curves to compare to the desensitisation of the victim receiver when it is operating at the noise floor. When we were considering the static MCL scenario in previous calculations this perhaps make sense as there are no statistical values involved. However, in addition to the statistical level of the interferer we also have the probability of the victim operating at or above the noise floor.

Observation 1: We have not yet agreed how to use the CCDF curves to set an interference requirement.

Using a statistical value, such as 99%, for the interferer without considering the victim wanted signal does not really give any indication of the effect of the interference on the victim network. For example, when considering ACIR we look at the effect on the victim throughput so both the interferer level and the wanted signal level are considered. For OTA blocking we also consider the difference between the wanted signal and the blocker.

Observation 2: For ACIR and OTA blocking statistical analysis both the level of the interferer and the wanted signal are considered.

Proposal 2: The metric used to set interferer level should be based on both the interferer power and the victim network wanted power so the effect on the victim network can be seen.

If desensitisation at noise floor only is with a statistical interferer level is considered then the CCDF percentage expected would be lower (95% for example), however if the throughput or SNR is considered as a metric then the effect on the victim network would be more realistic and a higher CCDF percentage would be required (99% for example).

Proposal 3: If interferer only CCDF analysis is used the percentage should be lower (95%) than if throughput or SNR is used, which would require a higher CCDT percentage (99%).

## 2.5	WF follow-up

The last WF in R4-2512578 (RAN4#116) has capture number of open issues. As we aim to proceed with the simulation results alignment towards the Tx spurious co-existence limits derivation, we need to progress on those open issues as well. In the proposal below, we propose updates on top of the WF in R4-2512578 (RAN4#116).

One of the issues discussed offline last meeting was how and where to capture outcomes of this technical discussion. Companies have agreed to capture conclusions in a TR, but it was not clear which TR to use. There are few candidates:

- TR 38.817-02 (General aspects for BS RF for NR): this is Rel-15 TR which was used to capture agreements leading to introduction of BS RF requirements for NR. Technically, this could be a proper placeholder, but there may be formal issues to be verified beforehand with MCC.

- TR 38.922 (Study on IMT parameters for 4400 - 4800 MHz, 7125 - 8400 MHz and 14800 - 15350 MHz): this TR was used in Rel-19 to capture agreements on the IMT parameters and their derivation for 3 frequency range to be studied in ITU-R towards 6G. This TR is not seen as proper for the Tx spur co-ex discussion.

Proposal 4: Decide on the following topics for the Tx spurious co-existence limits derivation:

- CDF %-tile: decide on the threshold value and the metric.

- ISD value for results derivation: 500 m

- Grid shift for results derivation: 100%

- Antenna correlation value for the OoB antenna gain for co-ex limits results derivation: (0, or 1): TBD

Evaluation scenarios for results derivation: Same value as for calibration, i.e., 19sites, 3 sectors, wrap around (if supported by the simulation platform).

- How and where to capture outcomes of this discussion: Do not generate new TR. Consider to reuse TR 38.817-2, subject to feedback from MCC.

# 3	Conclusions

In this paper we present the results of the simulated calibration procedure as well as results using the harmonic out of band antenna pattern, we make the following proposal:

Proposal 1: The array response should be based on the out of band harmonic array radiation pattern not the fundamental.

We have also discussed how the simulation results can be used to identify a requirement with the following observations and proposals:

Observation 1: We have not yet agreed how to use the CCDF curves to set an interference requirement.

Observation 2: For ACIR and OTA blocking statistical analysis both the level of the interferer and the wanted signal are considered.

Proposal 2: The metric used to set interferer level should be based on both the interferer power and the victim network wanted power so the effect on the victim network can be seen.

Proposal 3: If interferer only CCDF analysis is used the percentage should be lower (95%) than if throughput or SNR is used which would require a higher CCDT percentage (99%).

Finally in response to the issues in the previous WF:

Observation 3: Using the current set of results, we can see that using the harmonic aggressor antenna response:

- For BS-BS interference we have 8.5 to 10 dB margin over the existing requirement.

- For BS–UE we have >23dB margin over the existing requirement.

Furthermore, the following is proposed to progress on the topics covered in the previous WF:

Proposal 4: Decide on the following topics for the Tx spurious co-existence limits derivation:

- CDF %-tile: decide on the threshold value and the metric.

- ISD value for results derivation: 500 m

- Grid shift for results derivation: 100%

- Antenna correlation value for the OoB antenna gain for co-ex limits results derivation: (0, or 1): TBD

Evaluation scenarios for results derivation: Same value as for calibration, i.e., 19sites, 3 sectors, wrap around (if supported by the simulation platform).

- How and where to capture outcomes of this discussion: Do not generate new TR. Consider to reuse TR 38.817-2, subject to feedback from MCC.

# 4	References

[1]	Simulation assumptions for Coexistence spurious emission requirements_R4	 ZTE