# 3GPP TSG-RAN WG4 Meeting #116bis	R4-2514470

# Prague, Czech Republic, Oct. 13-17, 2025

Title: 	Introduction of 6MHz channel bandwidth: BS RF aspects

Agenda Item:	7.1.5.1

Source: 	Huawei, HiSilicon

Document for:	Discussion

# 1	Introduction

Rel-20 WI on UE RF enhancements for NR FR1 (Phase 5) in [1] includes objectives on 6 MHz channel bandwidth, as extracted below:

6MHz channel bandwidth - Specify the generic core requirements to support 6 MHz NR FR 1 channel bandwidth [RAN 4] - Only include 15 kHz SCS - Specify the spectrum utilization - Specify UE RF requirements for UL and DL - Specify BS RF requirements (OTA and conducted) for UL and DL - Specify the band specific requirements to introduce 6 MHz for NR band n5, n8, n12, n85, n29 [RAN 4] - BS and UE Channel bandwidth per operating band - UE reference sensitivity - When needed - Additional Maximum Power Reduction (A-MPR) - Once RAN 4 has introduced the new channel bandwidth, RAN 2 will introduce the necessary signalling for this channel bandwidth [RAN 2] - Note This should be similar to the signalling for 7 MHz channel bandwidth - Once RAN 4 has introduced the new channel bandwidth, RAN 3 will introduce the necessary signalling for this channel bandwidth [RAN 3] - Note This should be similar to the signalling for 7 MHz channel bandwidth NOTE: Once this WI is completed, the 6 MHz bandwidth can be introduced also for other bands (not addressed in this WI) via the appropriate basket WI. The support of 6MHz bandwidth is release independent.

In this contribution we provide initial discussion on introduction of 6MHz channel bandwidth, with the focus on BS RF requirements.

# 2	Discussion

# 2.1	Work-split

Introduction of a new channel bandwidth of 6 MHz could be simplified and implemented with reduced workload if we leverage and follow the exercise as already done in Rel-19 work item on 7 MHz channel bandwidth introduction. Therefore, it is suggested to reuse the same work-split as the baseline, subject to modifications, and confirmation by interested companies:

Proposal 1: Proceed with the following work-split, as the starting point.

| Technical specification | Companies |
| --- | --- |
| TS 38.104 | [Ericsson] |
| TS 38.141-1 | [Nokia] |
| TS 38.141-2 | Huawei |
| TS 38.101-1 | [CR responsible: Qualcomm Sub-parts split (draft CRs only): System parameters: Nokia Tx: Huawei Rx and Annexes (if needed): Qualcomm] |
| TS 38.101-4 | [Qualcomm] |
| TS 38.133 | [ZTE] |
| TS 38.307 | [China Telecom] |
| TS 37.104 | [ZTE] |
| TS 37.141 | [ZTE] |
| TS 38.106 (*) | [Ericsson] |
| TS 38.115-1 (*) | [CATT] |
| TS 38.115-2 (*) | [CATT] |
| TS 38.174 (*) | [Nokia] |
| TS 38.176-1 (*) | [Nokia] |
| TS 38.176-2 (*) | Huawei |
| TS 38.113 | [ZTE] |
| TS 37.113 | [Ericsson] |
| TS 37.105 | Huawei |
| TS 37.145-1 | Huawei |
| TS 37.145-2 | Huawei |
| (*) The NCR and IAB specs would need to be updated to clarify 6 MHz channel BW is not supported. |  |

# 2.2	Restrictions on the 6 MHz channel applicability: IAB, NR repeater

Referring to the work done in Rel-19 for 7 MHz and the WF in [2], it was agreed that 7 MHz will not be introduced for NR IAB specification, nor for the NCR:

The above agreement was implemented as a clarification note in the Operating bands section – see related implementation below for reference:

It is proposed to follow the same approach for the 6 MHz channel introduction, unless there is explicit interest:

Proposal 2: 6 MHz channel bandwidth should not be supported in the following IAB and NCR specifications: TS 38.106, TS 38.115-1, TS 38.176-2.

# 2.3	EMC specifications

Similar to what was done in Rel-19 for 7 MHz channel bandwidth, for the introduction of 6 MHz channel, there is a need to update two EMC specification, as listed below:

| Impacted existing TS/TR |  |  |  |
| --- | --- | --- | --- |
| TS/TR No. | Description of change | Target completion plenary# | Remarks |
| 38.113 | NR; Base Station (BS) ElectroMagnetic Compatibility (EMC) | #117 | Perf part |
| 37.113 | NR, E-UTRA, UTRA and GSM/EDGE; Multi-Standard Radio (MSR) Base Station (BS) Electromagnetic Compatibility (EMC) | #117 | Perf part |

There is only single modification required, which is to add 6 MHz to the list of NR channel bandwidths in the performance criteria section, see example below:

This is seen as unnecessary process complication and additional unnecessary workload, with no technical discussion required. Therefore, instead of explicitly listing specific NR channel bandwidths, it is proposed to make the EMC specification agnostic to the channel bandwidth, to avoid the need for such discussions in future.

Please note, that the NR UE EMC specification TS 38.124 covers performance criteria in very generic way, with no need to explicitly list any details such as channel bandwidth, FRC, SCS, etc. Related sentence from TS 38.124 is provided below for reference:

In the data transfer mode, the performance criteria shall be that the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channel as specified in annex A in TS 38.101-1 [3] or TS 38.101-2 [4], with parameters specified in clause 7.3.2 in TS 38.101-1 [3] or TS 38.101-2 [4] during the test sequence.

With the above, the following is proposed:

Proposal 3: Apply the following procedure for EMC specifications TS 38.113 and TS 37.113 to avoid the need for explicitly introduce 6 MHz channel bandwidth:

- During RAN4#117, handle TS 38.113 and TS 37.113 under the Maintenance agenda to implement channel bandwidth agnostic approach, e.g., align with the NR UE approach in TS 38.124.

- During RAN#110, remove EMC specifications TS 38.113 and TS 37.113 from the list of impacted specifications in the WID.

# 2.4	BS RF requirements: Total power dynamic range

Total power dynamic range requirement is derived with the assumption of 30 RB allocation; subject to decision on system parameters):

Table 1: Core requirement (conducted and OTA) for the Total power dynamic range

| BS channel bandwidth (MHz) | Total power dynamic range (dB) |  |  |
| --- | --- | --- | --- |
|  | 15 kHz SCS | 30 kHz SCS | 60 kHz SCS |
| 5 | 13.5 | 10 | N/A |
| 6 | 14.7 | N/A | N/A |
| 7 | 15.0 | N/A | N/A |
| 10 | 16.7 | 13.4 | 10 |
| 15 | 18.5 | 15.3 | 12.1 |
| 20 | 19.8 | 16.6 | 13.4 |
| 25 | 20.8 | 17.7 | 14.5 |
| 30 | 21.6 | 18.5 | 15.3 |
| 35 | 22.7 | 19.6 | 16.4 |
| 40 | 22.9 | 19.8 | 16.6 |
| 45 | 23.8 | 20.7 | 17.6 |
| 50 | 23.9 | 20.8 | 17.7 |
| 60 | N/A | 21.6 | 18.5 |
| 70 | N/A | 22.3 | 19.2 |
| 80 | N/A | 22.9 | 19.8 |
| 90 | N/A | 23.4 | 20.4 |
| 100 | N/A | 23.9 | 20.9 |

Related proposals:

Proposal 4: Define the Core requirement (both conducted and OTA) for the Total power dynamic range (15 kHz SCS, 30 RB allocation) as 14.7 dB.

Proposal 5: Define the Test requirement (both conducted and OTA) for the Total power dynamic range (15 kHz SCS, 30 RB allocation, 0.4 dB TT) as 14.3 dB.

# BS RF requirements: EVM window length

During discussion on 7 MHz in Rel-19, while the EVM requirements was reused as being agnostic to the channel bandwidth, the EVM window length was one of the topics discussed in relation to the UE RF and related FFT size required.

In case of 6 MHz channel, this topic seems to be straight-forward, as the values of the EVM window length and related parameters are the same for 5 MHz channel, as well as for 7 MHz channel. Therefore, 6 MHz channel will reuse same values.

Table 2: EVM window length for normal CP, FR1, 15 kHz SCS

| Channel bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (Note) (%) |
| --- | --- | --- | --- | --- |
| 5 | 512 | 36 | 14 | 40 |
| 6 | 512 | 36 | 14 | 40 |
| 7 | 512 | 36 | 14 | 40 |
| 10 | 1024 | 72 | 28 | 40 |
| 15 | 1536 | 108 | 44 | 40 |
| 20 | 2048 | 144 | 58 | 40 |
| 25 | 2048 | 144 | 72 | 50 |
| 30 | 3072 | 216 | 108 | 50 |
| 35 | 3072 | 216 | 108 | 50 |
| 40 | 4096 | 288 | 144 | 50 |
| 45 | 4096 | 288 | 144 | 50 |
| 50 | 4096 | 288 | 144 | 50 |
| NOTE: These percentages are informative and apply to a slot's symbols 1 to 6 and 8 to 13. Symbols 0 and 7 have a longer CP and therefore a lower percentage. |  |  |  |  |

Proposal 6: Reuse the EVM window length as follows:

| Channel bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (%) |
| --- | --- | --- | --- | --- |
| 6 | 512 | 36 | 14 | 40 |

# BS RF requirements: Occupied bandwidth test requirement

For the Occupied bandwidth test requirement, we need to define values of the Span, as well as the minimum number of measurement points. Span shall the double of the channel bandwidth, while the minimum number of measurement points is same as for 5 MHz and 7 MHz channels, as presented below:

Table 3: Span and number of measurement points for OBW measurements for FR1

| Bandwidth | BS channel bandwidth BWChannel (MHz) |  |  |  |  |  |  | Aggregated BS channel bandwidth BWChannel_CA (MHz) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 5 | 6 | 7 | 10 | 15 | 20 | > 20 | > 20 |
| Span (MHz) | 10 | 12 | 14 | 20 | 30 | 40 | 2×BWChannel |  |
| Minimum number of measurement points | 400 | 400 | 400 | 400 | 400 | 400 | 2×BWChannel100kHz |  |

Proposal 7: For the Occupied bandwidth test requirement, define Span as 12 MHz, with Minimum number of measurement points as 400.

# BS RF requirements: Dynamic range

For the Rx dynamic range requirement, there was previous discussion whether or not to define a new FRC. As it was concluded that there is no new FRC introduced for 7 MHz, we were reusing the same FRC (G-FR1-A2-1) as was used for 15 kHz SCS in channel bandwidths of 5 MHz, 10 MHz and 15 MHz. Therefore, it makes sense to reuse the same FRC also for 6 MHz channel.

Proposal 8: For RF Rx requirements, reuse FRC G-FR1-A2-1 for the 6 MHz channel with 15 kHz SCS.

With the above, and assuming SNR of 9.3 dB, NF of 5 dB for WA BS, NF of 10 dB for MR BS, NF of 13 dB for LA BS and Implementation Margin of 2.5 dB, the conducted core dynamic range requirement for 6 MHz is defined as:

Table 4: WA BS conducted core dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -73.6 | -84.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -70.7 | -82.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
| 6 | 15 | G-FR1-A2-1 | WA BS: -70.7 MR BS: -65.7 LA BS: -62.7 | WA BS: -81.7 MR BS: -76.7 LA BS: -73.7 | AWGN |
| 7 | 15 | G-FR1-A2-1 | -70.7 | -81.0 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.7 | -79.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.7 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |

Proposal 9: For Dynamic range conducted core requirement, define 6 MHz requirement with the following signal values:

Wanted signal:

- WA BS: -70.7 dB

- MR BS: -65.7 dB

- LA BS: -62.7 dB

Interfering signal:

- WA BS: -81.7 dB

- MR BS: -76.7 dB

- LA BS: -73.7 dB

For OTA core requirement, the same applies, being offset by ΔOTAREFSENS:

Table 5: WA BS OTA core dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 5 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -82.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |
| 6 | 15 | G-FR1-A2-1 | WA BS: -70.7 - ΔOTAREFSENS MR BS: -65.7 - ΔOTAREFSENS LA BS: -62.7 - ΔOTAREFSENS | WA BS: -81.7 - ΔOTAREFSENS MR BS: -76.7 - ΔOTAREFSENS LA BS: -73.7 - ΔOTAREFSENS | AWGN |
| 7 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -81.0- ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -79.3- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.4- ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.7- ΔOTAREFSENS | -77.5- ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4- ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.4- ΔOTAREFSENS |  |  |

Proposal 10: For Dynamic range OTA core requirement, define 6 MHz requirement with the following signal values:

Wanted signal:

- WA BS: -70.7 dB - ΔOTAREFSENS

- MR BS: -65.7 dB - ΔOTAREFSENS

- LA BS: -62.7 dB - ΔOTAREFSENS

Interfering signal:

- WA BS: -81.7 dB - ΔOTAREFSENS

- MR BS: -76.7 dB - ΔOTAREFSENS

- LA BS: -73.7 dB - ΔOTAREFSENS

Assuming TT value of 0.3 dB for the wanted signal, the conducted test requirement becomes:

Table 6: WA BS conducted test dynamic range

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) / BWConfig | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A2-15 | -73.6 | -84.7 | AWGN |
| 5 | 15 | G-FR1-A2-1 | -70.7 | -82.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
| 6 | 15 | G-FR1-A2-1 | WA BS: -70.4 MR BS: -65.4 LA BS: -62.4 | WA BS: -81.7 MR BS: -76.7 LA BS: -73.7 | AWGN |
| 7 | 15 | G-FR1-A2-1 | -70.7 | -81.0 | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.7 | -79.3 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.7 | -77.5 | AWGN |
|  | 30 | G-FR1-A2-2 | -71.4 |  |  |
|  | 60 | G-FR1-A2-3 | -68.4 |  |  |

Proposal 11: For Dynamic range conducted test requirement, define 6 MHz requirement with the following signal values:

Wanted signal:

- WA BS: -70.4 dB

- MR BS: -65.4 dB

- LA BS: -62.4 dB

Interfering signal:

- WA BS: -81.7 dB

- MR BS: -76.7 dB

- LA BS: -73.7 dB

For OTA test requirement, the same applies, being offset by ΔOTAREFSENS:

Table 7: WA BS OTA test dynamic range for f ≤ 6.0 GHz

| BS | Subcarrier | Reference | Wanted signal mean power (dBm) |  |  | Interfering | Type of |
| --- | --- | --- | --- | --- | --- | --- | --- |
| channel bandwidth (MHz) | spacing (kHz) | measurement channel (annex A.2) | f ≤ 3.0 GHz | 3.0 GHz < f ≤ 4.2 GHz | 4.2 GHz < f ≤ 6.0 GHz | signal mean power (dBm) / BWConfig | interfering signal |
| 5 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -82.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
| 6 | 15 | G-FR1-A2-1 | WA BS: -70.4 – ΔOTAREFSENS MR BS: -65.4 – ΔOTAREFSENS LA BS: -62.4 – ΔOTAREFSENS | WA BS: -70.4 – ΔOTAREFSENS MR BS: -65.4 – ΔOTAREFSENS LA BS: -62.4 – ΔOTAREFSENS | WA BS: -70.4 – ΔOTAREFSENS MR BS: -65.4 – ΔOTAREFSENS LA BS: -62.4 – ΔOTAREFSENS | WA BS: 81.7 – ΔOTAREFSENS MR BS: -76.7 – ΔOTAREFSENS LA BS: -73.7 – ΔOTAREFSENS | AWGN |
| 7 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -81.0 – ΔOTAREFSENS | AWGN |
| 10 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -79.3 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |
| 15 | 15 | G-FR1-A2-1 | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -70.4 – ΔOTAREFSENS | -77.5 – ΔOTAREFSENS | AWGN |
|  | 30 | G-FR1-A2-2 | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS | -71.1 – ΔOTAREFSENS |  |  |
|  | 60 | G-FR1-A2-3 | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS | -68.1 – ΔOTAREFSENS |  |  |

Proposal 12: For Dynamic range conducted test requirement, define 6 MHz requirement with the following signal values:

Wanted signal:

- WA BS: -70.4 dB – ΔOTAREFSENS

- MR BS: -65.4 dB – ΔOTAREFSENS

- LA BS: -62.4 dB – ΔOTAREFSENS

Interfering signal:

- WA BS: -81.7 dB – ΔOTAREFSENS

- MR BS: -76.7 dB – ΔOTAREFSENS

- LA BS: -73.7 dB – ΔOTAREFSENS

# BS RF requirements: ACS

For ACS requirement, despite adding a new channel BW value, there is need to define the value of Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap. Based on the pre-defined formula captured in the TR 38.817-2, the following was derived:

Table 8: OTA ACS interferer frequency offset for BS type 1-O

| BS channel bandwidth of the lowest/highest carrier received (MHz) | Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap (MHz) | Type of interfering signal |
| --- | --- | --- |
| 5 | ±2.5025 | 5 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |
| 6 | ±2.5125 |  |
| 7 | ±2.5075 |  |
| 10 | ±2.5075 |  |
| 15 | ±2.5125 |  |
| 20 | ±2.5025 |  |
| 25 | ±9.4675 | 20 MHz DFT-s-OFDM NR signal, 15 kHz SCS, 100 RBs |
| 30 | ±9.4725 |  |
| 35 | ±9.4625 |  |
| 40 | ±9.4675 |  |
| 45 | ±9.4725 |  |
| 50 | ±9.4625 |  |
| 60 | ±9.4725 |  |
| 70 | ±9.4675 |  |
| 80 | ±9.4625 |  |
| 90 | ±9.4725 |  |
| 100 | ±9.4675 |  |

Proposal 13: For ACS, define the value of Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap as ±2.5125 MHz.

# BS RF requirements: ICS

As discussed in R4-2507437, selection of the RB number for the Interfering signal should consider TS 38.211 specification, where the range of RB values for the PUSCH is limited to a predefined set which is due to the DFT process principles. The bandwidth of the PUSCH in terms of resource blocks () shall fulfil the following equation:

where  is a set of non-negative integers.

For our discussion, and limiting it by the interferer RB values for 5MHz and 10MHz wanted channels (i.e., 10RB and 25RB, respectively), we can derive the range of allowed values to the following:

MRBPUSCH=[10,12,15,16,18,20,24,25]

Currently, the interfering signal for 5 MHz wanted signal is set to 10 RB, while for 7 MHz wanted signal the interfering signal is set to 15 RB. Therefore, for 6 MHz wanted signal it may be reasonable to select value of 12RB for the interfering signal.

Proposal 14: Define ICS interferer bandwidth of the PUSCH in terms of resource blocks as [12RB].

Depending on the final decision, the ICS requirement for 6 MHz channel bandwidth is expected to look as presented in table 9:

Table 9: Wide Area BS in-channel selectivity

| BS channel bandwidth (MHz) | Subcarrier spacing (kHz) | Reference measurement channel | Wanted signal mean power (dBm) | Interfering signal mean power (dBm) | Type of interfering signal |
| --- | --- | --- | --- | --- | --- |
| 3 | 15 | G-FR1-A1-20 | -102.8 | -83.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 6 RBs |
| 5 | 15 | G-FR1-A1-7 | -100.6 | -81.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 10 RBs |
| 6 | 15 | G-FR1-A1-7 | -100.6 | [-81.4] [-80.6] [-79.6] | DFT-s-OFDM NR signal, 15 kHz SCS, [10, 12, or 15] RBs |
| 7 | 15 | G-FR1-A1-7 | -100.6 | -79.6 | DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs |
| 10, 15, 20, 25, 30, 35 | 15 | G-FR1-A1-1 | -98.7 | -77.4 | DFT-s-OFDM NR signal, 15 kHz SCS, 25 RBs |

# BS RF requirements: Other requirements

There is a range of BS RF requirements, where limits are not modified, and the only modification needed is to add “6 MHz” to the list of NR channel bandwidths. Therefore:

Proposal 15: Update the following requirements by adding 6 MHz to the list of NR channel bandwidths:

- ACLR / CACLR

- OTA ACLR / OTA CACLR,

- ACS / OTA ACS,

- Blocking / OTA blocking,

- EIS / EISREFSENS.

# 3	Conclusions

Based on the above discussion, the following proposals were formulated:

Proposal 1: Proceed with the following work-split, as the starting point.

| Technical specification | Companies |
| --- | --- |
| TS 38.104 | [Ericsson] |
| TS 38.141-1 | [Nokia] |
| TS 38.141-2 | Huawei |
| TS 38.101-1 | [CR responsible: Qualcomm Sub-parts split (draft CRs only): System parameters: Nokia Tx: Huawei Rx and Annexes (if needed): Qualcomm] |
| TS 38.101-4 | [Qualcomm] |
| TS 38.133 | [ZTE] |
| TS 38.307 | [China Telecom] |
| TS 37.104 | [ZTE] |
| TS 37.141 | [ZTE] |
| TS 38.106 (*) | [Ericsson] |
| TS 38.115-1 (*) | [CATT] |
| TS 38.115-2 (*) | [CATT] |
| TS 38.174 (*) | [Nokia] |
| TS 38.176-1 (*) | [Nokia] |
| TS 38.176-2 (*) | Huawei |
| TS 38.113 | [ZTE] |
| TS 37.113 | [Ericsson] |
| TS 37.105 | Huawei |
| TS 37.145-1 | Huawei |
| TS 37.145-2 | Huawei |
| (*) The NCR and IAB specs would need to be updated to clarify 6 MHz channel BW is not supported. |  |

Proposal 2: 6 MHz channel bandwidth should not be supported in the following IAB and NCR specifications: TS 38.106, TS 38.115-1, TS 38.176-2.

Proposal 3: Apply the following procedure for EMC specifications TS 38.113 and TS 37.113 to avoid the need for explicitly introduce 6 MHz channel bandwidth:

- During RAN4#117, handle TS 38.113 and TS 37.113 under the Maintenance agenda to implement channel bandwidth agnostic approach, e.g., align with the NR UE approach in TS 38.124.

- During RAN#110, remove EMC specifications TS 38.113 and TS 37.113 from the list of impacted specifications in the WID.

Proposal 4: Define the Core requirement (both conducted and OTA) for the Total power dynamic range (15 kHz SCS, 30 RB allocation) as 14.7 dB.

Proposal 5: Define the Test requirement (both conducted and OTA) for the Total power dynamic range (15 kHz SCS, 30 RB allocation, 0.4 dB TT) as 14.3 dB.

Proposal 6: Reuse the EVM window length as follows:

| Channel bandwidth (MHz) | FFT size | Cyclic prefix length for symbols 1-6 and 8-13 in FFT samples | EVM window length W | Ratio of W to total CP length for symbols 1-6 and 8-13 (%) |
| --- | --- | --- | --- | --- |
| 6 | 512 | 36 | 14 | 40 |

Proposal 7: For the Occupied bandwidth test requirement, define Span as 12 MHz, with Minimum number of measurement points as 400.

Proposal 8: For RF Rx requirements, reuse FRC G-FR1-A2-1 for the 6 MHz channel with 15 kHz SCS.

Proposal 9: For Dynamic range conducted core requirement, define 6 MHz requirement with the following signal values:

Wanted signal:

- WA BS: -70.7 dB

- MR BS: -65.7 dB

- LA BS: -62.7 dB

Interfering signal:

- WA BS: -81.7 dB

- MR BS: -76.7 dB

- LA BS: -73.7 dB

Proposal 10: For Dynamic range OTA core requirement, define 6 MHz requirement with the following signal values:

Wanted signal:

- WA BS: -70.7 dB - ΔOTAREFSENS

- MR BS: -65.7 dB - ΔOTAREFSENS

- LA BS: -62.7 dB - ΔOTAREFSENS

Interfering signal:

- WA BS: -81.7 dB - ΔOTAREFSENS

- MR BS: -76.7 dB - ΔOTAREFSENS

- LA BS: -73.7 dB - ΔOTAREFSENS

Proposal 11: For Dynamic range conducted test requirement, define 6 MHz requirement with the following signal values:

Wanted signal:

- WA BS: -70.4 dB

- MR BS: -65.4 dB

- LA BS: -62.4 dB

Interfering signal:

- WA BS: -81.7 dB

- MR BS: -76.7 dB

- LA BS: -73.7 dB

Proposal 12: For Dynamic range conducted test requirement, define 6 MHz requirement with the following signal values:

Wanted signal:

- WA BS: -70.4 dB – ΔOTAREFSENS

- MR BS: -65.4 dB – ΔOTAREFSENS

- LA BS: -62.4 dB – ΔOTAREFSENS

Interfering signal:

- WA BS: -81.7 dB – ΔOTAREFSENS

- MR BS: -76.7 dB – ΔOTAREFSENS

- LA BS: -73.7 dB – ΔOTAREFSENS

Proposal 13: For ACS, define the value of Interfering signal centre frequency offset from the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap as ±2.5125 MHz.

Proposal 14: Define ICS interferer bandwidth of the PUSCH in terms of resource blocks as [12RB].

Proposal 15: Update the following requirements by adding 6 MHz to the list of NR channel bandwidths:

- ACLR / CACLR

- OTA ACLR / OTA CACLR,

- ACS / OTA ACS,

- Blocking / OTA blocking,

- EIS / EISREFSENS.

# 4	References

[1]	RP-252942	UE RF enhancements for NR FR1, Phase 5, WID

[2]	R4-2502870	WF on requirements for 7MHz channel bandwidth, RAN4#114