3GPP TSG-RAN WG4 Meeting #117	                                                                                            R4-2520161

Dallas, US, 13th –17th Oct, 2025

Source: 	CATT

Title: 	Discussion on UE RF requirements for Ku band HD-FDD VSAT

Agenda item:	7.6.4.1

Document for:	Approval

- Introduction

In RAN4 #116-bis meeting, the WF [1] for NTN and IoT NTN phase 2 HD-FDD VSAT has been approved. However, there remain some issues need to be discussed. This paper shares some views on NTN and IoT NTN phase 2 HD-FDD VSAT.

- Discussion

In RAN4 #116-bis meeting, a WF for HD-FDD VSAT has been approved, as shown below:

- Topic #1: UE RF requirements for HD-FDD for Ku band with FR1 numerology ### Sub-topic 1-1: Issues for HD-FDD requirements #### Issue 1-1-1: Antenna type assumption for HD-FDD VSAT - WF - The working assumption for Phase 1 is that the HD-FDD VSAT antenna type is a single physical aperture with Tx/Rx switching capability. - Other implementations are not precluded. #### Issue 1-1-2: RF requirements for fixed VSAT for HD-FDD - WF - For FR1-NTN Ku band n248, n247, the FDD requirements for Type 2 VSAT and Type 3 VSAT are based on the Type 5 and Type 6 requirements regarding maximum output power, OTA reference sensitivity level, and maximum input level. The same principle should be applied to HD-FDD. - Further check the regulation related requirements related to NS_203N~NS_208N for the fixed VSAT operating in n248 and n247 #### Issue 1-1-3: Transmit ON/OFF time mask for HD-FDD - WF - Consider reusing existing Transmit ON/OFF time mask requirement as starting point. #### Issue 1-1-4: Treatment on HD-FDD specifications. - WF - Start with FDD requirements as the baseline and further check if any RF requirements need to be modified for HD-FDD. - Update the SAN RF requirements if necessary. #### Issue 1-1-5: EIRP requirements for HD-FDD - WF - Consider the existing FDD VSAT EIRP requirements for HD-FDD as the starting point - FFS on whether to modify. #### Issue 1-1-6: EIS requirements for HD-FDD - WF - Consider the existing FDD VSAT EIS requirements for HD-FDD as the starting point - FFS on whether to modify. Sub-topic 1-2: Issues for VSAT with small antennas (e.g., 20x20cm) #### Issue 1-2-1: New VSAT type - WF - Study potential new VSAT type(s) for small antenna with the following candidates: - VSAT that support LEO only - VSAT that support both GSO and LEO #### Issue 1-2-2: Assumptions for VSAT with small antenna - WF - Conduct implementation and feasibility studies for the small antenna considering, but not limited to, the following aspects: - Antenna size, number of antenna elements, antenna gain, Tx power per element and link budget evaluation for supporting LEO only or supporting both GSO and LEO. #### Issue 1-2-3: ACLR and ACS for the new VSAT with small antenna - WF - Consider the existing Ku-band ACLR and ACS as the starting point - FFS on whether to modify. - Encourage companies to provide justifications for whether to re-use the existing specifications. #### Issue 1-2-4: Specifications impact for HD-FDD VSAT with small antenna - WF - FFS in the next meeting.

2.1 General

The WID is aimed to introducing NTN HD-FDD for Ku band in phase 1. Firstly, the applicability of HD-FDD VSAT requirements should be discussed. Considering the demands of stakeholders for HD-FDD VSAT, the introduction of the HD-FDD architecture is actually mainly aimed at reducing the implementation difficulty of the NTN VSAT, which does not imply a reduction in the NTN VSAT capability. Meanwhile, considering that the requirements defined for Ku band in FR1 are all OTA requirements, while currently OTA requirements do not include content of RedCap. Therefore, when defining requirements for HD-FDD VSAT, only the impact caused by the HD-FDD feature should be considered, rather than regarding HD-FDD VSAT as a RedCap device by default.

Observation 1: For NTN VSAT operating in Ku band, the HD-FDD VSAT is not a RedCap VSAT.

2.2 HD-FDD VSAT requirements

Meanwhile, the impact of introducing FR1 Ku band HD-FDD VSAT on the spec should be determined. It is foreseeable that for VSAT type 2, 3, 5 and 6, the effect of introducing of HD-FDD on RF requirements will be negligible.

For transmit ON/OFF time mask, as the transient period of FR1 Ku band VSAT is already 10us, hence the existing Transmit ON/OFF time mask requirement could be reused.

Proposal 1: The existing Transmit ON/OFF time mask requirement could be reused for HD-FDD VSAT.

For EIRP requirements, as the introduction of HD-FDD VSAT has no significant impact on the VSAT antenna, it could be considered that there is no additional impact on the EIRP requirements. Therefore, the existing FDD VSAT EIRP requirements could be reused for HD-FDD VSAT.

Proposal 2: The existing FDD VSAT EIRP requirement could be reused for HD-FDD VSAT.

For EIS requirements, the REFSENS for HD-FDD is typically more stringent than FDD (no Tx self-interference and lower IL). However, in Ku band, IL for FDD is usually negligible, rendering the tightening of EIS for HD-FDD compared to FDD essentially negligible. Therefore, the existing FDD VSAT EIS requirements could be reused for HD-FDD VSAT.

Proposal 3: The existing FDD VSAT EIS requirement could be reused for HD-FDD VSAT.

During the previous meeting, the impact on SAN RF requirements by introducing HD-FDD VSAT was discussed as well. Considering that the introduction of HD-FDD scheme is primarily concentrated on the VSAT side, no potential impact on the SAN RF requirements have been identified.

Proposal 4: No SAN RF impact has been observed by introducing HD-FDD VSAT.

2.3 New HD-FDD VSAT type RF requirements

For the new HD-FDD VSAT type with small size antenna, the following table shows the RF requirements need to be defined:

Table 2.3-1 NTN VSAT RF requirements for HD-FDD VSAT operating in Ku band FR1 numerology

| NTN VSAT Tx/Rx requirement | Proposals for new type HD-FDD VSAT with small siza antenna |
| --- | --- |
| 9.2.1 NTN VSAT maximum output power | Specification impact. Small antenna size will decrease the gain → TRP and EIRP impact |
| 9.2.2 Off-axis EIRP emission density limit within the operating band | No specification impact. |
| 9.3.3 Transmit ON/OFF time mask | No specification impact. |
| 9.5.1 Occupied bandwidth | No specification impact. |
| 9.5.2 Out of Band Emissions | No specification impact. |
| 9.5.3 Spurious Emissions | No specification impact. |
| 9.6.2 Antenna performance | No specification impact. |
| 9.7 Additional regional requirements indicated by NS | No specification impact. |
| 10.3 OTA reference sensitivity level | Specification impact. Small antenna size will decrease the gain, however for HD-FDD VSAT, the Tx and Rx may use single antenna panel. → reevaluate the EIS |
| 10.4 Maximum input level | Specification impact. To introduce new reference measurement channel for HD-FDD VSAT |
| 10.5 Adjacent channel selectivity | No specification impact. |
| 10.6 Blocking characteristics | No specification impact. |
| 10.8 Receiver antenna off-axis performance | No specification impact. |

Proposal 5: To use the proposals in Table 2.3-1 for new type NTN HD-FDD VSAT with small size antenna RF requirements.

2.4 Specification impact for HD-FDD VSAT

In the discussion above, we have outlined the potential impact of introducing HD-FDD VSAT for the RAN4 specification. Meanwhile, the impact to RAN1/2 spec could also be evaluated. From RAN1 perspective, the following table shows the potential impact on RAN1 specs:

Table 2.4-1 Potential impact of introducing of HD-FDD VSAT on RAN1 specifications

| RAN1 specification | Impact | Note |
| --- | --- | --- |
| TS 38.211 | No specification impact | Transition time NRx-Tx and NTx-Rx has been defined for both FR1 and FR2. |
| TS 38.213 | New IE needed but UE behavior unchanged, no study needed | From CollisionHandlingOfHDFDDOperation to CollisionHandlingOfHDFDDOperation-VSAT |
| TS 38.214 | Wording change but UE behavior unchanged, no study needed | From “For the case of a reduced capability half-duplex UE” To “For the case of a reduced capability half-duplex UE or a half-duplex VSAT UE” |

Through analysis of the aforementioned impact on the RAN1 specification, it is worth noting that all the modifications are band agnostic and numerology agnostic.

Proposal 6: The impact of introducing HD-FDD VSAT to the RAN1 specification is band agnostic and numerology agnostic.

From RAN2 perspective, the new IE and capabilities related to HD-FDD for both SAN and HD-FDD VSAT should be defined. Since the IE and capabilities related to HD-FDD in the current spec are coupled with RedCap, it is necessary to define new IE and capabilities that are decoupled from RedCap to ensure the proper functioning of HD-FDD VSAT. From the perspective of SAN, at least a new HD-FDD allowed IE and capabilities decoupled from RedCap needs to be defined. Otherwise, SAN will not be able to indicate that the HD-FDD VSAT can operate in the cell, causing the HD-FDD VSAT to be barred. From the perspective of HD-FDD VSAT, a similar approach could also be considered. Therefore, the following IE and capabilities should be defined:

| Definitions for parameters | Per | M | FDD-TDD DIFF | FR1-FR2 DIFF |
| --- | --- | --- | --- | --- |
| [halfDuplexVSAT-Allowed] The presence of this field indicates that the cell supports half-duplex FDD VSAT. | Band | NO | FDD only | NO |
| [halfDuplexFDD-TypeA-VSAT] Indicates whether the VSAT only supports half-duplex operation for FDD in the indicated band(s). | Band | NO | FDD only | NO |

Through analysis of the aforementioned impact on the RAN2 specification, it is worth noting that all the modifications are band agnostic and numerology agnostic as well.

Proposal 7: The impact of introducing HD-FDD VAST to the RAN2 specification is band agnostic and numerology agnostic.

- Conclusion

This contribution provides our initial views on RF requirements for HD-FDD VSAT. The following proposals are concluded as follows:

Observation 1: For NTN VSAT operating in Ku band, the HD-FDD VSAT is not a RedCap VSAT.

Proposal 1: The existing Transmit ON/OFF time mask requirement could be reused for HD-FDD VSAT.

Proposal 2: The existing FDD VSAT EIRP requirement could be reused for HD-FDD VSAT.

Proposal 3: The existing FDD VSAT EIS requirement could be reused for HD-FDD VSAT.

Proposal 4: No SAN RF impact has been observed by introducing HD-FDD VSAT.

Proposal 5: To use the proposals in Table 2.3-1 for new type NTN HD-FDD VSAT with small size antenna RF requirements.

Proposal 6: The impact of introducing HD-FDD VSAT to the RAN1 specification is band agnostic and numerology agnostic.

Proposal 7: The impact of introducing HD-FDD VAST to the RAN2 specification is band agnostic and numerology agnostic.

- References

- R4-2515082, Way Forward for [116bis][330] NR_IoT_NTN_Ph2_Part2, MTK, RAN4 #116-bis