# 3GPP TSG-RAN WG4 Meeting #117		R4-2520216

# Dallas, USA, Nov. 17-21, 2025

Agenda Item:	8.2.1

Source: 	Amazon

Title: 	Consideration on 6GR waveform

Document for:	Discussion

# Introduction

The 6G Radio Study Item was approved at the RAN#108 meeting (RP-251881) [1]. The main objectives are:

- Single technology framework based on a stand-alone architecture (Note1) to support the agreed existing and new services, and to satisfy the usage scenarios, requirements, deployment scenarios and design principles with acceptable performance/complexity trade-off, as determined by the RAN requirements in [RP-250810] and [TR38.914], including: [RAN1], [RAN2], [RAN3], [RAN4] - Ensuring appropriate set of functionalities, minimize the adoption of multiple options for the same functionality, avoid excessive configurations, excessive UE capabilities and UE capabilities reporting. - Energy efficiency and energy saving: both for network and device. - Enhanced spectral efficiency. - Enhanced overall coverage, focus on cell-edge performance and UL coverage. - Wider channel bandwidth (at least 200MHz) support for 6G deployments at least above 2 GHz, around 7 GHz. - Re-use of existing 5G mid-band (~3.5GHz) site grid for 6G deployments in at least around 7 GHz and targeting comparable coverage to 5G mid-band. - Target scalable and forward compatible design for diverse device types. - Improved spectrum utilization and operations taking into account diverse spectrum allocations. - Aim at using common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs. - Aim at a harmonized 6G Radio design for TN and NTN, including their integration. - System simplification, including reducing configuration complexity, enabling more efficient Cell/UE management, etc. Note1: the term stand-alone architecture does not imply any particular Core network architecture, which is up to SA2 discussion.

In RAN4#116bis, the following is agreed:

R4-2514650	WF on 6G system parameter

- Framework study for waveform

  - Model and evaluate the RF performance of different waveform candidates and PAPR reduction techniques pending on RAN1 inputs

    - Evaluation cases could be selected step-by-step, based on RAN1 progress

    - Whether to investigate RF impacts of DL DFT-s-OFDM pending on RAN1 agreement

- Devices Type: The following aspects could be further studied

  - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range.

    - Study on the NW impact should also be considered

  - Study how to better support variety devices from RAN4 perspective

    - E.g. differentiate devices, use cases, features

  - Other aspects are not precluded

- Collaborate on the framework with other WGs:

  - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

# Discussion

PAPR reduction techniques

Unlike terrestrial base stations, PA efficiency is highly critical in case of NTN satellite payloads, whether regenerative or bent-pipe, due to limited on-board power sources. Therefore, 6G should consider different OFDM crest factor reduction techniques applicable to downlink transmissions from NTN base stations.

## GNSS-less operation

GNSS-less is an important use case for 3GPP NTN in the evolution toward 6G, as it addresses a key limitation today’s communication system: 5G NTN depends on the navigation system which can be unavailable or unreliable in various environments such as dense urban areas, indoors, or in regions affected by interference and spoofing. By enabling 6G to operate independently of GNSS, NTN can provide a more resilient and ubiquitous communication framework, capable of supporting critical applications in emergency response, autonomous mobility, industrial IoT.

A few designs needed to support GNSS-less NTN

- Downlink synchronization signal should be robust to large doppler shift

- Uplink RACH design must be robust to large doppler shift and large timing uncertainty (detail can be further discussed)

- Support enhanced close-loop RTD and RTT adjustment. Need to consider the signaling overhead

# Conclusion

In this contribution, we propose the following:

PAPR reduction

Proposal 1: Explore and study a variety of waveform candidates to improve the PAPR of downlink transmissions from NTN base stations.

GNSS-resilient proposals:

Proposal 2: Support transmission of multiple preambles by the UE in different PRACH Occasions.

Proposal 3: Support on-demand transmission of a second preamble by the UE based on feedback from the gNB, e.g., via Random Access Response.

Proposal 4: For transmitting multiple preambles by the UE, the relation between the root indices to be studied with the objective of achieving maximum robustness to peak detection inaccuracies.