3GPP TSG-RAN WG4 Meeting #117	R4-2520254

Dallas, USA, Nov. 17-21, 2025

Title: 	Co-existence on CA_n12-n29 and CA_n28-n67

Source: 	Huawei, HiSilicon

Agenda item:	4.8.1

Document for:	Approval

# Introduction

RAN4#116bis approved an WF of [1], where BS to BS co-existence issue is listed as one of the items to be further studied. This contribution shares our views in order to proceed with the discussion.

# Discussion

## BS-to-BS co-existence

The below Figure 1 is an excerpt of spectrum arrangement for two example band combinations from TR 38.768.

Figure 1: An excerpt of frequency arrangements for example BCs from TR38.768

First of all, the WIDs [2, 3] for Rel-19 and Rel-20 clearly describe that “Co-located and synchronized network deployment for both carriers”.

Another aspect is that the WIs were established in order to fully utilize an SDL band such as n29 together with an FDD low band such as n12 in order to resolve poor data speeds due to low-band congestion. According to the justification part of [2], it seems that n29 and n12 are already deployed.

According to our preliminary analysis assuming the existing base stations, it wouldn’t be feasible for a pair of n12 and n29 or a pair of n28 (lower 30 MHz) and n67 to co-exist under co-located scenario. For example, the existing n12 BS Rx (n12 UL reception) shall have sufficient rejection against n12 DL at least over 729-746 MHz so that the BS Rx doesn’t have sufficient isolation against n29 DL.

In this case, in order to introduce CA_n12-n29 or CA_n28-n67, the existing BSs implementation may have to be changed.

Hence, before going into the deep BS co-existence study, it would be really helpful which bands are widely used, and the respective proponents holding the bands have plan to change the existing BS implementation or not.

## UE-to-UE co-existence

Unlike BS-to-BS co-existence, UE-to-UE co-existence may or may not happen depending on distance between UEs, Tx power of an aggressor UE, received power of a victim UE and/or relative frequency positions for the Tx resource (aggressor) and the Rx resource (victim), etc.

In terms of co-channel interference from FDD band (aggressor) to SDL band (victim), there was a draft CR [4] endorsed in RAN4#116bis. Though we don’t have a strong view, it may not be reasonable to put more restriction on FDD band due to promotion of SDL utilization, given that FDD band’s spectrum resource specifically for UL is limited.

# Conclusions

In this paper we provided our views on R20 LB-LB SW.

For BS-to-BS co-existence

Observation 1: The WIDs for Rel-19 and Rel-20 target at “Co-located and synchronized network deployment for both carriers”.

Observation 2: According to Rel-19 WID, n12 and n29 are already widely deployed.

Observation 3: The existing BS following 3GPP requirements and not considering co-existence between n12 and n29 or between n28 (lower 30 MHz) and n67 wouldn’t have sufficient isolations between an FDD band and an SDL band so that the co-existence is not feasible under co-located scenario.

Observation 4: If the observation 3 is right, the existing BSs’ implementation may have to be changed in order to introduce e.g., CA_n12-n29 or CA_n28-n67 into the proponent’s network.

Proposal: Regarding CA_n12-n29 and CA_n28-n67, clarify which bands are widely deployed, and the respective proponents holding the corresponding bands have plan to change the existing BS implementation or not or one of the bands per band combination.

For UE-to-UE co-existence

Observation 5: Imposing a more limitation (more stringent UE-to-UE co-existence requirement and associated resource block restriction) on FDD band may not be reasonable, given that FDD band’s spectrum resource specifically for UL is very valuable.

Observation 6: If an operator (s) who uses Low band CA switch, e.g., CA_n12-n29, has either the upper portion of an FDD band or the entire FDD band and either the lower portion of an SDL band or the entire SDL band, it may be good for the operator to make a decision, since their UEs are the most affected by the decision in any case.

# Reference

- R4-2515156, Way Forward for [116bis][320] NR_low_band_CA_switching, Moderator (Apple)

- RP-252694, Revised WID on Low NR band carrier aggregation via switching, Telus

- RP-252953, New WID on enhancement of low NR band carrier aggregation via switching, Moderator (Huawei)

- R4-2514603, (NR_LBCA_Sw-Core) Missing requirements for low NR band CA via switching in TS38.101-1, MediaTek Inc.