3GPP TSG-RAN WG4 Meeting #117	R4-2520332

Dallas, USA, Nov 17 – 21, 2025

Agenda Item:	8.8.2

Source:	Huawei, HiSilicon

Title:	Discussion on RAN4-driven AI/ML use cases

Document for:	Approval

- Introduction

Guided by the framework of the RAN4 agenda in [1], RAN4 driven use cases are discussed in Q4'25 and Q1'26. RAN4 impacts of other WG driven use cases and RAN for AI will be discussed after Q1'26.

According to the WF from last meeting [2], in this paper, we provide our views on the RAN4 driven AI/ML uses cases.

- AI for SRS assisted channel reconstruction

## Motivations for SRS enhancement in 6G

SRS plays a critical role in CSI acquisition, particularly as 6G evolves toward an increased number of CSI-RS ports, and CSI acquisition via CSI-RS confronts greater challenges.

- First, NW is constrained in obtaining high-accuracy CSI from CSI feedback, as it may heavily rely on UE’s AI capabilities, including AI based CSI compression capability and AI based frequency and/or spatial domain CSI prediction capability. Both technologies are key enablers for supporting a larger number of CSI-RS ports and are hot topics in RAN1’s 6G AI work [3]. AI based CSI compression is required to reduce CSI feedback overhead while preserving acceptable CSI recovery accuracy, whereas AI based frequency and/or spatial domain CSI prediction is leveraged to reduce CSI-RS overhead. If two-sided models are involved, an inter-vendor collaboration challenge emerges as an additional barrier to achieving high-accuracy CSI acquisition.

- Second, the power consumption of AI-based CSI feedback is higher than that of SRS transmission. This is because the transmission of SRS and CSI feedback is of the same order of magnitude, while AI-based CSI feedback incurs additional power consumption for AI model operations. This initial analysis is based on the following assumptions, though we recognize that lower-complexity AI model designs are expected to reduce power consumption in real-world implementations. Conducting a trade-off analysis between performance and complexity when selecting AI-based CSI feedback or SRS transmission is beneficial for UE power saving.

|  |  | AI based CSI feedback | eType2 like CSI feedback | SRS |
| --- | --- | --- | --- | --- |
| AI based CSI compression | Computational Complexity | 10M FLOPs | 0.37M FLOPs | / |
|  | Power consumption | ACompressionAI CSI×10-XmAh | ACompressioneTypeII×10-X-2mAh | / |
| AI based frequency and/or spatial domain CSI prediction | Computational Complexity | 40M FLOPs | / | / |
|  | Power Consumption | APredictionAI CSI×10-XmAh | / | / |
| Transmission | Power Consumption | ATransmissionAI feedback×10-X mAh | ATransmissioneTypeII feeback×10-X mAh | ATransmissionSRS×10-X mAh |

Observation 1: NW is constrained in obtaining high-accuracy CSI from CSI feedback for a larger number of CSI-RS ports supported by 6GR, as it may heavily rely on UE’s AI capabilities.

Observation 2: The power consumption of AI-based CSI feedback for a larger number of CSI-RS ports is higher than that of SRS transmission.

Considering that CSI acquisition via SRS offers greater flexibility to NW to acquire CSI with higher spatial and frequency resolution, thereby helping enhance downlink transmission performance, and also supports UE power saving, SRS is an attractive complement to, or even a potential substitute for, CSI-RS in numerous 6G scenarios.

Observation 3: SRS is an attractive complement to, or even a potential substitute for, CSI-RS in numerous 6G scenarios.

## Considerations for 6G SRS enhancement scenarios

CSI acquisition via SRS confronts several challenges, including a) SRS power imbalance, b) channel aging and c) coverage limitation. Based on whether SRS suffers from a), b) and/or c), the scenarios can be categorized into the following cases.

|  | Condition on SINR | Scenarios description | SRS issues |  |  | Candidate Solutions | If issues are solved, whether CSI-RS is needed |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | a) power imbalance | b) channel aging | 3) coverage limitation |  |  |
| Case 1 | High SINR | Cell center without strong interference | × | × | × | / | No |
| Case 2 | Medium to Low SINR | Power-limited scenarios (e.g., mean and cell edge, indoor), interference-limited scenarios | √ | × | √/× | SRS power imbalance compensation | No |
| Case 3 | High SINR | Mobility scenarios with frequency hopping (long period to collect full CSI for DL transmission) | × | √ | × | Fusion of SRS and UL DMRS [4] | No |
| Case 4 | Low SINR | Coverage-limited scenarios, mobility scenarios with frequency hopping | √ | √ | √ | Fusion of SRS and CSI-RS | Yes |

In Case 1, though SRS IL exists, UE can correct the power imbalances by itself. For example, for PC3 with two ports, when the primary port power is below (23 dBm - ∆TRxSRS,1), the secondary port power can be supplemented by UE to keep it the same as that of the primary port.

In Case 2, UE may perform self-compensation but cannot fully compensate the insertion loss, resulting in a residual power imbalance. For example, for PC3 with two ports, when the primary port power is above (23 dBm - ∆TRxSRS,1) and ∆TRxSRS,1 is 3 dB, if the primary port power is 21, 22 or 23 dBm, the secondary port power would be 20 dBm, 20 dBm and 20 dBm, respectively. Thus, the actual power deltas, i.e., the remaining insertion loss, are 1, 2 or 3 dB depending on the primary reference port power. It means that even though the insertion loss ∆TRxSRS,1 is 3 dB, the values that network uses for correcting DL channel estimation may not always exactly follow ∆TRxSRS,p. The power imbalance issue is exacerbated by the support for a higher number of antennas and upper mid-band spectrum in 6GR, for example, an 8Rx UE operating in the U6G.

Observation 4: If SRS power imbalance is well compensated, the coverage of antenna switching SRS can be extended, without relying on CSI-RS.

In Case 3, the scenario of focus is that longer intervals required for CSI acquisition from SRS with frequency hopping. This channel aging issue is exacerbated by the support for a larger bandwidth, more antennas at UE to be sounded, and a larger number of UEs with emerging services in a cell - all of which align with the evolving directions of 6GR. Leveraging other RSs - for example, the UL DMRS - in a complementary fashion through configured or dynamically scheduled PUSCHs enables more flexible updates of CSI. By using AI and occasionally present UL DMRS to perform CSI prediction, this channel aging issue could be resolved. The detailed solution can be found in our companion paper submitted to RAN1 [4].

Observation 5: If SRS channel aging is well resolved, the coverage of antenna switching SRS can be extended, without relying on CSI-RS.

In Case 4, we consider a coverage-limited scenario in which the primary port reaches its maximum transmission power, and even in the absence of power imbalance and channel aging, the accuracy of CSI derived from SRS is unacceptable. In this scenario, fusion of CSI-RS and SRS could be a candidate solution to achieve high-accuracy CSI.

## AI based SRS residual power imbalance compensation

### Historical discussions on SRS power imbalance in RAN4 and RAN1

Antenna switching SRS was introduced by RAN1, while the power imbalance issue was identified in RAN4. SRS power imbalance is induced by insertion loss, specifically, variations in losses arising from differences in switches and antenna traces across diversity branches. SRS power imbalance hinders the accurate acquisition of DL CSI. In 5G, RAN4 has defined a set of requirements for ∆TRxSRS across different frequency bands and antenna configurations, as shown in Section 6.2.4 of 38101-1. For example, for ‘t1r6’ and ‘t2r6’, the value of ∆TRxSRS is 10 dB for bands whose FUL_high is higher than the FUL_low  of n104 (U6G, 6425~7125MHz). As 6G evolves toward higher frequency bands and a larger number of UE Rx, the magnitude of SRS power imbalance induced by insertion loss will consequently increase. If this imbalance is not well compensated, the accuracy of CSI acquisition will be dramatically affected.

Observation 6: As 6G evolves toward higher frequency bands and a larger number of UE Rx, the magnitude of SRS power imbalance induced by insertion loss will consequently increase.

In Case 2 of Section 2.2, if SRS power imbalance is well compensated, the coverage of antenna switching SRS can be extended. As analyzed in Section 2.1, investigating compensation for SRS power imbalance is urgent for 6GR to support a larger number of CSI-RS ports. This is because even for UEs that do not support AI-based CSI compression and AI-based frequency and/or spatial domain CSI prediction, NW can still achieve high-accuracy CSI in many scenarios.

Observation 7: Investigating compensation for SRS power imbalance to improve the accuracy of CSI is urgent for 6GR to support a larger number of CSI-RS ports.

Non-AI based compensation methods can simply be the scaling of the estimated UL received signals or the estimated UL channel matrix. Regardless of whether the UE compensates for insertion loss, it is beneficial for the NW to identify the presence and magnitude of residual power imbalance across diversity branches.

RAN4 in R18 has identified several candidate solutions for supporting UE to report the actual residual power imbalance, which has been forwarded to RAN1 with reference to LS [5]. Based on simulation results provided by companies within RAN1, and as referenced in the FL Summary in [6], it has been doubly confirmed that SRS power imbalance results in non-negligible performance degradation. However, due to that there exists multiple companies hold concern towards the directly/indirectly reporting of SRS IL imbalance, the conclusion is that there is no consensus to support directly/indirectly reporting of SRS IL imbalance in RAN1.

Observation 8: SRS power imbalance issue have been confirmed in both RAN4 and RAN1; however, RAN4-identified solutions for supporting non-AI based compensation method failed to achieve consensus in RAN1.

### Motivations on AI SRS power imbalance compensation

Key reasons for no consensus in 3GPP RAN1 are as follows. Many companies argue that pre-compensation/self-supplement could be sufficient for power-unlimited UEs, and for power-limited UEs at the cell edge, even a few dB of residual power imbalance is insufficient to justify the benefit of dynamic UE reporting. This may be valid for UEs in low-frequency bands with fewer antennas. However, for 6G-oriented UEs in higher-frequency bands with more antennas, it cannot be claimed that the power imbalance is only a few dB. 3GPP RAN4’s existing indicators already reach as high as 10 dB, and for 6G, this indicator will only increase further. Even if some UEs can maintain nearly balanced power via self-compensation, for other UEs that still have residual power imbalance after self-compensation, improving channel recovery accuracy through NW compensation is valuable for enhancing 6G system performance.

Observation 9: For UEs with more antennas in higher-frequency bands that cannot maintain power balance, compensation at NW is beneficial to enhancing 6G system performance.

Another concern is the signaling overhead issue. Considering that different UEs may adopt distinct strategies for whether or how to implement pre-compensation, and even for a single UE, its capability to pre-compensate the loss induced by ∆TRxSRS,p (p=0, 1, …) varies under different PCMAX_L, f, c scenarios, dynamic reporting could be signaling intensive.

Companies also suspect that, when considering realistic channel estimation at gNB, even if UE reports the residual power imbalance to gNB, when the received SRS power at the gNB is low, whether this reporting would really help gNB to improve the channel estimation performance.  This view is consistent with the analysis of our simulation results. As shown in Figure 1, at medium to low SNRs, even with exact reporting, obvious compensation loss is still observed with non-AI based compensation is performed at the NW side. The reason is that while amplifying the received signal power, it not only amplifies the channel information but also the noise. The performance loss caused by noise amplification is greater than the performance gain from channel information amplification, which may therefore result in negative gain.

Figure 1 SGCS for non-AI-based compensation under SRS power imbalance

Observation 10: Non-AI compensation has two key issues: one is the signaling overhead caused by dynamic UE reporting of residual power imbalance, and the other is that under low SNR conditions, even with such reporting, high-precision channel recovery cannot be achieved.

If AI-based compensation methods are introduced, the aforementioned situation will change. On one hand, AI has a certain generalization capability and does not necessarily require UEs to dynamically report the actual values of residual power imbalance. This helps reduce signaling overhead. On the other hand, AI can learn the nonlinear relationship between channel estimation under power imbalance and that under power balance even in low SNR scenarios. By compensating for residual power imbalance, channel estimation performance can be improved to approach that under no power imbalance.

Observation 11: AI based SRS power imbalance compensation has the potential to avoid reporting actual values of residual power imbalance, thereby reducing signaling overhead, and to improve compensation performance even under low SNRs.

### Preliminary evaluation results

The model input of AI model is the measured CSI derived from SRS with power imbalance, while the output is the measured CSI free of such power imbalance. Measured CSI is in the format of channel matrix. One measured channel matrix is extracted every 4 RBs, with a dimension of [M×N], where M denotes the number of CSI-RS ports and N denotes the number of UE Rx. The AI model is designed to process the [M×N] channel matrix corresponding to 4 RBs each time. On the one hand, this aligns with the typical precoding granularity of 4 RBs; on the other hand, it helps reduce the dimension of the model’s input, thereby achieving a low-complexity model.

Assuming a bandwidth of 100 MHz with a total of 272 RBs, the RBs are divided such that each group consists of 4 RBs, resulting in 68 groups in total, thus requiring the AI model to perform 68 inferences. For these 68 inferences, the same AI model is used, which indicates that the AI model has a certain generalization capability in the frequency domain.

While one can explore the use of AI models to simultaneously address both the SRS power imbalance issue and the channel aging issue, however, considering that these two issues do not coexist in all scenarios, it is also valuable to address them separately. When only SRS power imbalance exists, addressing this issue alone results in lower complexity of the AI model.

We focus on the scenario where only SRS power imbalance exists. Assuming SRS is configured with 40ms period and 4 frequency hops (with 68 RBs per frequency hop), it requires 160ms to obtain the full-bandwidth CSI. Since we do not consider the channel aging issue, we set the Doppler frequency to 1 Hz. Note that SRS is usually not configured for full-bandwidth transmission due to UE power constraints; otherwise, it would result in low received power at gNB for each RE, which severely impairs channel estimation performance.

Figure 2 SRS frequency hopping pattern

During model training, the model input is the SRS measurements with residual power imbalance, and the model output is the measured CSI that are free of residual power imbalance at the same time instance. Regarding how to obtain labels in practical networks, the following solutions are identified.

- CSI feedback based on CSI-RS

- SRS measurements that are processed via time domain filtering to achieve high enough SNR

- SRS measurements with perfect pre-compensation at UE

During model testing, the model input remains the SRS measurements with residual power imbalance. Once the input SRS measurements are compensated using AI, the model output is the corresponding measured CSI with negligible power imbalance.

Considering that SRS measurements are applied in downlink PMI calculation under the scenario of channel reciprocity, the evaluation KPI is calculated as follows:

- First, obtain full-bandwidth ideal channel matrix for D slot

- Then perform SVD decomposition for each subband to derive eigenvectors

- Subsequently, find the full-bandwidth channel matrix constructed from AI-recovered CSI at the S slots closest to the D slot, and also perform SVD decomposition for each subband to derive its eigenvectors

- Finally, compare these eigenvectors (from the AI-recovered CSI) with the eigenvectors obtained from the ideal channel matrices to calculate SGCS

For modeling the SRS power imbalance distribution, two cases are considered: 1) actual values of [0, -2.5, -5, -7.5] dB, and 2) a distribution where [0, -2.5, -5, -7.5] dB serve as the mean values, with uniform [-1,1] dB randomness added around each mean, denoted as [0, -2.5, -5, -7.5] ±1dB. Accordingly, two AI models are trained - one for scenario-specific use case and the other for generalized use case, respectively. Specifically, the training data for the first AI model consists of CSI generated from the actual values, while the training data for the second model consists of CSI generated from the distribution [0, -2.5, -5, -7.5] ±1dB. For both cases, training datasets are collected covering a range of SNRs, which helps achieve model’s generalization across SNRs. The other simulation parameters are listed in the following table.

Table 1. Simulation parameters for AI based SRS residual power imbalance compensation

| Parameters | Values |
| --- | --- |
| Carrier frequency/BW | 5GHz/100MHz/272RBs |
| CSI-RS ports | 32 |
| Tx/Rx number @UE | 1T4R |
| Channel model | TDL-A XPL medium |
| Doppler | 1Hz |
| TDD ULDL pattern | DDDDDDDSUU |
| Power imbalance distribution | [0, -2.5, -5, -7.5] dB |
|  | [0, -2.5, -5, -7.5]±1dB |
| SRS pattern | 4 frequency hopping, 40ms between two SRS hopping |

If CSI-RS measurements are adopted as the ground truth during model training, the performance gains delivered by the AI model may stem from two sources: first, compensation for residual SRS power imbalance, and second, enhanced channel estimation accuracy. This is because the received power of DL CSI-RS is significantly higher than that of UL SRS—and higher received power directly improves SNR for channel estimation. As a result, channel estimation accuracy based on DL CSI-RS is naturally superior to that based on UL SRS. This creates a higher-accuracy baseline: the AI’s gains not only correct errors from SRS power imbalance but also narrow the gap between UL SRS-based estimation and the CSI-RS-derived baseline.

By contrast, if SRS measurements free of residual power imbalance are used as the ground truth, the AI model’s performance gains are attributable purely to the compensation of residual SRS power imbalance. No additional gain from "enhanced channel estimation" exists here, as the ground truth itself already represents UL SRS measurements with ideal power balance—eliminating the accuracy gap present when CSI-RS is used as the reference.

Figure 3 SGCS for AI-based compensation under SRS residual power imbalance

It can also be observed that if, during the training phase, the AI model has been exposed to dataset statistics derived from the distribution [0, -2.5, -5, -7.5] ± 1 dB, then during the inference phase, the AI’s compensation performance remains desirable for testing data that still originates from this distribution—approaching the performance achieved in the specific scenario where the model is trained at [0, -2.5, -5, -7.5] dB and tested at [0, -2.5, -5, -7.5] dB.

Observation 12: AI-based SRS residual power imbalance compensation delivers desirable performance even under medium-to-low SNR conditions. Moreover, it provides an error tolerance of at least 1 dB for reporting—meaning there is no need to report updates to the SRS residual power imbalance when changes fall within this 1 dB range.

### Template for details and evaluation procedures

According to the WF in [2], the aforementioned details of the proposed AI based SRS residual power imbalance compensation is summarized by using the exemplary template.

Proposal 1: Details of AI-based SRS residual power imbalance compensation are summarized in Table 2.

Table 2. AI based SRS residual power imbalance compensation

| AI model input | Input in training | UL measured channel matrix from SRS with residual power imbalance |
| --- | --- | --- |
|  | Input in inference | UL measured channel matrix from SRS with residual power imbalance |
| AI model output | Label in training (if applicable) | CSI measurement without power imbalance, candidate solutions could be: - CSI feedback based on CSI-RS - SRS measurements that are processed via time domain filtering to achieve high enough SNR - SRS measurements with perfect pre-compensation at UE |
|  | Output in inference | Ideal DL channel matrix |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Dataset construction: A total of 100,000 TTIs (Transmission Time Intervals) are allocated as follows: - 70,000 TTIs: Only S slots are used for model training; - 10,000 TTIs: Both S slots and D slots are used for model validation; - 20,000 TTIs: Both S slots and D slots are used for model testing. |
| model location for inference |  | NW-sided AI |
| Collaboration/interaction between UE and NW |  | 1. Inference: UE reporting on the range of SRS residual power imbalance for model training and model selection 2. Data collection/monitoring: UE reporting/assistance information for constructing labels, e.g., CSI feedback based on CSI-RS or SRS measurements with perfect pre-compensation |
| Evaluation methodology |  | Parameters; Values. Carrier frequency/BW; 5GHz/100MHz/272RBs. CSI-RS ports; 32. Tx/Rx number @UE; 1T4R. Channel model; TDL-A XPL medium. Doppler; 1Hz. TDD ULDL pattern; DDDDDDDSUU. Power imbalance distribution; [0, -2.5, -5, -7.5] dB. [0, -2.5, -5, -7.5]±1dB. SRS pattern; 4 frequency hopping, 40ms between two SRS hopping. |
| Evaluation assumption |  | A common AI model is trained which applies to each channel matrix extracted from 4RBs. The model input/output is the channel matrix with the dimension [M×N], where M denotes the number of CSI-RS ports and N denotes the number of UE Rx. Channel aging issue is not considered in this use case. |
| Evaluation KPI |  | SGCS Steps for SGCS calculation during model testing - First, obtain full-bandwidth ideal channel matrix for D slot - Then perform SVD decomposition for each subband to derive eigenvectors - Subsequently, find the full-bandwidth channel matrix constructed from AI-recovered CSI at the S slots closest to the D slot, and also perform SVD decomposition for each subband to derive its eigenvectors - Finally, compare these eigenvectors (from the AI-recovered CSI) with the eigenvectors obtained from the ideal channel matrices to calculate SGCS |
| Evaluation benchmark |  | 1) SRS measurements without residual power imbalance 2) non-AI based SRS residual power imbalance compensation |
| preliminary evaluation results |  | •AI/ML based SRS residual power imbalance compensation outperforms non-AI/ML based compensation across the entire SNR range, with more obvious performance gains observed at medium to low SNRs. •AI/ML is capable of compensating without actual values reporting, meaning there is no need to report updates to the SRS residual power imbalance when changes fall within reasonable range. |
| High level potential specification impact |  | 1) UE assistance information/reporting related to SRS residual power imbalance 2) Potential UE RF related testing |
| Feasibility issues, including complexity, and other aspects related to implementation |  | CNN-based AI model |

Beyond the technical details outlined above, evaluation procedures are also provided for facilitating companies to identify this use case.

Proposal 2: In RAN4 6G AI, consider supporting AI based SRS residual power imbalance compensation. The evaluation procedures are listed as below.

  - Step-1: Assumptions on simulation parameters, including channel models, number of UE antennas, frequency band and SRS residual power imbalance distributions are determined to be used in the evaluation.

  - Step-2: Evaluation methodology by modelling SRS residual power imbalance to baseband evaluation with the following procedure

    - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

      - Training dataset and testing dataset construction

      - Evaluation metric

      - AI models and training type

      - Other details are not precluded

- AI for DPoD

The nonlinear characteristics of PA significantly impact the performance (e.g., EVM, MPR) of RF and affect the signal quality and coverage. To address such issue, AI DPoD can be considered which utilizes neural network for compensating nonlinearity of PA, as shown in Figure 4.

Figure 4 AI/ML based DPoD

For AI based DPoD, apart from the basic performance gain evaluation considering individual setting, it is also necessary to study the generalization performance of AI models across multiple scenarios/configurations. One of key reasons behind this point is that vendors cannot train individual AI DPoD models for each parameter combination, which could be an enormous figure in practice. Typical factors for model generalization evaluation include various PA models (both memory- and memoryless-models), bandwidth configurations, waveforms, MCS, etc. Taking generalization for different PA models as an example, we need to consider both individual AI models for each PA model and a unified AI model for multiple PA models, and compare their performance under each PA model. More details about the evaluation methodology can be referred to TR38.843.

It could be non-trivial for AI DPoD models to simultaneously achieve satisfying generalization performance under all potential factors. For example, if we train a unified AI DPoD model to compensate the non-linear distortion from both memory- and memoryless-PA models, the AI model may find it difficult to learn a suitable common pattern for both scenarios (since the PA non-linearity characteristics are very different) and suffer from severe performance loss. To overcome such challenge, we can leverage some auxiliary reference signal to provide insights of current nonlinear characteristics of PA, e.g., DMRS or known data signal.

Proposal 3: For AI DPoD, performance implications of different characteristics of PA need to be considered.

- Conclusions

Observation 1: NW is constrained in obtaining high-accuracy CSI from CSI feedback for a larger number of CSI-RS ports supported by 6GR, as it may heavily rely on UE’s AI capabilities.

Observation 2: The power consumption of AI-based CSI feedback for a larger number of CSI-RS ports is higher than that of SRS transmission.

Observation 3: SRS is an attractive complement to, or even a potential substitute for, CSI-RS in numerous 6G scenarios.

Observation 4: If SRS power imbalance is well compensated, the coverage of antenna switching SRS can be extended, without relying on CSI-RS.

Observation 5: If SRS channel aging is well resolved, the coverage of antenna switching SRS can be extended, without relying on CSI-RS.

Observation 6: As 6G evolves toward higher frequency bands and a larger number of UE Rx, the magnitude of SRS power imbalance induced by insertion loss will consequently increase.

Observation 7: Investigating compensation for SRS power imbalance to improve the accuracy of CSI is urgent for 6GR to support a larger number of CSI-RS ports.

Observation 8: SRS power imbalance issue have been confirmed in both RAN4 and RAN1; however, RAN4-identified solutions for supporting non-AI based compensation method failed to achieve consensus in RAN1.

Observation 9: For UEs with more antennas in higher-frequency bands that cannot maintain power balance, compensation at NW is beneficial to enhancing 6G system performance.

Observation 10: Non-AI compensation has two key issues: one is the signaling overhead caused by dynamic UE reporting of residual power imbalance, and the other is that under low SNR conditions, even with such reporting, high-precision channel recovery cannot be achieved.

Observation 11: AI based SRS power imbalance compensation has the potential to avoid reporting actual values of residual power imbalance, thereby reducing signaling overhead, and to improve compensation performance even under low SNRs.

Observation 12: AI-based SRS residual power imbalance compensation delivers desirable performance even under medium-to-low SNR conditions. Moreover, it provides an error tolerance of at least 1 dB for reporting—meaning there is no need to report updates to the SRS residual power imbalance when changes fall within this 1 dB range.

Proposal 1: Details of AI-based SRS residual power imbalance compensation are summarized in Table 2.

Table 2. AI based SRS residual power imbalance compensation

| AI model input | Input in training | UL measured channel matrix from SRS with residual power imbalance |
| --- | --- | --- |
|  | Input in inference | UL measured channel matrix from SRS with residual power imbalance |
| AI model output | Label in training (if applicable) | CSI measurement without power imbalance, candidate solutions could be: - CSI feedback based on CSI-RS - SRS measurements that are processed via time domain filtering to achieve high enough SNR - SRS measurements with perfect pre-compensation at UE |
|  | Output in inference | Ideal DL channel matrix |
| Assumption on training | Training type | Offline training |
|  | Label construction (if applicable) | Dataset construction: A total of 100,000 TTIs (Transmission Time Intervals) are allocated as follows: - 70,000 TTIs: Only S slots are used for model training; - 10,000 TTIs: Both S slots and D slots are used for model validation; - 20,000 TTIs: Both S slots and D slots are used for model testing. |
| model location for inference |  | NW-sided AI |
| Collaboration/interaction between UE and NW |  | 1. Inference: UE reporting on the range of SRS residual power imbalance for model training and model selection 2. Data collection/monitoring: UE reporting/assistance information for constructing labels, e.g., CSI feedback based on CSI-RS or SRS measurements with perfect pre-compensation |
| Evaluation methodology |  | Parameters; Values. Carrier frequency/BW; 5GHz/100MHz/272RBs. CSI-RS ports; 32. Tx/Rx number @UE; 1T4R. Channel model; TDL-A XPL medium. Doppler; 1Hz. TDD ULDL pattern; DDDDDDDSUU. Power imbalance distribution; [0, -2.5, -5, -7.5] dB. [0, -2.5, -5, -7.5]±1dB. SRS pattern; 4 frequency hopping, 40ms between two SRS hopping. |
| Evaluation assumption |  | A common AI model is trained which applies to each channel matrix extracted from 4RBs. The model input/output is the channel matrix with the dimension [M×N], where M denotes the number of CSI-RS ports and N denotes the number of UE Rx. Channel aging issue is not considered in this use case. |
| Evaluation KPI |  | SGCS Steps for SGCS calculation during model testing - First, obtain full-bandwidth ideal channel matrix for D slot - Then perform SVD decomposition for each subband to derive eigenvectors - Subsequently, find the full-bandwidth channel matrix constructed from AI-recovered CSI at the S slots closest to the D slot, and also perform SVD decomposition for each subband to derive its eigenvectors - Finally, compare these eigenvectors (from the AI-recovered CSI) with the eigenvectors obtained from the ideal channel matrices to calculate SGCS |
| Evaluation benchmark |  | 1) SRS measurements without residual power imbalance 2) non-AI based SRS residual power imbalance compensation |
| preliminary evaluation results |  | •AI/ML based SRS residual power imbalance compensation outperforms non-AI/ML based compensation across the entire SNR range, with more obvious performance gains observed at medium to low SNRs. •AI/ML is capable of compensating without actual values reporting, meaning there is no need to report updates to the SRS residual power imbalance when changes fall within reasonable range. |
| High level potential specification impact |  | 1) UE assistance information/reporting related to SRS residual power imbalance 2) Potential UE RF related testing |
| Feasibility issues, including complexity, and other aspects related to implementation |  | CNN-based AI model |

Proposal 2: In RAN4 6G AI, consider supporting AI based SRS residual power imbalance compensation. The evaluation procedures are listed as below.

  - Step-1: Assumptions on simulation parameters, including channel models, number of UE antennas, frequency band and SRS residual power imbalance distributions are determined to be used in the evaluation.

  - Step-2: Evaluation methodology by modelling SRS residual power imbalance to baseband evaluation with the following procedure

    - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

      - Training dataset and testing dataset construction

      - Evaluation metric

      - AI models and training type

      - Other details are not precluded

Proposal 3: For AI DPoD, performance implications of different characteristics of PA need to be considered.

- References

[1] RAN4 6G Plan and Meeting Arrangement, R4-2511652, RAN4 #116, Aug, 2025.

[2] WF on 6G AI, R4-2514637, RAN4 #116b, Aug, 2025.

[3] Chair notes RAN1 #122b, RAN1 #122b, Aug, 2025.

[4] Views on AI/ML in 6GR air interface, R1-2508740, Huawei, Hisilicon, Nov, 2025.

[5] LS on the UE SRS IL imbalance issue, R4-2303519, Feb, 2023.

[6] FL summary on UE SRS IL imbalance issue, R1-2308458, Aug, 2023.