# 3GPP TSG-RAN WG4 Meeting #117                                   R4-2520431

# Dallas, USA, Nov. 17-21, 2025

Agenda item:	8.2.5

Source:	CMCC

Title:	Discussion on 6GR device type

Document for:	Discussion

Introduction

In RAN#108, the technology study item for 6G radio (6GR) has been approved [1]. The anticipated services of 6GR range from immersive communication and massive communication to integrated sensing and communication. Furthermore, 6G services can be provided by both terrestrial network (TN) and non-terrestrial network (NTN). in last meeting, WF[2] and LS[3] are approved which show the high level guideline for further discussion.

- In this contribution, we show our initial analysis on device type.

Discussion

Device types

According to the RAN #109 device type discussion, we have following services

Proposal 1: 6G Massive Communication (IoT) shall be supported for FR1. - 6GR should have a common/scalable design that supports the above usage scenario in addition to eMBB - Prioritize 6GR design for eMBB - The above usage scenario should not overlap with Ambient IoT and NB-IoT Proposal 2: For 6G Massive Communication (IoT), the minimum number of UE receive/transmit antenna is 1 for lowest-tier device - Reflect this agreement in the device type section of the TR Moderator proposal 1: It is agreed to capture following into TR 38.914 section 5.4: - Mobile broadband - Immersive Communication - Massive Communication (IoT) - Sensing - AI - Voice - Regulatory services Moderator proposal 2: It is agreed that ‘Observability’ will be discussed in architecture requirements. Moderator proposal 3: Whether to capture following into TR 38.914 section 5.4 will be further discussed: - [Non-Terrestrial Network] - [Aerial] - [FWA] - [Vehicles] - [Mission Critical Communication] - [Positioning and/or Navigation] - [TSN] - [HRLLC] Moderator proposal 4: - 6GR should study the sensing modes, including TRP monostatic, TRP-TRP bistatic, TRP-UE DL, UE-TRP UL, UE-UE bistatic and UE monostatic. 6GR should study cooperative sensing/multi-static sensing mechanism involving multiple UEs and TRPs

- Agreement - The following aspects could be further studied - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered - Study how to better support variety devices from RAN4 perspective - E.g. differentiate devices, use cases, features - Other aspects are not precluded - Collaborate on the framework with other WGs: - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

For IoT devices, 3GPP has defined several IoT technologies as shown below with different RF requirements, e.g. A-IoT, NB-IoT, eMTC, e/Redcap, LTE categories, etc. Considering regular technology iteration and potential re-farming of previous network in 6G era, 6GR should support device type with similar battery-life and cost to the existing LPWA technologies, i.e. NB-IoT/eMTC. Other IoT use cases should also be supported by 6GR with scalable design. [5]

For eMBB devices, 3GPP RAN4 has defined several sets of requirements based on different UE RF performance. For example: For NR FR2, the UE type is coupled with power class and RAN4 define RF requirements based on the category of UE power class.

In 6G, RAN4 should further discuss the methodology of how to differentiate UE types, following list some options for further discussion.

- Option 1: Different UE type is coupled with different UE capability sets

  - Option 1a: Define minimum capability sets. Different UE types is coupled with different UE capability sets on top of the minimum capability sets

- Option 2: LTE category like UE types, different types are classified based on different throughput

- Option 3: combine LTE category like UE types and FR2 power class related method. I.e. different UE types are classified based on the combination of throughput and RF front end categories.

- To better combine FWA, eMBB including high-end and low end, IoT including high-end and low end devices, our suggestion is to further discuss device types from following aspects, the [] means further study is needed

- Maximum UE bandwidth

  - FFS the minimum CBW which is related to the sync raster design, e.g. FFS whether different UE device types support different sync raster step

- Tx/Rx antenna number and maximum MIMO layers

- Duplex mode, Full-duplex / Half-duplex

- Modulation order

- Coverage extension

- [power class]

- Proposal 1: for device type, it’s suggested to take above aspects into consideration.

- Following list our preliminary suggestion for different characteristics of device types

|  | - Max UE bandwidth | - Tx/Rx antenna number | - Duplex mode | - Max modulation order | - coverage | - [power class] |
| --- | --- | --- | --- | --- | --- | --- |
| - [FWA] | - 400M | - [16T, 16R] | - TDD - FDD | - [1024] QAM | - normal | - PC1 |
| - eMBB | - [400M] | - 4T, 8R | - TDD - FDD | - [256QAM] | - normal | - PC1.5 |
| - [low end eMBB] | - 100M | - 4T, 8R | - TDD - FDD | - 256QAM | - normal | - PC1.5 |
| - High end IoT | - 20M | - 1T, 1/2R | - HD-FDD - TDD - FDD | - 64QAM | - normal | - TBD |
| - Lowest tier IoT | - 5M | - 1T,1R | - HD-FDD - TDD | - 16QAM | - Normal+10dB | - [20dBm] |

- Proposal 2: it’s suggested to further discuss device types with above table as example.

Conclusion

- This contribution makes the following observations and proposals.

- Maximum UE bandwidth

  - FFS the minimum CBW which is related to the sync raster design, e.g. FFS whether different UE device types support different sync raster step

- Tx/Rx antenna number and maximum MIMO layers

- Duplex mode, Full-duplex / Half-duplex

- Modulation order

- Coverage extension

- [power class]

- Proposal 1: for device type, it’s suggested to take above aspects into consideration.

- Following list our preliminary suggestion for different characteristics of device types

|  | - Max UE bandwidth | - Tx/Rx antenna number | - Duplex mode | - Max modulation order | - coverage | - [power class] |
| --- | --- | --- | --- | --- | --- | --- |
| - [FWA] | - 400M | - [16T, 16R] | - TDD - FDD | - [1024] QAM | - normal | - PC1 |
| - eMBB | - [400M] | - 4T, 8R | - TDD - FDD | - [256QAM] | - normal | - PC1.5 |
| - [low end eMBB] | - 100M | - 4T, 8R | - TDD - FDD | - 256QAM | - normal | - PC1.5 |
| - High end IoT | - 20M | - 1T, 1/2R | - HD-FDD - TDD - FDD | - 64QAM | - normal | - TBD |
| - Lowest tier IoT | - 5M | - 1T,1R | - HD-FDD - TDD | - 16QAM | - Normal+10dB | - [20dBm] |

- Proposal 2: it’s suggested to further discuss device types with above table as example.

References

- RP-251881, New SID: Study on 6G Radio, NTT DOCOMO

- R4-2514641, WF on system parameter, HW

- R4-2514643, LS on 6G parameter, HW

- R1-2508069, LS on PA models in 6GR waveform evaluations, HW

- 6GIG, Views on supporting of diverse device types, CMCC,