3GPP TSG-RAN WG4 Meeting #117	R4-2520555

Dallas, US, November 17th – November 21st, 2025

Source:	Nokia, Nokia Shanghai Bell

Title:	Discussion on multi-RAT spectrum sharing for 6G

Agenda item:	8.9

Document for:	Approval

Introduction

At RAN4#116bis meeting, the following was agreed on the spectrum sharing between 5G and 6G. A number of open issues were also captured in the way forward [1]. In this paper, we will continue the discussion on the potential impacts on RAN4 requirements.

## Sub-topic 1-0: Agenda handling - Agreements MRSS should be considered in the ultimate RAN4 decision, including RAN4’s input to interim milestone. Regarding some of the system parameters (e.g. SCS, sync raster, channel raster etc.), separated agenda will be designated for the scenarios of with (in spectrum sharing) and without (in system parameter) MRSS considered. - If feasible, RAN4 strives to define unified system parameters for the scenarios with and without MRSS. - Revisit this agenda arrangement after Feb. meeting in 2026 for potentially merging the discussion under the single agenda. Before RAN4 receives concrete design from RAN1/2, RAN4 discussion will be based on the agreed hypothesis. ## Sub-topic 1-1: General aspects #### Issue 1-1-1: Migration scheme - Agreements - RAN4 focus on MRSS between 6GR and NR until further RANP guidance - RAN4 focus on inter-RAT mobility between 6GR and NR #### Issue 1-1-2: Scenarios for MRSS between 6GR and NR - Agreements - For operating frequency range or band for MRSS between 6GR and NR - Start RAN4 6G-5G MRSS study in FR1 - Whether to consider 6G-5G MRSS in FR2-1 is further discussed ## Sub-topic 1-2: Aspects to facilitate MRSS #### Issue 1-2-0: General consideration Agreement - To facilitate MRSS between 6GR and NR, take following aspects as starting point for RAN4 study - System parameters - RF requirements - RRM requirements - Interference handling - Taking following table as starting point to discuss which system parameters are impacted by MRSS. System parameters; Whether impacted by MRSS. Numerology; Maybe impacted. Channel raster; Maybe impacted. Sync raster; Maybe impacted. Waveform; FFS. Modulation; FFS. Channel bandwidth; FFS.

System parameters

In last RAN4 meeting, it was discussed which system parameters are impacted by MRSS.

In our view, one of the key requirements for the 6G frame structure and numerology is smooth coexistence with 5G NR, particularly in the MRSS scenario, as explicitly stated in 6G SID. Taking the learnings from DSS for migration from 4G to 5G, it is clear that MRSS has to be more efficient, and it needs to be an integral part of the 6GR design. This includes related MRSS considerations in the studies on frame structure and numerology for 6GR (as noted in the SID directly) to allow support for resource-element level sharing between 5G and 6G to maximize efficiency and avoid additional guard overhead in frequency or time. This, in turn, requires not only that the subcarrier spacing is the same in both 5G and 6G, but also the following: the OFDM symbol timing is aligned, the subcarriers are in frequency sync, and preferably that the RB boundaries are also aligned. Additionally, opportunities for hardware reuse and coexistence should be optimized. Effective interference management and aligned interference characteristics are also crucial. These requirements suggest that it should be possible to configure an overlapping resource grid for 5G and 6G, ensuring that OFDM symbol boundaries and RB boundaries are fully aligned.

Consequently, the following was agreed on the channel bandwidth, SCS, the number of symbols in a slot etc. in the latest RAN1 meetings, catering to these requirements and making the 6G time/frequency structures naturally RE-level compatible with the 5G time/frequency structures (bolded text relevant to MRSS):

## RAN1#122 (August/2025) Agreements relevant to MRSS:

Agreement

- 6GR takes the following SCS as start point for discussion for all the signals/channels except PRACH.

  - For sub 6GHz

    - The following subcarrier spacing is at least supported

      - 15kHz SCS for FDD, 30kHz SCS for TDD

    - FFS: 30kHz SCS for FDD for around e.g., 1-2.5GHz

    - FFS: 7.5kHz SCS for sub1GHz (FDD)

    - Whether to discuss the FFS will be subject to RANP decision.

  - For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

  - FFS: For around 15GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz, 120kHz

    - Whether to discuss it will be subject to RANP decision

  - For between 24.25GHz - 52.6GHz

    - Subcarrier spacing 120kHz is supported

  - FFS whether to allow using additional subcarrier spacing for SSB

- FFS subcarrier spacing for PRACH and up to initial access discussion.

Agreement

- 6GR supports normal cyclic prefix, i.e., same as the normal CP defined in NR.

  - FFS potential need for other CP

## RAN1#122bis (October/2025) Agreement relevant to MRSS: ## Agreement ## For communication, 6GR considers NR frame structure used as a starting point for the study item, - Resource defined by one subcarrier and one symbol is called as resource element (RE). - Resource block (RB) is defined where the number of consecutive subcarriers per RB is the same for all numerologies and the number of subcarriers per RB is 12 - Radio Frame length is 10ms - Each radio frame is split into 10 subframes, each with a duration of 1 ms - For given SCS and for given symbol, the symbol duration, normal CP length and boundary is same as NR design. - A slot is defined as supporting 14 consecutive symbols for normal CP case and all subcarrier spacings.

Proposal 1: RAN4 to acknowledge the RAN1#122 agreements on MRSS related scs for sub 6GHz frequency range and discuss if there is any RAN4 related aspects that needs to be consider for scs selection for sub 6GHz or can the RAN1 agreement be taken also as RAN4 baseline for sub 6GHz.

RF requirements

MRSS related RF issue from previous RAN4 meeting.

#### Issue 1-2-6: RF requirements - Proposals from companies - P1(ZTE): for MRSS BS, apply new 6GR BS RF requirements to MRSS BS supporting both 5G and 6G. - P2 (LGE): RAN4 to start RF discussion on the spectrum aggregation with MRSS e.g., 6G PCell(MRSS)+6G SCell - P3 (Ericsson): - When designing UE RF requirements, take into account that 6GR UEs will operate on the same carrier as legacy UEs due to MRSS. - When designing BS RF requirements, take into account that compatibility between 6GR and legacy requirements is needed to facilitate multi-standard BS that can handle both 6GR and legacy RATs - P4 (ISSDU): RAN4 to evaluate 5G/6G RF coexistence in FR1 under MRSS with a quantifiable baseline approach. - Way forward - Further discuss RF requirements impacted by MRSS

  - RAN4 to continue scs discussion for around 7GHz based on RAN1#122 down selection aiming to provide additional information from RAN4 perspective for RAN1 on this matter.For around 7GHz

    - The following subcarrier spacing options can be studied

      - 30kHz, 60kHz

RAN4 to continue discussion on channel raster and sync raster impact to MRSS and whether waveform, modulation and channel bandwidth has impact to MRSS. In our view Waveform needs to be OFDM-based and have compatible subcarrier spacing, RAN1 is working on this, nothing is need from RAN4 point of view now. Modulation order or channel bandwidth do not have impact to MRSS hence RAN4 do not need to consider those in MRSS studies. So, all in all RAN4 does not need major RF work to guarantee MRSS at this stage of 6G SI apart from channel raster and sync raster. Important is that RAN4 follows RAN1 progress closely even RAN1 will send liaisons’ to RAN4 eventually.

Proposal 2: RAN4 to focus RF work on MRSS topic for channel raster and sync raster definition.

RRM requirements

From last meeting, RRM requirements for MRSS were not discussed online but some proposals were captured in the WF for further consideration.

#### Issue 1-2-7: RRM requirements - Proposals from companies - P1 (LGE): RAN4 to start RRM discussion on the MRSS depending on the progress of other WGs. - P2 (Ericsson): RAN4 should study MRSS based measurement and handover requirements if needed, taking into account the RAN1/2 design of MRSS. - P3 (ISSDU): RAN4 to study adaptive RRM coordination for MRSS under partial frequency overlap and different numerology configurations, focusing on dynamic stability and measurable RRM performance. - Way forward - Further discuss RRM requirements impacted by MRSS

On RRM aspect, many of the UE procedures and corresponding RRM requirements depend significantly on the 6GR reference signaling design done by RAN1 e.g. RRM measurements, mobility, SCell activation etc. While RAN1 has agreed to consider NR frame structure as a starting point, the synchronization signals for 6GR are still under discussion. In addition, when MRSS is considered, it is unclear how the 5G and 6G are shared on the radio level in detail. It would be very difficult to discuss the potential impact on RRM requirements due to MRSS without further knowledge from RAN1. Therefore, we would propose to wait for RAN1 discussion on the MRSS details before RAN4 to identify the potential impact on RRM requirements due to MRSS.

Proposal 3: RAN4 to wait for RAN1 discussion on the MRSS details before discussing the potential impact on RRM requirements due to MRSS.

Conclusion

In this paper we briefly discuss MRSS and possible RAN4 work on high level. However, as the RAN4 work has to wait a bit on the RAN1 progress we make the following Proposals:

Proposal 1: RAN4 to acknowledge the RAN1#122 agreements on MRSS related scs for sub 6GHz frequency range and discuss if there is any RAN4 related aspects that needs to be consider for scs selection for sub 6GHz or can the RAN1 agreement be taken also as RAN4 baseline for sub 6GHz.

Proposal 2: RAN4 to focus RF work on MRSS topic for channel raster and sync raster definition.

Proposal 3: RAN4 to wait for RAN1 discussion on the MRSS details before discussing the potential impact on RRM requirements due to MRSS.

- References

- R4-2514646, WF for [116bis][108] 6G spectrum sharing, CMCC