# 3GPP TSG-RAN WG4 Meeting #117	R4-2520739

# Dallas, US, 17th – 21th Octobor, 2025

Source: 	vivo

Title: 	Discussion on 6G modulation

Agenda Item:	8.2.2

Document for:	Approval

Introduction

In this contribution, initial views on 6G system parameters are provided, based on WF[1] of #116bis meeting.

Discussion

The agreement of RAN4 #116bis meeting regarding modulation for 6G is as follows:

Agreement of RAN4 #116bis: - Agreement - RAN4 evaluation work focus on the feasibility study and RF requirements impact - To establish the foundational evaluation framework in RAN4 firstly when no solid progress and inputs from RAN1 - Identify the main affected requirements for modulation evaluations - The existing 5G NR requirements will serve as the baseline, which are subject to future updates based on RAN4’s 6G UE RF discussions. - Align on the evaluation assumptions. - Study on how to align on the transmitter chain model, including PA model, for consistent evaluations on the modulation. - RF evaluation could be done firstly for 5G supported modulations with new assumptions for 6G study, such as assumed new spectrum, CBW, new PA models, etc. Co-ordination with 6G UE RF study is needed. - Both link-level and system-level simulations should be performed as usual for high-order modulations study done by RAN4 in prior releases, pending on the progress of RAN1. - Model and evaluate the performance and the implementation complexity of higher-order modulations, e.g. 1024QAM on the UL and/or new constellations - For high order QAM with uniform constellation, e.g. 1024QAM on the UL, RAN4 can work concurrently with RAN1 studies. - For new non-uniform constellation, the evaluation in RAN4 should depend on RAN1 progress and request.

For higher-order modulation studies, the overall research approach should align with previous modulation order studies conducted during the LTE or NR eras. It is necessary to first investigate feasibility and performance benefits, followed by considering the definition of corresponding requirements.

The feasibility study should be divided into two parts: simulation-based feasibility study and implementation-based feasibility study. Both link-level simulations and system-level simulations are required. Link-level simulations need to identify the minimum required SNR at which 1024QAM demonstrates a performance gain compared to 256QAM. System simulation is targeted as supplementary for link level simulation to further confirm the scenario that UL 1024QAM is applicable.

For the implementation-based feasibility study, the discussion should focus on the impairment sources. EVM is a composite metric resulting from multiple impairment sources: PA non-linearity, transmitter noise, phase noise, IQ imbalance, etc., which involve complex trade-offs. As shown in the fig below (excerpted from the UL 256QAM Tx EVM analysis), implementation-specific budget allocation between different EVM contributors must be considered. Under a given EVM budget, it is essential to evaluate whether each constituent impairment source can meet its corresponding SNR target. If not, these limitations will constrain the achievable EVM performance.

Fig 1. EVM requirement for each contributor, UL 256QAM [2]

Proposal 1: The higher-order modulation evaluation should be started in RAN4 for UL 1024QAM and based on simulation-based feasibility study and implementation-based feasibility study with trade-off between different EVM contributor, including PA non-linearity, transmitter noise, phase noise, IQ imbalance, etc.

The corresponding study of UL 256QAM and the EVM requirement of DL 1024QAM could be considered as a reference for simulation assumption and feasibility study.

Proposal 2: For UL 1024QAM, the study of EVM on DL 1024QAM could be considered as a starting point, the simulation assumptions should be aligned firstly. Support of larger channel bandwidth, e.g., 200MHz should be considered in the impairment of PA.

Furthermore, the impact of higher-order modulation on the spec requirements also need identify, including EVM, MPR and minimum output power. Higher-order modulation signals exhibit larger PAPR and greater sensitivity to PA nonlinearity. This necessitates increased power back-off to maintain signal integrity, thereby leading to a higher MPR. Concurrently, the stricter SNR requirement of higher-order modulations mandates an elevated minimum output power to guarantee signal quality, as shown in Table 6.4.2.1-2 and 6.3.1-1 in TS38.101-1. These two factors collectively compress the UE's practical power dynamic range. Consequently, while MPR analysis can be advanced based on predefined EVM specifications, its study must simultaneously account for dynamic range management to prevent excessive reduction in the available power range.

Proposal 3: Evaluate the main affected RF requirements for 1024QAM UL modulation, including EVM, MPR and minimum output power. MPR evaluation should consider the dynamic range impact.

| ## 6.3 Output power dynamics ### 6.3.1 Minimum output power The minimum controlled output power of the UE is defined as the power in the channel bandwidth for all transmit bandwidth configurations (resource blocks), when the power is set to a minimum value. The minimum output power is defined as the mean power in at least one sub-frame 1 ms. The minimum output power shall not exceed the values specified in Table 6.3.1-1. Table 6.3.1-1: Minimum output power Channel bandwidth; (MHz); 3,5,10,15,20; 25,30,35,40,45,50; 60,70,80,90,100. REF_SCS; - (kHz); - 15; - 30. Minimum output power; - (dBm); - -40; - -40+10log10 (BWChannel /20); - -40+10log10 (BWChannel /20). Measurement bandwidth; - (MHz); - MBW=REF_SCS*(12*NRB+1)/1000. NOTE: The minimum output power value is rounded to the nearest number down to one decimal point.. |  |
| --- | --- |
| Table 6.4.2.1-2: Parameters for Error Vector Magnitude Parameter; Unit; Level. UE Output Power; dBm; Table 6.3.1-1. UE Output Power for 256 QAM; dBm; Table 6.3.1-1 + 10 dB. Operating conditions; Normal conditions. |  |

- Conclusion

Proposal 1: The higher-order modulation evaluation should be started in RAN4 for UL 1024QAM and based on simulation-based feasibility study and implementation-based feasibility study with trade-off between different EVM contributor, including PA non-linearity, transmitter noise, phase noise, IQ imbalance, etc.

Proposal 2: For UL 1024QAM, the study of EVM on DL 1024QAM could be considered as a starting point, the simulation assumptions should be aligned firstly. Support of larger channel bandwidth, e.g., 200MHz should be considered in the impairment of PA.

Proposal 3: Evaluate the main affected RF requirements for 1024QAM UL modulation, including EVM, MPR and minimum output power. MPR evaluation should consider the dynamic range impact.

- References

[1] R4-2514641, WF for 6GR system parameter, Huawei, HiSilicon, RAN4#116bis.

[2] R4-166954, WF on MPR\AMPR simulation assumptions for UL 256QAM, Huawei, Hisilicon, Qualcomm, CMCC, Nokia, Skyworks, Ericsson, RAN4#80.