3GPP TSG-RAN WG4 Meeting # 117	R4-2520742

Dallas, US, Nov. 17th – 21st, 2025

Source: 	vivo, MediaTek

Title: 	Further Discussion on PC1.5 HPUE Requirements for Rel-20 NTN

Agenda Item:	7.6.3

Document for:	Approval

Introduction

In RAN#109, a Rel-20 WI [1] for NTN in RAN4 package has been approved. In this WI, the first part is the HPUE introduce Iot-NTN PC1.5 1Tx.

In RAN4#116bis, a comprehensive discussion paper was submitted in [2], in which it was proposed to re-use most of Rel-19 study. An accompanying draft CR was also submitted in [3] but postponed. A WF document [4] was agreed.

In this contribution, further discussion was made on the remaining issues.

Discussion

General

In the WF [4], there are the following issues:

Issue 2-1-1: Scope clarification, feasibility

Agreement:

- The feasibility of PC 1.5 single Tx NB-IoT NTN UE in both handhelds and non-handhelds is confirmed.

- Both category NB1 and NB2 are in the scope of this WI.

- The eMTC-based IoT-NTN is out of scope of this release.

Issue 2-1-2: Work plan on the TR

Agreement:

- Suggest to update the WID in the next RAN meeting to include a TR for NTN HPUE

Issue 2-1-6: Capability signalling for PC1.5 NB-IoT NTN UE

Agreement:

- The signaling to indicate support for PC1.5 is needed to be added to LTE RRC signalling with preference on early implementation.

Issue 2-1-7: Power boosting for PC1.5 NB-IoT NTN UE

Agreement:

- Power boosting is not considered for PC1.5 NB-IoT NTN UE.

For the general part, it was clarified that the PC1.5 1Tx would be applied for both handheld and non-handheld, and eMTC is excluded. A TR and early implementation of signalling are being considered. Power boosting is not considered.

Most of the agreements are already concrete, except the TR part. It is still not quite clear what kind of scope should this TR include. One basic question is if a TR is available, whether it is for the whole WI, or just for PC1.5 single Tx part. If it is targeted for PC1.5 1Tx, then there is a risk that the contents could be too limited for a dedicated TR, since the technical discussion for this PC1.5 area is quite limited.

Proposal 1: If a TR is being considered, it is proposed to have some further discussion what would be the TR scope for WI of enhanced requirements for NR NTN and IoT NTN Phase 2.

RF requirements

In the previous WF, quite some agreements were reached on UE RF part. The P-MPR as SAR solution, ACLR, SEM, and No Rx impact have been agreed.

The main remaining issues are MPR and A-MPR, and this is discussed as below.

MPR:

As in the WF, the following contents were agreed:

Issue 2-2-6: MPR requirements for PC1.5 NB-IoT NTN UE

Agreement:

- Take the following tables as starting point for the MPR for PC1.5 NB-IoT NTN UE (Based on MPR = maximum {0, PC1 MPR - 2dB})

Table 2-2-6-1 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ [0] dB | [0] dB | ≤ [0] dB | ≤ [1.5] dB | [0] dB | ≤ [1.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table 2-2-6-2 Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [0] dB | ≤ [0] dB |  | ≤ [0] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [0.5]dB |  | ≤ [0.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [2] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

- Interested companies can provide further evaluation in the next RAN4 meeting based on following assumptions to check whether modifications on the above two tables are needed

  - Option 1

    - Calibration points for PC1.5 (based on NR PC2 PA):

      - DFT-s-OFDM QPSK 20 MHz

      - 100RB0

      - 4dB post PA loss /or [5] dB

      - 1dB MPR

      - Carrier leakage: 28 dBc

      - IQ image: 28 dBc / or [31] dBc

      - CIM3: 60 dBc

      - NR ACLR: 31dBc

  - Option 2

    - DFT-s-OFDM 1RE (Single Tone)

    - Centre Single Tone [select one from: 3.75kHz pi/2 BPSK, pi/4 QPSK, 15 kHz pi/2 BPSK, 15KHz QPSK]

    - [3] or [4] dB post PA loss

    - 0dB MPR

    - Carrier leakage 28dBc

    - IQ image:28 dBc

    - CIM3: 60 dBc

  - The result from companies provided in the next RAN4 meeting will be taken into account when deciding the final value

The detailed explanation of the starting point, which is based on MPR = maximum {0, PC1 MPR - 2dB} has been included in last meeting discussion paper. The basic thinking is that similar architecture and components reference, i.e. NR PC2 or GSM PA, are used for PC1.5 as in the discussion, and the power difference between PC1 and PC1.5 is directly used as an offset for MPR.

Still, there are two more views on the starting point:

- View 1: More evaluation can be considered.

- View 2: More relaxed IL can be considered. (Also related to view 1)

We do not object new evaluations. However, many tests and simulation were already done in Rel-19. If it is for the same reference components such as NR PC2 PA, actually there is not too much new simulation/tests can be done, as long as assumptions are the same. Since the simulation/tests are focusing on the emission and other metrics on various power level, thus the declared power class would not have significant impact.

Observation 1: The declared power class would not impact tests / simulations, as long as the same reference components are used / assumptions are the same.

However, still some adjustments can be considered. E.g. more IL would means less MPR offset. If 4dB IL be used, then the MPR = maximum {0, PC1 MPR – 1dB} could be considered.

Proposal 2: The starting point of MPR can be defined as maximum {0, PC1 MPR - 2dB}, minor adjustments can be considered based on new assumptions / evaluations.

Table X1: [Tentative] Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Single-Tone

| Modulation | π/2 BPSK, π/4 QPSK |  |  | π/2 BPSK, π/4 QPSK |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  | 3.75 kHz |  |  |
| Tone positions for 1 Tone allocation | 0-1 | 2-9 | 10-11 | 0-5 | 6-41 | 42-47 |
| MPR | ≤ [0] dB | 0 dB | ≤ [0] dB | ≤ [1.5] dB | 0 dB | ≤ [1.5] dB |
| Note 1: For single-tone allocation, the listed modulations and SCS are supported. |  |  |  |  |  |  |

Table X2: [Tentative] Maximum Power Reduction (MPR) for UE category NB1 and NB2 Power Class 1.5, Multi-Tone

| Modulation | QPSK |  |  |  |
| --- | --- | --- | --- | --- |
| SCS | 15 kHz |  |  |  |
| Tone positions for 3 Tones allocation | 0-2 | 3-5 and 6-8 |  | 9-11 |
| MPR | ≤ [0] dB | ≤ [0] dB |  | ≤ [0] dB |
| Tone positions for 6 Tones allocation | 0-5 and 6-11 |  |  |  |
| MPR | ≤ [0.5] dB |  | ≤ [0.5] dB |  |
| Tone positions for 12 Tones allocation | 0-11 |  |  |  |
| MPR | ≤ [2] dB |  |  |  |
| Note 1: For multi-tone allocation, the listed modulation and SCS are supported. |  |  |  |  |

For the table in the draft CR, it is noted that for edge single tone case, the tentative [0] value looks a bit duplicate with other resource allocation. This is however deliberate, since different values may still be applied based on further evaluation.

A-MPR

In RAN4#114, the following way forward for A-MPR are as following, which is based on earlier discussion in [2].

Issue 2-2-7: A-MPR requirement for band 256 PC1.5 NB-IoT NTN UE

Agreement:

- Take the following as the starting point for the A-MPR for band 256 PC1.5 NB-IoT NTN UE with NS_24, depending on the final MPR value

Table 2-2-7.1 Additional Maximum Power Reduction (A-MPR) for category NB1 and NB2 UE for PC1.5

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_24 | 6.5B.4.4.3 | 256 | ≤ [8~8.5] |

- Interested companies can provide further evaluation in the next RAN4 meeting based on the assumptions in issue 2-2-6, including whether the above A-MPR is applicable for all frequencies of band 256 or a subset of frequencies of band 256.

  - The result from companies provided in the next RAN4 meeting will be taken into account when deciding the final value

Issue 2-2-8: A-MPR requirement for band 255 PC1.5 NB-IoT NTN UE

Agreement:

- A-MPR is not needed for band 255 PC1.5 NB-IoT NTN UE with NS_02N

- FFS on A-MPR evaluation for band 255 PC1.5 single Tx NB-IoT NTN UE with additional ETSI requirements associated with NS_10N and NS_14N

For band 256, if no more analysis provided, [8.5]dB A-MPR can be taken as baseline. There is also proposal that for different region of band 256, different A-MPR can be used. This is meaningful to control A-MPR values to a more moderate level. However, this would be heavily depended on test / simulation, which is a bit questionable in current tight schedule.

Proposal 3: Using [8.5] dB as initial value of A-MPR for band 256. Different A-MPR for different subset of frequencies of band 256 can be considered if sufficient evaluation is available.

For band 255, agreements already made for NS_02N, which was also defined for PC2/1. For evaluation of other NS values, it would be pretty much dependent on whether sufficient contributions can be provided.

Proposal 4: A-MPR for NS_10N and NS_14N would be dependent on further input.

Others

Another remaining issue is duty cycle related. The following item is as following:

Issue 2-2-3: UE RF front end assumptions and related accommodations

Way Foward:

- FFS on power handling challenges when NB-IoT NTN UE is scheduled for long duration e.g. 256ms in UL with high power e.g. +29dBm. (e.g. potential accommodations to UE maximum output power requirements)

- The timeline of the PC1.5 IoT NTN work shall not be impacted by this issue, even if no solution is agreed, no discussion after RAN4#118.

The origin of this item is from [5], in which the power reduction related to 25% duty cycle was mentioned and considered. The intention in that paper is assume that the UE might be in power class reduction mode due to a long duration UL, thus the +29dBm output power might be problematic.

However, whether FDD should be applied to the duty cycle based power backoff, is currently still in dispute in TN. There is a dedicated item in maintenance part to discuss this issue, and involving multiple thread for this issue seems not necessary.

It should be also noted that there is no need for duplexer for NB-Iot based Iot-NTN, thus one less factor need to be considered for PC1.5 1Tx.

Proposal 5: The duty cycle related power back off need not be discussed in this WI, since similar discussion is ongoing in maintenance part.

Conclusion

General

Proposal 1: If a TR is being considered, it is proposed to have some further discussion what would be the TR scope for WI of enhanced requirements for NR NTN and IoT NTN Phase 2.

RF requirements

Observation 1: The declared power class would not impact tests / simulations, as long as the same reference components are used / assumptions are the same.

Proposal 2: The starting point of MPR can be defined as maximum {0, PC1 MPR - 2dB}, minor adjustments can be considered based on new assumptions / evaluations.

Proposal 3: Using [8.5] dB as initial value of A-MPR for band 256. Different A-MPR for different subset of frequencies of band 256 can be considered if sufficient evaluation is available.

Proposal 4: A-MPR for NS_10N and NS_14N would be dependent on further input.

Others

Proposal 5: The duty cycle related power back off need not be discussed in this WI, since similar discussion is ongoing in maintenance part.

- References

[1] RP-252962, New WID on Enhanced requirements for NR NTN and IoT NTN Phase 2, RAN#109

[2] R4-2513564, Discussion on PC1.5 HPUE Requirements for Rel-20 NTN, RAN4#116bis

[3] R4-2513565, draft CR for Introduction of PC1.5 for NB-Iot based Iot-NTN, vivo, RAN4#116bis

[4] R4-2515157, WF for [116bis][318] NR_IoT_NTN_Ph2, Moderator(CHTTL), RAN4#116bis

[5] R4-2514383, NB-IoT NTN PC1.5 requirements, Qualcomm, RAN4#116bis