3GPP TSG-RAN WG4 Meeting #117			                        R4-2520757

Dallas, USA, Nov. 17-21, 2025

Agenda item:	7.7.3

Source: 	CMCC

Title: 	Discussion on coexistence for UAV

Document for:	Discussion

# 1. Introduction

At the last meeting, some progress was made in the discussion regarding high-power UAVs. However, there are still many differences in the aspect of coexistence analysis, such as the priority of analyzing coexistence scenarios, coexistence analysis parameters, etc. This document will present the viewpoints of the operators on the above issues.

# 2. Discussion

## 2.1 Co-existence Scenario

Last meeting we have reached the following agreement.

Issue 2-1-1: Coexistence scenarios - Agreements: - RAN4 to consider the following synchronized coexistence scenarios in Table 2.2.1-1 as the starting point. - Further down-selection for the simulation scenario is not precluded. Table 2.2.1-1: Co-existence scenarios between UAV and TN network Aggressor; Victim. No.; deployment scenario UL/DL; System BW duplex mode; deployment scenario UL/DL; System BW duplex mode; Simulation frequency; ACLR/ACS requirements. 1; TN UL; 100MHz TDD; UAV UL; 100MHz TDD; 4 GHz; UAV BS ACS. 2; UAV DL; 100MHz TDD; TN DL; 100MHz TDD; 4 GHz; UAV BS ACLR. 3; TN DL; 100MHz TDD; UAV DL; 100MHz TDD; 4 GHz; UAV UE ACS. 4; UAV UL; 100MHz TDD; TN UL; 100MHz TDD; 4 GHz; UAV UE ACLR. 5; TN UL; [20MHz] FDD; UAV UL; [20MHz] FDD; 2 GHz; UAV BS ACS. 6; UAV DL; [20MHz] FDD; TN DL; [20MHz] FDD; 2 GHz; UAV BS ACLR. 7; TN DL; [20MHz] FDD; UAV DL; [20MHz] FDD; 2 GHz; UAV UE ACS. 8; UAV UL; [20MHz] FDD; TN UL; [20MHz] FDD; 2 GHz; UAV UE ACLR. Issue 2-2-1: Deployment scenario - Agreements: - RAN4 needs to further discuss deployment scenarios for coexistence analysis - Option 1: UMa - Option 2: RMa - Option 3: UMa and RMa

In terms of coexistence scenarios, the last meeting listed 4 coexistence scenarios for TDD and FDD respectively. To further narrow down the co-existence scenario scope, some analyses are conducted on each of the 8 scenarios. By comparing with the existing interference in the UAV PC3 scenario, it was analyzed whether additional co-existence studies are necessary. Scenario 1 and 5 was the interference from ground UE to the UAV BS. Since a similar scenario would exist in the UAV PC3 scenario and the assumption in R20 was that BSs for UAV services were deployed on the ground by adjusting the tilt angle upwards, the interference received by the R20 UAV BS was smaller, so no additional co-existence study is needed in Rel-20.

Scenario 2 and 6 involves interference from the UAV BS to the ground UE. This interference still exists in the UAV PC3 scenario as well, so no additional coexistence research is required in this WID.

Observation 1: The assumption in the UAV UE antenna type and the increase of output power will not affect the interference in scenarios 1, 2, 5 and 6.

Proposal 1: For scenario 1, 2, 5 and 6, no additional coexistence study is needed in Rel-20.

There are some different understanding on the analysis of scenarios 3 and 7. Scenario 3 involves interference from the ground BS to the UAV UE. From our perspective, the study on the R20 UAV focuses on the impact of the increase in UAV transmission power on the ground BS. In addition, in the R18 research, the Rx requirements of the PC3 UAV have been defined. Therefore, considering these two points together, since the transmission power of the ground BS has not changed and the Rx requirements of the UAV have been specified, no additional research is required.

Observation 2: PC3 UAV Rx requirements has been specified in TS 38.101-1.

Proposal 2: Compared with previous studies, the transmission power of the ground BSs in this research has not been changed. For scenario 3 and 7, no additional coexistence study is needed in Rel-20.

Table 1: Co-existence scenarios between UAV and TN network

|  | Aggressor |  | Victim |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| No. | deployment scenario UL/DL | System BW duplex mode | deployment scenario UL/DL | System BW duplex mode | Simulation frequency | ACLR/ACS requirements | Note |
| 1 | TN UL | 100MHz TDD | UAV UL | 100MHz TDD | 4 GHz | UAV BS ACS |  |
| 2 | UAV DL | 100MHz TDD | TN DL | 100MHz TDD | 4 GHz | UAV BS ACLR |  |
| 3 | TN DL | 100MHz TDD | UAV DL | 100MHz TDD | 4 GHz | UAV UE ACS |  |
| 4 | UAV UL | 100MHz TDD | TN UL | 100MHz TDD | 4 GHz | UAV UE ACLR | High priority |
| 5 | TN UL | [20MHz] FDD | UAV UL | [20MHz] FDD | 2 GHz | UAV BS ACS |  |
| 6 | UAV DL | [20MHz] FDD | TN DL | [20MHz] FDD | 2 GHz | UAV BS ACLR |  |
| 7 | TN DL | [20MHz] FDD | UAV DL | [20MHz] FDD | 2 GHz | UAV UE ACS |  |
| 8 | UAV UL | [20MHz] FDD | TN UL | [20MHz] FDD | 2 GHz | UAV UE ACLR | High priority |

## 2.2 Deployment scenario and Network layout

Issue 2-2-1: Deployment scenario - Agreements: - RAN4 needs to further discuss deployment scenarios for coexistence analysis - Option 1: UMa - Option 2: RMa - Option 3: UMa and RMa Issue 2-2-2: Network layout - Agreements: - RAN4 needs to further discuss network layout used for coexistence analysis - Option 1: co-located - Option 2: co-located and non-collocated Figure 2.1.1-1: Illustration of co-located network layout Figure 2.1.1-2: Illustration of non-collocated network layout - Other network layouts such as only covering UAV routes are not precluded. - ISDUAV =X*ISDNR, FFS on X - ISD for UAV: 2-4km for UAV ISD

For the Deployment scenario, both the UMa and RMa deployment scenarios are of great significance for operators. In the UMa scenario, high-power UAV are mainly used for emergency rescue and cargo transportation. Similarly, the application of high-power unmanned aerial vehicles in the RMa scenario is also very important. Considering the significant difference in ISD between the two scenarios, from the perspective of operators, we suggest conducting some coexistence analysis for both scenarios.

Proposal 3: It is suggested to consider both the UMa and RMa scenarios.

For network deployment, from the perspective of the operator, building new sites for UAV BSs separately and adopting the co-located deployment can reduce the construction cost. Therefore, the co-located deployment is more in line with the actual situation. In addition, for the assumption of ISD, currently it is 2-4 kilometers. The larger station spacing when deploying UAV base stations makes the impact of co-located and non-co-located less significant. Therefore, it is recommended to choose the option 1 co-located scenario for the analysis. Regarding the ISD of UAV, in actual testing and deployment, the ISD of the UAV BS may be larger, even reaching 6 km. Therefore, it is recommended to consider setting the ISD to 6 km.

Proposal 4: Select the co-located scenario for use in the coexistence analysis.

Proposal 5: Consider setting the ISD to 6 km.

## 2.3 Co-existence Analysis

During last meeting, the point of discussion was whether the coexistence research should adopt simulation or link calculation. The main point was whether the link budget could reflect the superimposed interference from multiple UAV to the ground BS. A consensus was that a UAV cell is likely to have multiple UAVs, and these UAVs are randomly distributed horizontally and vertically in the air. The situation with significant interference would occur when the UAV is far from the UAV BS but very close to a ground base station. For a ground BS, multiple UAVs with maximum output power being close together is an extremely conner case. Therefore, a situation where a UAV is very close to a BS can be regarded as a typical case.

Observation 3: For a ground BS, multiple UAVs with maximum output power being close together is an extremely conner case.

Proposal 5: It is suggested to conduct coexistence analysis through link calculation.

In the Tdoc from the previous meeting, we conducted an analysis using the ACLR and ACS requirements of TN PC2 UE. The detailed analysis process can be found in document R4-2513745. For the case of UAV UE with maximum output power and very close to the BS, the analysis results are recorded in table 2. From the results, it can be observed that when the UAV terminal is 200 meters above the ground base station, there is still some interference margin for the ground.

Figure 3. Co-existence scenario for UAV UE with Ground BS2

Table 2. Interference from UAV UE to Ground BS2

| Frequency | 2.6GHz | 4.9GHz |
| --- | --- | --- |
| UAV Tx / EIRP (dBm) | 33 | 33 |
| Bandwidth (MHz) | 100 | 100 |
| UAV Tx power spectral density (dBm/MHz) | 13 | 13 |
| Ground BS interference threshold (dBm/MHz) | -110 | -110 |
| Ground BS antenna gain (dB) | 23 | 23 |
| Side lobe suppression (dB) | 30 | 30 |
| Distance (km) | 0.2 | 0.2 |
| PL (dB) | 86.8 | 92.3 |
| Clutter loss (dB) | 0 | 0 |
| Shadow fading loss (dB) | 0 | 0 |
| UAV ACLR (dB) | 31 | 31 |
| BS ACS (dB) | 46 | 46 |
| ACIR (dB) | 30.9 | 30.9 |
| Additional isolation required (dB) | -1.6 | -7.1 |

In addition to this, compared with TN UL->TN UL, UAV UE has max 6-7dB higher antenna gain/power gain calculated from deployment scenario while victim BS antenna gain towards UAV UE is at least 12-15dB less than that towards TN UE. besides, UAV UE number is much less than TN network. for TN network, one network has 30-50 TN UE but UAV can only support less than 10 UE to avoid crash. So in total, legacy PC3/PC2 TN requirements can be reused and no additional co-existence analysis is needed.

Proposal 6: no additional co-existence simulation is needed, and legacy PC3/PC2 TN UE requirements can be reused.

# 3. Conclusions

In this contribution, UAV simulation assumptions are discussed with following proposals:

Observation 1: The assumption in the UAV UE antenna type and the increase of output power will not affect the interference in scenarios 1, 2, 5 and 6.

Proposal 1: For scenario 1, 2, 5 and 6, no additional coexistence study is needed in Rel-20.

Observation 2: PC3 UAV Rx requirements has been specified in TS 38.101-1.

Proposal 2: Compared with previous studies, the transmission power of the ground BSs in this research has not been changed. For scenario 3 and 7, no additional coexistence study is needed in Rel-20.

Proposal 3: It is suggested to consider both the UMa and RMa scenarios.

Proposal 4: Select the co-located scenario for use in the coexistence analysis.

Proposal 5: Consider setting the ISD to 6 km.

Observation 3: For a ground BS, multiple UAVs with maximum output power being close together is an extremely conner case.

Proposal 5: It is suggested to conduct coexistence analysis through link calculation.

Proposal 6: no additional co-existence simulation is needed, and legacy PC3/PC2 TN UE requirements can be reused.

# 4. Reference

[1] R4-2515159, Way Forward for [116bis][319] NR_UAV, Qualcomm Incorporated, CMCC