# 3GPP TSG-RAN WG4 Meeting #117                               R4-2520770

# Dallas, US, 17th-21st Nov., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on RF requirements for Rel-20 HPUE

# Agenda Item:   7.1.4

# Document for:  Approval

- Introduction

In last meeting, RAN4 achieved some preliminary agreements on 4Tx HPUE with up to 32dBm maximum output power (MOP) ans PC1.5 2Tx HPUE for FDD bands as follows [1].

1.1 MPR/A-MPR assumptions Agreement: - Evaluate assuming following antenna isolation assumptions - For CPE, assuming both 10dB and 20dB, - For FWA, assuming 20dB. Come back once MPR results are available, to evaluate if it is possible to select a single CPE antenna isolation assumption based on the MPR results. 1.2 NS to be evaluated Agreement: - Evaluate A-MPR for NS_04 and NS_47 for n41. - Maximum output power Agreement: - 29 dBm is the maximum output power. - ACLR assumption Agreement: - 31 dB is the agreed ACLR value. - NS to be evaluated Agreement: - Agree to evaluate PC1.5 A-MPR for NS_03, NS_05, NS_48, and NS_49. - Encourage companies to check if NS_03U, NS_05U, and NS_100 are needed. - If needed, how to define UTRA ACLR for PC1.5 - REFSENS degradation Agreement: - Encourage companies to provide the general methodology for deriving the REFSENS degradation (RSD) in next meeting.

In this contribution, we provided some of our considerations on the HPUE with 4x26dBm PA configuration and PC1.5 FDD HPUE with 2x26dBm PA configuration.

- Discussion

## 2.1 MOP for HPUE with 4x26dBm PA configuration

In the WID, 4Tx with 4x26dBm PA configuration has been confirmed for CPE/FWA UE TDD single carrier with the MOP up to 32dBm[2]. During the discussion in RAN#109 meeting, companies had very different views on the MOP of 4Tx CPE/FWA UE with 4x26dBm PA configuration and also the power class. Some companies proposed that it is possible to achieve 32dBm MOP with 4x26dBm PA configuration considering full PA power capabilities. Although higher maximum transmission power is beneficial, a new power class, maybe PC1bis, needs to be introduced to avoid conflict with current PC1 with 31dBm MOP.

For power class, there already exists 4Tx PC1 NTN HPUE with 4x26dBm PA configuration and 31dBm MOP in Rel-19. ACLR requirement for PC1 NTN PHUE was specified to 34dB as a final compromise, which is 3dB lower than 37dB ACLR requirement for PC1 TN UE. To avoid inconsistent between TN UE and NTN UE, reuse PC1 for 4Tx HPUE is a good choice.

Observation 1: 4Tx PC1 NTN HPUE has been introduced in Rel-19 and 34dB ACLR was specified.

Besides, an additional power class with different MOP will have impact on RAN2 specification. Currently in RAN2 specification P-Max is up to 33dBm. If we define a new power class with 32dBm MOP, with widely recognized +2dB upper tolerance, then RAN2 needs to redefine P-Max.

Observation 2: A new power class with 32dBm maximum output power will impact current RAN2 specification.

For MOP, as is well known, UE is not allowed to always transmit with MOP because of power backoff caused by ACLR, SEM, IBE, EVM requirements as well as network indication. UE PHR depends on PCMAX defined in TS38.101-1/-2, where the configured maximum output power, PCMAX, is within a certain range. Then with reported PCMAX, NW will indicate transmitted power for UE. Moreover, with higher PA output power in each chain, more severe RIMD products can be foreseen for 4Tx with 4x26dBm PA configuration, which may further limit UE’s output power. This phenomenon has be pointed out and tested in Skyworks’ contribution on 4Tx NTN HPUE in Rel-19, where it was found that even under 34dB ACLR, 32dBm output power can not be achieved with same 4x26dBm PA configuration[3]. In this case, we hold a negative attitude towards whether 32dBm MOP is achievable.

Observation 3: UE is not allowed to always transmit with maximum output power because of power backoff and network configuration. 4Tx with 4x26dBm PA configuration will cause more serious reverse IMD compared to legacy UE, which may further limit UE’s output power.

Overall, we prefer to define 4Tx CPE/FWA UE with 4x26dBm PA configuration as PC1 UE, but we understand that additional 1dB is beneficial for UL coverage. If 32dBm output power is feasible, RAN4 can further study how to deal with this potential 1dB more output power than PC1 and how to reflect it in other aspects rather than MOP and tolerance, such as power boosting or something else.

Proposal 1: Define 4Tx CPE/FWA UE with 4x26dBm PA configuration as PC1 UE with 31dB maximum output power and +2/-3dB tolerance.

Proposal 2 : If 32dBm output power is feasible, treat the potential 1dB more output power than PC1 as power boosting.

## 2.2 UL MIMO and Tx Diversity for 4Tx CPE/FWA UE

It has been confirmed in the objectives that both 4x4 UL MIMO and Tx Diversity are supported for the maximum output power up to 32dBm operating in example bands n41, n77/n78, n79, n104. The Tx RF requirements related to UL MIMO and Tx Diversity with multiple transmit antenna connectors up to a maximum of four in TS38.101-1 are summarized in Table 1, where some of the requirements apply to the sum from all UE transmit antenna connectors while others are not.

Table 1  Tx RF requirements for UL MIMO and Tx Diversity in TS38.101-1

| Tx RF requirements |  | For UL MIMO | For Tx Diversity |
| --- | --- | --- | --- |
| - 6.2D/G Transmit power for UL MIMO/Tx Diversity | - - UE maximum output power - - MPR/A-MPR - - Configured transmitted power | Apply to the sum of the mean power from all UE transmit antenna connectors, and is related to UE power class | Apply to the sum of the mean power from all UE transmit antenna connectors, and is related to UE power class |
| - 6.3D/G Output power dynamic for UL MIMO/Tx Diversity - 6.4D/G Transmit signal quality for UL MIMO/Tx Diversity - 6.5D/G Output RF spectrum emissions for UL MIMO/Tx Diversity | - - Minimum output power - - Power control - - Occupied bandwidth - - Out of band emission - - Spurious emissions | Apply to the sum of the mean power from all UE transmit antenna connectors, and is UE power class agnostic | Apply to the sum of the mean power from all UE transmit antenna connectors, and is UE power class agnostic |
|  | - - Transmit OFF power - - Transmit ON/OFF time mask - - In-band emissions - - Transmit intermodulation | Apply at each transmit antenna connector | Apply at each transmit antenna connector |
|  | - - Frequency error - - EVM and EVM equalizer spectrum flatness - - Carrier leakage | Apply per layer |  |
|  | - - Time alignment error - - Coherent UL MIMO | Difference between any two different transmit antenna connectors |  |

Based on the summary in Table 1, we can find that only Maximum output power, MPR/A-MPR and Configured transmitted power apply to sum of the mean power from all UE transmit antenna connectors, and are related to UE power class. Other Tx RF requirements can keep unchanged for HPUE with up to 32dBm MOP thanks to the WI on 4Tx PC1.5 UE.

Proposal 3: Output power dynamic, Transmit signal quality and Output RF spectrum emissions for UL MIMO and Tx diversity can keep unchanged for HPUE with up to 32dBm MOP.

MPR/A-MPR requirements should be evaluated for CPE/FWA UE with 4x26dBm PA configuration once the power class is decided, considering the more severe RIMD products caused by 4Tx, which would make it difficult for a UE to meet OOB emission requirements. Consequently, larger MPR/A-MPR requirements may be needed.

Proposal 4: MPR/A-MPR requirements for UL MIMO and Tx Diversity can be evaluated once the power class is decided.

ULFPTx feature was introduced in Rel-16 as UL MIMO enhancement for non-coherent UE and partial coherent UE and there are three ULFPTx modes defined in total. When introducing 4Tx PC1.5 in Rel-18, ULFPTx Mode 1 was considered in Phase 1 with 4x23dBm PA configuration and was extended to Mode 2 in Phase 2 considering PC1.5 UE with 2x23dBm+2x26dBm PA configuration and 4x26dBm PA configuration. Thus PUSCH configurations 4Tx ports for both 1 layer and 2 layers for both Mode-1 and Mode-2 were added to TS38.101-1 [4].

For HPUE with 4x26dBm PA configuration to achieve 31/32dBm MOP, ULFPTx Mode-0 (Mode-full power) can be precluded considering Mode-0 is used for the UE with full-rated PA on each Tx chain, namely 31/32dBm PA.

For ULFPTx Mode-1, with 1-layer TPMI=13 and 2-layer TPMI=6 as shown in the table above, 4x26Bm PA configuration could enable ULFPTx.

For ULFPTx Mode-2, feature ul-FullPwrMode2-TPMIGroup-r16 would indicate the UE supported TPMI group(s) which delivers full power. 7 TPMI groups G0-G6 have been predefined in TS38.306 as shown in the table below. UE would report its supported TPMI group in ul-FullPwrMode2-TPMIGroup-r16 and gNB could have a basic assumption of UE’s PA architecture according to this TPMI group information. However, for 4x26dBm PA configuration, none of the TPMI groups can support full-power transmission. Namely, with partial PAs it is impossible to achieve 31/32dBm MOP, unless other improved PA configurations are introduced in the future, for example, 2x26dBm+2x29dBm PA configuration or 4x29dBm PA configuration. Therefore, ULFPTx Mode-2 can be further considered if other PA configurations are introduced in the future.

Proposal 5: 4Tx HPUE with 4x26dBm PA configuration could indicate ULFPTx Mode-1 to achieve 31/32dBm MOP. ULFPTx Mode-2 can be further considered if other PA configurations are introduced in the future.

For Rx requirements, there should be no changes thanks to the work on 4Tx PC1.5 UE.

- Proposal 6: There is no impact on Rx requirements when introducing HPUE with up to 32dBm MOP.

## 2.3 RF requirements for PC1.5 2Tx CPE/FWA for FDD bands

In current TS38.101-1, PC1.5 with 2Tx for TDD bands, such as n41, n77/n78, n79, and related 2x2 UL MIMO and TxD requirements are specified. These requirements are agnostic of NR bands. If RAN4 would like to introduce new FDD bands for PC1.5 UE with 2Tx, some legacy requirements should also apply. Except maximum output power and ACLR, MPR requirements in current specification should also apply to FDD bands.

Proposal 7: The MPR requirements of PC1.5 2Tx requirements for TDD bands can be applied to FDD bands for PC1.5 UE with 2Tx.

Another difference between PC1.5 2Tx FDD and PC1.5 2Tx TDD is that RSD requirements should be defined for 2Tx FDD bands due to the self-interference impacts on Rx from its own Tx. There are two example bands, n1 and n25, to be defined in this WID. In current TS38.101-1, the RSD requirements from PC3 to PC2 for FDD bands n1 and n25 for UE supporting Tx Diversity are defined as:

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 7 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  | 0 |  | 0 | 0 | 0 | 0 | 0 | - | 0 | 0 | 0 |
| n25 |  | 1.5 |  | 1.5 | 1.6 | 1.6 | 1.7 | 6.0 | 6.2 | 6.7 | 7.1 |  |

It can be found that 0dB RSD is defined for n1 due to large Tx-Rx carrier centre frequency separation. Therefore, for PC1.5 2Tx UE in band n1, it could be foreseen that the RSD=0dB from PC3 to PC1.5 for UE supporting Tx Diversity in band n1.

Proposal 8: It is proposed that RSD=0dB from PC3 to PC1.5 for PC1.5 UE supporting Tx Diversity in band n1.

For band n25, we evaluate the RSD from PC3 to PC1.5 for PC1.5 UE supporting Tx Diversity as shown in Table 2.

Proposal 9: To consider the the RSD from PC3 to PC1.5 for PC1.5 UE supporting Tx Diversity as shown in Table 2.

Table 2  RSD from PC3 to PC1.5 for PC1.5 UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 7 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 |  | 2.4 |  | 2.5 | 2.5 | 2.6 | 2.8 | 8.8 | 9.1 | 9.8 | 10.2 |  |

In addition to that, with our experience in the past, there will be more new FDD bands introduced in the spec to support PC1.5 2Tx. It was a time-consuming work in terms of the PC2 2Tx FDD discussion in Rel-18/19. Thus to learn a lesson from PC2 2Tx FDD, we think it would be better to come up with a general approach to defined the RSD requirements from PC3 to PC1.5. Then some evaluations are needed to compare 2x26dBm PA configuration and 1x23dBm PA configuration.

Proposal 10: It is proposed to study a general approach to defined the RSD requirements from PC3 to PC1.5 for all PC1.5 2Tx FDD bands.

- Conclusion

Observations:

Observation 1: 4Tx PC1 NTN HPUE has been introduced in Rel-19 and 34dB ACLR was specified.

Observation 2: A new power class with 32dBm maximum output power will impact current RAN2 specification.

Observation 3: UE is not allowed to always transmit with maximum output power because of power backoff and network configuration. 4Tx with 4x26dBm PA configuration will cause more serious reverse IMD compared to legacy UE, which may further limit UE’s output power.

Proposals:

Proposal 1: Define 4Tx CPE/FWA UE with 4x26dBm PA configuration as PC1 UE with 31dB maximum output power and +2/-3dB tolerance.

Proposal 2 : If 32dBm output power is feasible, treat the potential 1dB more output power as power boosting.

Proposal 3: Output power dynamic, Transmit signal quality and Output RF spectrum emissions for UL MIMO and Tx diversity can keep unchanged for HPUE with up to 32dBm MOP.

Proposal 4: MPR/A-MPR requirements for UL MIMO and Tx Diversity can be evaluated once the power class is decided.

Proposal 5: 4Tx HPUE with 4x26dBm PA configuration could indicate ULFPTx Mode-1 to achieve 31/32dBm MOP. ULFPTx Mode-2 can be further considered if other PA configurations are introduced in the future.

- Proposal 6: There is no impact on Rx requirements when introducing HPUE with up to 32dBm MOP.

Proposal 7: The MPR requirements of PC1.5 2Tx requirements for TDD bands can be applied to FDD bands for PC1.5 UE with 2Tx.

Proposal 8: It is proposed that RSD=0dB from PC3 to PC1.5 for PC1.5 UE supporting Tx Diversity in band n1.

Proposal 9: To consider the the RSD from PC3 to PC1.5 for PC1.5 UE supporting Tx Diversity as shown in Table 2.

Table 2  RSD from PC3 to PC1.5 for PC1.5 UE supporting Tx Diversity

| Operating Band | 3 MHz (dB) | 5 MHz (dB) | 7 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n25 |  | 2.4 |  | 2.5 | 2.5 | 2.6 | 2.8 | 8.8 | 9.1 | 9.8 | 10.2 |  |

Proposal 10: It is proposed to study a general approach to defined the RSD requirements from PC3 to PC1.5 for all PC1.5 2Tx FDD bands.

- Reference

- R4-2515087 Way Forward for [116bis][316] NR_UE_RF_Ph5, RAn4#116bis, Prague, CZ, Oct. 2025.

- RP-252962 New WID: UE RF enhancements for NR FR1, Phase 5, RAN#109, Beijing, CN, Sept. 2025.

- R4-2511320 MPR and antenna isolation evaluation for 4Tx PC1, RAN4#116, Skyworks, Bengaluru, India, Aug. 2025.

- R4-2321787 draft Big CR to TS 38.101-1: 4Tx requirements, RAN4#109, Chicago, USA, Nov. 2023.