# 3GPP TSG-RAN WG4 Meeting #117                               R4-2520771

# Dallas, US, 17th-21st Nov., 2025

# Source:        ZTE Corporation, Sanechips

# Title:           Discussion on 6MHz BS RF requirements

# Agenda Item:   7.1.5.1

# Document for:  Approval

- Introduction

In last meeting, RAN4 had achieved a lot of agreements on BS RF requirements for 6MHz CBW and a few issues were under alignment in this meeting as follows so that we can prepare the draft CRs, including ACS, Narrowband blocking, Receiver intermodulation and ICS[1]. In this contribution, we provide our views on these BS RF requirements.

- Discussion

ACS

For ACS, the main issue is to derive the interfering signal centre frequency offset from the lower/upper Base Station RF bandwidth edge or sub-block edge inside a sub-block gap, where the configuration of interfering signal is 5MHz CBW and 15kHz SCS with full RB allocation. The center frequency of interfering signal has naturally 2.5MHz offset. Besides, an additional offset is needed to avoid orthogonality between sub-carriers of wanted signal and interfering signal. The calculation formula is shown in Section 7.4.1 of TR38.817-02 as follows:

The ACS requirement may be specified with an offset calculated for the exact position of the interfering signal such that the sub-carrier grid of the interferer is ½ of the sub-carrier spacing offset from the sub-carrier grid of the wanted signal. Furthermore, the sub-carrier spacing of the interferer and the wanted signal could be set the same, such that the offset is achieved for all sub-carriers of the interferer and wanted signal. However, since only the first few sub-carriers tend to impact ACS performance it is preferable to apply 15 kHz sub-carrier spacing for the interferer regardless of the wanted signal sub-carrier spacing, since the 15 kHz sub-carrier spacing has the highest spectrum utilization. Testing with the minimum distance from the active PRBs of the interferer and the wanted signal bandwidth edge is seen as more important than aligning the sub-carrier spacings. The required increment in frequency offset can be calculated by applying the following equation: where and is the nominal frequency offset (which is equal to half of the sum of the channel bandwidth of the wanted signal and the channel bandwidth of the interfering signal) and is the calculated offset increment.

Based on the formula above, we can calculate that

F_Interferer = (6MHz + 5MHz)/2 = 5.5MHz = 5500kHz

x = ceil( 5500kHz / (15kHz/2) ) = 734

Delta_F_Interferer = 15kHz/2 * ( 734 + (1+734)mod2 ) - 5500kHz = 7.5kHz*735 - 5500kHz = 12.5kHz

Then the interfering signal centre frequency offset for ACS is:

5MHz/2 + 12.5kHz = 2.5125MHz

Proposal 1: The Interfering signal centre frequency offset in ACS requirements for 6MHz CBW is ±2.5125 (MHz).

Narrowband blocking

For narrowband blocking, the main issue is to derive the interfering RB centre frequency offset to the lower/upper Base Station RF Bandwidth edge or sub-block edge inside a sub-block gap, where the configuration of interfering signal is 5MHz CBW and 15kHz SCS with 1 RB allocation. The center frequency of the 1RB interfering signal refers to the frequency location between the two central sub-carriers, whose offset from the carrier edge is

guard band + half SCS + half RB + m*180 = 340kHz + m*180kHz

m = 0, 1, 2, 3, 4, 9, 14, 19, 24

Same as that in ACS, to avoid orthogonality between sub-carriers of wanted signal and interfering signal, an additional 12.5kHz offset is needed. Moreover, the frequency offset between the wanted signal and the interfering signal is further increased by 7.5 kHz (½ sub-carrier) considering the centre of the interfering RB refers to the frequency location between the two central sub-carriers. Then the interfering signal centre frequency offset for narrowband blocking is:

340kHz + m*180kHz + 12.5kHz + 7.5kHz = 360kHz + m*180kHz

m = 0, 1, 2, 3, 4, 9, 14, 19, 24

Proposal 2: The Interfering signal centre frequency offset in NBB requirements for 6MHz CBW is ±(360 + m*180) (kHz), m = 0, 1, 2, 3, 4, 9, 14, 19, 24.

Receiver intermodulation

For the interfering signal center frequency offset in intermodulation requirement, the calculation formulas are introduced in TR38.817-02:

For the general receiver intermodulation requirement, the frequency offset of the modulated interfering signal centre frequency from the lower/upper Base Station RF Bandwidth edge is calculated by the following equation: ∆fMod=15+BWinterferer2 Where BWinterferer = Channel bandwidth of the modulated interfering signal (in MHz) The modulated interfering signal resource blocks should be placed adjacent to the transmission bandwidth configuration edge which is closer to the Base Station RF Bandwidth edge. Then the frequency offset of the CW interfering signal from the lower/upper Base Station RF Bandwidth edge is calculated by the following equation to ensure the intermodulation products fall (almost) on the edge resource blocks of the operating BS channel bandwidth at least with the lowest SCS defined for the wanted signal channel bandwidth: ∆fCW=7.5−Gwanted−Ginterferer2 Where Gwanted = Minimum guardband with the lowest SCS defined for the wanted signal channel bandwidth (in MHz) Ginterferer = Minimum guardband with the lowest SCS defined for the interfering signal channel bandwidth (in MHz)

Then interfering signal center frequency offset can be calculated as:

Delta_f_CW = 7.5MHz - (292.5kHz - 242.5kHz)/2 = 7.475MHz

Proposal 3: The interfering signal centre frequency offset in intermodulation requirement could be ±7.475 (MHz) for CW.

For the interfering RB center frequency offset in narrowband intermodulation requirement, the calculation formulas are introduced in TR38.817-02:

For the narrowband receiver intermodulation requirement, the frequency offset of the modulated interfering RB centre frequency from the lower/upper Base Station RF Bandwidth edge is calculated by the following equation: ∆fMod=700+Gwanted+SCSwanted2+RBinterferer2 Where Gwanted = Minimum guardband with the highest SCS defined for the wanted signal channel bandwidth (in kHz) SCSwanted = The highest SCS defined for the wanted signal channel bandwidth (in kHz) RBinterferer = RB size with the highest SCS defined for the modulated interfering signal channel bandwidth (in kHz) Then the frequency offset of the CW interfering signal from the lower/upper Base Station RF Bandwidth edge is calculated by the following equation to ensure the intermodulation products fall (almost) on the edge resource blocks of the operating BS channel bandwidth with the highest SCS defined for the wanted signal channel bandwidth: ∆fCW=∆f'Mod−SCSwanted/2−RBinterferer/2−Gwanted2

Then interfering RB center frequency offset can be calculated as:

Delta_f_Mod = 700kHz + 292.5kHz + 15kHz/2 + 180kHz/2 = 1090kHz

which should be adjusted to 1240kHz based on the RB offset. The offset of CW is:

Delta_f_CW = (1240kHz - 15kHz/2 - 180kHz/2 - 292.5kHz)/2 = 425kHz

Proposal 4: The interfering RB centre frequency offset in narrowband intermodulation requirement could be ±425 (kHz) for CW and ±1240 (kHz) for 5MHz interfering signal.

ICS

When discussing ICS of 7MHz CBW in Rel-19, RAN4 also had divergence on the number of interfering signal. For 7MHz, NRB is 35, wanted signal occupies 15 RBs, then the RB number of interfering signal was defined as 15 finally. In our understanding, the wanted signal and interfering signal should be located at two sides of centre frequency with as equivalent RB numbers as possible. Considering that for 6MHz, NRB is 30, wanted signal occupies 15 RBs, then the RB number of interfering signal can also be 15.

Proposal 5: Propose the type of interfering signal for ICS of 6MHz CBW is same as 7MHz CBW, i.e., DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs.

- Conclusion

Proposal 1: The Interfering signal centre frequency offset in ACS requirements for 6MHz CBW is ±2.5125 (MHz).

Proposal 2: The Interfering signal centre frequency offset in NBB requirements for 6MHz CBW is ±(360 + m*180) (kHz), m = 0, 1, 2, 3, 4, 9, 14, 19, 24.

Proposal 3: The interfering signal centre frequency offset in intermodulation requirement could be ±7.475 (MHz) for CW.

Proposal 4: The interfering RB centre frequency offset in narrowband intermodulation requirement could be ±425 (kHz) for CW and ±1240 (kHz) for 5MHz interfering signal.

Proposal 5: Propose the type of interfering signal for ICS of 6MHz CBW is same as 7MHz CBW, i.e., DFT-s-OFDM NR signal, 15 kHz SCS, 15 RBs.

- Reference

- R4-2515084 Way Forward for [116bis][329] NR_UE_RF_Ph5_Part2 -- 6 MHz channel, Samsung, Nokia, Ericsson, Skyworks, Apple, Huawei, CATT, ZTE, Qualcomm, RAN4#116bis, Prague, CZ, Oct. 2025.