3GPP TSG-RAN WG4 Meeting #117							   R4-2520815

Dallas, USA, 17th – 21th Nov, 2025

Agenda Item: 	8.2.5

Source: 	MediaTek Inc.

Title: 	Considerations on scalable and forward compatible design for diverse device types

Document for: 	Discussion

# 1	Introduction

Following the 6G SI approved in RAN#108 and revised at RAN#109 [1], the first 6G discussion started from RAN4#116bis meeting. During RAN4#116bis, a WF for 6GR system parameter [2] was agreed, classifying topics and directions for further study. In this paper, we provide our views on number of Tx/Rx in relation with the kind of device that is targeted.

# 2	Number of Tx/Rx and Device type

In the last RAN 4 Meeting, RAN4 approved WF on device types in 6G system parameters [2] as follow

- Agreement for Device Types

  - The following aspects could be further studied

    - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range.

      - Study on the NW impact should also be considered

    - Study how to better support variety devices from RAN4 perspective

      - E.g. differentiate devices, use cases, features

    - Other aspects are not precluded

  - Collaborate on the framework with other WGs:

    - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues

Based on the 6GR SI, the main objectives of 6G are to support both existing and new services along with the consideration on scalable and forward compatible design for diverse device types.

RAN#109 reached the following agreement regarding device type i.e. that no further WG discussion take place until further notice from RAN plenary:

Proposal 3: To investigate further: - Motivations/justifications behind the proposed diverse device types, which should be a limited set - Whether/how to have one or more device types for eMBB or 6G IoT - Whether/how to have other device types for, e.g., XR/immersive experiences, FWA, VUE, wearables/RedCap, sensing, NTN-specific, AI agents, collaborative robots, etc. - Whether/how to explicitly standardize device types - Ensuring forward compatibility - Minimizing/avoiding potential market fragmentation Note: the terminology “device type” is subject to further discussion and possible refinement. Proposal 4: In terms of diverse device types, study further: - Possible parameters/factors, e.g.: - Number of Tx antennas/chains - Number of Rx antennas/chains - Power classes - Maximum UE bandwidth (DL/UL) - Peak data rate (DL/UL) - Maximum MIMO layers (DL/UL) - Duplex mode - Max modulation order (DL/UL) - CA/spectrum aggregation (DL/UL) - UE processing capabilities - Coverage - Energy efficiency - Mobility/speed - Sensing - AI Note: some of the above parameters/factors may be related with form factor Note: aim to have a focused/limited set of parameters/factors for a device type - The value(s) for the identified parameters for a device type Conclusion: proposal 1 is endorsed, proposal 3 & 4 are endorsed for RAN only (no WG discussion) in # 109 Plenary Meeting report

Pending RAN plenary progress on the above discussion, it is a clear objective of the 6GR Study to aim for a “common 6G Radio design, which meets mobile broadband service requirements as high priority, to also meet vertical needs”. Correspondingly, handheld UEs such as smartphones and non-handheld UEs such as CPE/FWA are priority cases that should be considered as baseline in early 6G stage.

Some MNOs have also requested that low-end IoT be considered early on (ongoing discussions in RAN1), which, with consideration of a scalable and forward compatible design, prompts identifying a common minimum set of functionalities, a technology Nucleus, for all devices. Identifying the “bottom-end” for 6GR technology is poised to help prevent any future technology fragmentation. In view of this, it is therefore important to also consider the lowest boundary for 6GR devices in early 6G stage.

Furthermore, a long debate did take place in 5G-Advanced to define “2Rx XR” device. While the exact definition thereof is debatable, it is important to recognize first that form-factor limitations may restrict a device from supporting all capabilities of the above “eMBB” baseline – and second that a glasses form-factor will be important for 6G.

But even if we have a very comprehensive discussion on which device type to be introduced in 6G, it is impossible to predict the future. Therefore, a forward compatible framework is very important. 3GPP spec needs to be flexible to accommodate any new market needs.

Proposal 1: RAN4 should consider the handheld UEs (e.g., smart phone) and non-handheld UEs (e.g. CPE/FWA) as eMBB baseline. RAN4 should also consider low-end IoT (i.e. bottom-end) as well as wearable (akin to 2Rx XR) early on. This allows RAN4 to proceed with evaluation/analysis around example potential typical 6GR scenarios and devices.

Proposal 2: For evaluation purpose, RAN4 can consider certain assumptions of RF parameters (# of Tx/Rx, power class, CBW ... ) with the understanding that these do not imply any decision on requirements to device type.

For information perspective, we can share some example potential typical devices for 6G in Table 1 as follows.

Table 1. Example of potential typical 6G devices

| Parameter list | Non handheld (1) | Handheld (2) | Wearable (3) | Low-end IoT |
| --- | --- | --- | --- | --- |
| TX/Rx capability antenna | - 7GHz: 4T/8R - 3~4.9GHz: 2T/4-8R - 1~3GHz: 2T/4R - Sub 1GHz: 2T/2R | - 7GHz: 4T/4R - 3~4.9GHz: 2T/4R - 1~3GHz: 2T/4R - Sub 1GHz: 1T/2R | - 1~7GHz: 1T/2R - Sub 1GHz: 1T/1R | 1T/1R |
| DL MIMO layer | Up to 8 layer (depending on FR) | Up to 4 layer (depending on FR) | Up to 2 layer | 1 layer |
| RF BW (max) | 200 MHz | 200 MHz | 20 MHz | 20 MHz |
| SCS | FDD: 15kHz TDD: 30kHz | FDD: 15kHz TDD: 30kHz | FDD: 15kHz TDD: 30kHz | FDD: 15kHz TDD: 30kHz |
| FFT size | - 1~7GHz(TDD): 8K FFT - 1~3GHz (FDD): 8K FFT - sub 1GHz (FDD):4K FFT |  |  |  |
| Default output power | 26 dBm [or 29 dBm] | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm |
| DL max modulation order | [1024] QAM | [1024] QAM | 64 QAM | 64 QAM |
| UL max modulation order | 256QAM | 256QAM | 64 QAM | 64 QAM |
| NOTE 1: e.g. CPE/FWA NOTE 2: e.g. Smartphone NOTE 3: e.g. AR glasses |  |  |  |  |

# 3	Summary

In this paper, we provide our views on 6G system parameters with the following observations and proposals for number of Tx/Rx and device type as follow

Proposal 1: RAN4 should consider the handheld UEs (e.g., smart phone) and non-handheld UEs (e.g. CPE/FWA) as eMBB baseline. RAN4 should also consider low-end IoT (i.e. bottom-end) as well as wearable (akin to 2Rx XR) early on. This allows RAN4 to proceed with evaluation/analysis around example potential typical 6GR scenarios and devices.

Proposal 2: For evaluation purpose, RAN4 can consider certain assumptions of RF parameters (# of Tx/Rx, power class, CBW ... ) with the understanding that these do not imply any decision on requirements to device type.

For information perspective, we can share some example potential typical devices for 6G in Table 1 as follows.

Table 1. Example of potential typical 6G devices

| Parameter list | Non handheld (1) | Handheld (2) | Wearable (3) | Low-end IoT |
| --- | --- | --- | --- | --- |
| TX/Rx capability antenna | - 7GHz: 4T/8R - 3~4.9GHz: 2T/4-8R - 1~3GHz: 2T/4R - Sub 1GHz: 2T/2R | - 7GHz: 4T/4R - 3~4.9GHz: 2T/4R - 1~3GHz: 2T/4R - Sub 1GHz: 1T/2R | - 1~7GHz: 1T/2R - Sub 1GHz: 1T/1R | 1T/1R |
| DL MIMO layer | Up to 8 layer (depending on FR) | Up to 4 layer (depending on FR) | Up to 2 layer | 1 layer |
| RF BW (max) | 200 MHz | 200 MHz | 20 MHz | 20 MHz |
| SCS | FDD: 15kHz TDD: 30kHz | FDD: 15kHz TDD: 30kHz | FDD: 15kHz TDD: 30kHz | FDD: 15kHz TDD: 30kHz |
| FFT size | - 1~7GHz(TDD): 8K FFT - 1~3GHz (FDD): 8K FFT - sub 1GHz (FDD):4K FFT |  |  |  |
| Default output power | 26 dBm [or 29 dBm] | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm | FDD 23 dBm, TDD 26 dBm |
| DL max modulation order | [1024] QAM | [1024] QAM | 64 QAM | 64 QAM |
| UL max modulation order | 256QAM | 256QAM | 64 QAM | 64 QAM |
| NOTE 1: e.g. CPE/FWA NOTE 2: e.g. Smartphone NOTE 3: e.g. AR glasses |  |  |  |  |

# References

- [1] RP-252912, Revised Study on 6G Radio, NTT DOCOMO, RAN#109, Sep 2025

- [2] R4-2514641, WF for 6GR system parameter, Feature lead (Huawei, HiSilicon)

- [3] draft_MeetingReport_RAN_109_250918_eom, MCC, RAN#109, Sep 2025