3GPP TSG-RAN4 Meeting #117	R4-2520821

# Dallas, USA, November 17th – 21st, 2025

Source:	LG Electronics

Title: 	(6G system parameters) Device types

Agenda Item:	8.2.5

Document for:	Discussion

Introduction

RAN4 made WF [1] for following 6G system parameters in RAN4#116bis.

- Waveform

  - Framework study for waveform

  - PA model

- Modulation

- Channel bandwidth (CBW)

  - Max CBW

  - Min CBW

  - FFT size

  - Numerology

  - Spectrum utilization

  - Asymmetric CBWs

- Channel arrangement

  - Channel raster

  - Sync raster

  - Channel spacing

- Irregular channel bandwidth

- Number of Tx and Rx

- Device types

We would like to provide our views on 6G device types including number of Tx and Rx.

Discussion

Device types

RAN4 Agreement

RAN4#116bis Device types - The following aspects could be further studied - Investigate detailed RF/BB implementation feasibility and constraints related to different devices assumption, for example, size/form factors and use cases (e.g., IoT, Wearable, Smartphone, FWA devices). These should include concrete assumptions, e.g. number of antennas, CBW, power class, and supported modulation per frequency range. - Study on the NW impact should also be considered - Study how to better support variety devices from RAN4 perspective - E.g. differentiate devices, use cases, features - Other aspects are not precluded - Collaborate on the framework with other WGs: - Provide necessary inputs to RAN/other WGs with RAN4's study outcome for the above-mentioned issues # of Tx and Rx - Study the framework of spec impact with different number of Tx/Rx - Evaluate number of Tx/Rx from both a performance and implementation complexity perspective. - E.g. device size (e.g. foldable smartphone)/form factors constraints, antenna design - Co-ordinate with RAN and RAN1 for number of Tx/Rx

6G UE types is under discussion in RAN. Table 2-1 shows UE types, antenna number, and Tx power.

Table 2-1 : 6G UE type under discussion in RAN

|  | eMBB | IoT | FWA | Automotive |
| --- | --- | --- | --- | --- |
| Antennas(Tx/Rx) | 4 / 8 | 1 / 1 | 8 / 8 | 1 / 2 |
| Tx power | Normal | Low | High | High |

RAN4 assumed example device types, IoT, Wearable, Smartphone, and FWA in last RAN4 meeting,

For the example device types, key factor of feasibility is power & thermal, form factor & antenna integration, and use cases.

- Power & Thermal are major constraints for IoT and Wearables, driving low complexity RF/BB design.

- Form Factor & Antenna Integration are the primary drivers for all devices, limiting the number of antennas in smaller devices and enabling large arrays in FWA/Smartphones.

- Use Case dictates required data rates, which in turn defines necessary CBW and modulation orders, directly impacting the complexity and cost of the RF and BB chipsets.

Considering these factors, RF/BB implementation feasibility and constraints by Device Type can be summarized as Table 2-2.

Table 2-2 : RF/BB implementation feasibility and constraints by Device Type

| Feature | IoT (Narrowband) | Wearable (Smartwatch) | Smartphone (High-end) | FWA |
| --- | --- | --- | --- | --- |
| Size/Form Factor | Very Small | Small | Medium | Large |
| Use Case | Low data rate, long battery life, wide coverage | Medium data rate, calls, notifications, health monitoring | High data rate, mobility, multi-media | Very high data rate, stable connection, replace wireline broadband |
| Frequency Range | U1GHz | U6GHz | U6GHz, Around 7GHz, Around 15GHz, FR2-1 | U6GHz, Around 7GHz, Around 15GHz, FR2-1 |
| Antennas (TX/RX) | 1T1R, 1T2R for diversity | 2T2R | 4T4R (U6GHz, Around 7GHz), 8T8R (Around 15GHz, FR2-1) | 4T4R (U6GHz, Around 7GHz), 8T8R (Around 15GHz, FR2-1) |
| CBW | Narrow (200kHz) | Up to 20 MHz | Up to 90 MHz (U6GHz(FDD)), Up to 200 MHz (U6GHz(TDD), Around 7GHz), Up to 400 MHz (Around 15GHz, FR2-1) | Up to 90 MHz (U6GHz(FDD)), Up to 200 MHz (U6GHz(TDD), Around 7GHz), Up to 400 MHz (Around 15GHz, FR2-1) |
| Power Class (PC) | PC3 | PC3/PC2 | PC3/PC2/PC1.5/PC1 | PC3/PC2/PC1.5/PC1 |
| Modulation Support (DL) | Up to 64QAM | Up to 256QAM | Up to [1024]QAM | Up to [4096]QAM |
| Modulation Support (UL) | Up to 64QAM | Up to 64QAM | Up to [1024]QAM | Up to [1024]QAM |
| Duplex Mode | HD-FDD | HD-FDD/FDD/TDD | FDD/TDD | FDD/TDD |

Proposal 1: Consider RF/BB implementation feasibility and constraints in Table 2-2 for IoT, Wearable, Smartphone, and FWA.

Conclusions

This document discusses 6G device types and provides our view.

Proposal 1: Consider RF/BB implementation feasibility and constraints in Table 2-2 for IoT, Wearable, Smartphone, and FWA.

Table 2-2 : RF/BB implementation feasibility and constraints by Device Type

| Feature | IoT (Narrowband) | Wearable (Smartwatch) | Smartphone (High-end) | FWA |
| --- | --- | --- | --- | --- |
| Size/Form Factor | Very Small | Small | Medium | Large |
| Use Case | Low data rate, long battery life, wide coverage | Medium data rate, calls, notifications, health monitoring | High data rate, mobility, multi-media | Very high data rate, stable connection, replace wireline broadband |
| Frequency Range | U1GHz | U6GHz | U6GHz, Around 7GHz, Around 15GHz, FR2-1 | U6GHz, Around 7GHz, Around 15GHz, FR2-1 |
| Antennas (TX/RX) | 1T1R, 1T2R for diversity | 2T2R | 4T4R (U6GHz, Around 7GHz), 8T8R (Around 15GHz, FR2-1) | 4T4R (U6GHz, Around 7GHz), 8T8R (Around 15GHz, FR2-1) |
| CBW | Narrow (200kHz) | Up to 20 MHz | Up to 90 MHz (U6GHz(FDD)), Up to 200 MHz (U6GHz(TDD), Around 7GHz), Up to 400 MHz (Around 15GHz, FR2-1) | Up to 90 MHz (U6GHz(FDD)), Up to 200 MHz (U6GHz(TDD), Around 7GHz), Up to 400 MHz (Around 15GHz, FR2-1) |
| Power Class (PC) | PC3 | PC3/PC2 | PC3/PC2/PC1.5/PC1 | PC3/PC2/PC1.5/PC1 |
| Modulation Support (DL) | Up to 64QAM | Up to 256QAM | Up to [1024]QAM | Up to [4096]QAM |
| Modulation Support (UL) | Up to 64QAM | Up to 64QAM | Up to [1024]QAM | Up to [1024]QAM |
| Duplex Mode | HD-FDD | HD-FDD/FDD/TDD | FDD/TDD | FDD/TDD |

References

[1] R4-2514650, WF on 6G system parameter, TSG-RAN4 Meeting #116bis, October 2025