3GPP TSG-WG4 Meeting #117	R4-2520894

# Dallas, US, 17 – 21 November, 2025

# Title:	Band n29 MSD due to Band n71 25 MHz CBW

# Agenda Item:	4.2.8

# Source:	Skyworks Solutions, Inc.

# Document for:	Approval

- Introduction

Most of the band n71 changes required to reflect the mandatory support of 25 MHz channel bandwidth (CBW) were captured in the agreed-upon WF agreements [1]. These agreements were reached based on the TS 38.101-1 Rel 19.2.0 for which RAN4 only needed to re-evaluate the Band n85 cross-band isolation MSD for CA_n71-n85. However, BCS4/5 was added to CA_n29-n71 in Rel 19.3.1, meaning that the Band n29 cross-band isolation MSD is also impacted. This contribution re-evaluates the Band n29 PC3 and PC2 MSD for CA_n29-n71.

- Discussion

The Band n29 MSD needs to be re-evaluated for CA_n29A-n71A due to a late BCS4/5 request as reproduced below.

Figure 1 below shows the measured Band n71 power amplifier (PA) output noise level affecting the DL Band n29 
5 MHz CBW SCC with the following test conditions:

- The Band n71 PA PC3 linearity calibrated to meet –30 dBc ACLR at the PC3 MPR1, using DFT-s-OFDM QPSK 20 MHz 100RBstart = 0 SCS 15 kHz waveform,

- The PA output noise is measured at the PC3 MPR0 assuming 4 dB post-PA loss and DFT-s-OFDM QPSK 25 MHz 20RBstart113.

Figure 1: Measured PC3 1UL 25MHz CBW PA emissions at 27 dBm output power for CA_n29A-n71A and LCRB = 20RBstart = 113. The 5 MHz DL SCC is highlighted in red.

Table 1 summarizes the Band n29 5 MHz CBW MSD where we assume 6 dB main/diversity interference imbalance.

Table 1: post MRC calculations for cross band MSD in n29 DL due to 25MHz n71 UL CBW.

| Band n71 UL CBW / LCRB | [MHz]/# | 25 / 20(RBstart= 113) |  |
| --- | --- | --- | --- |
| Meas Tx noise in n85 5MHz SCC | [dBm/CBW] | -15.9 |  |
| 5MHz CBW scaled REFSENS | [dBm/CBW] | -97.0 |  |
| RX noise floor wo Tx noise: Main/Div | [dBm/CBW] | -93.0 | -93.0 |
| Tx-Rx / Tx-Ant duplexer isolation/rejection | [dB] | 40 | 35 |
| LNA to antenna insertion loss | [dB] | 4 |  |
| Antenna isolation | [dB] | 0 | 7 |
| PC3 1Tx interference levels: Main/Div | [dBm/CBW] | -52.0 | -58.0 |
| REFSENS degradation after MRC | cor [dB] | 39.5 |  |

Based on the RAN4 PRD rules, we propose that:

- The test point for Band n71 25MHz UL CBW becomes the so-called “first mandatory” MSD test point,

- The test point for Band n71 20MHz UL CBW remains specified and becomes the so-called “second optional” MSD test point.

Proposal 1: In Table 7.3A.6-1 and for CA_n29-n71, consider adopting the Band n29 PC3 MSD changes highlighted in yellow for MSD due to cross-band isolation interference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 39.56,9 | ACLR1 |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 17.59 | ACLR2 |
| NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 9: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination |  |  |  |  |  |  |  |  |  |

The PC21Tx and the PC22Tx MSD requirements are calculated using the HPUE MSD simplification look-up tables:

- PC21Tx MSD = PC3 MSD + 3 dB = 42.5 dB

- PC22Tx MSD = PC3 MSD + 6 dB = 45.5 dB.

Proposal 2: In Table 7.3A.6-1a-1 and for CA_n29-n71, consider adopting the Band n29 PC2 MSD changes highlighted in yellow for MSD due to cross-band isolation interference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 42.56,8 45.57,8 | ACLR1 |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 20.46,8 23.37,8 | ACLR2 |
| NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UEs supporting PC2 with 1Tx NOTE 7: Applicable to UEs supporting PC2 with 2Tx NOTE 8: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination |  |  |  |  |  |  |  |  |  |

- Conclusion

This contribution proposes to adopt the following changes to complete the RF requirements impacted by the introduction of Band n71 25 MHz CBW.

Proposal 1: In Table 7.3A.6-1 and for CA_n29-n71, consider adopting the Band n29 PC3 changes highlighted in yellow for MSD due to cross-band isolation interference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 39.56,9 | ACLR1 |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 17.59 | ACLR2 |
| NOTE 6: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 9: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination |  |  |  |  |  |  |  |  |  |

Proposal 2: In Table 7.3A.6-1a-1 and for CA_n29-n71, consider adopting the Band n29 PC2 MSD changes highlighted in yellow for MSD due to cross-band isolation interference.

| UL band | DL band | UL Fc | UL BW | SCS of UL band | UL RB Allocation | DL Fc | DL BW | MSD | Cross-band Interference source |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | (MHz) | (MHz) | (kHz) | LCRB | (MHz) | (MHz) | (dB) |  |
| n71 | n29 | 685.5 | 25 | 15 | 20 (RBstart=113) | 719.5 | 5 | 42.56,8 45.57,8 | ACLR1 |
| n71 | n29 | 688 | 20 | 15 | 20 (RBstart=86) | 719.5 | 5 | 20.46,8 23.37,8 | ACLR2 |
| NOTE 4: Applicable to UE not supporting n71 optional maximum symmetrical UL/DL channel bandwidth NOTE 5: Applicable to UE supporting n71 optional maximum symmetrical UL/DL channel bandwidth. NOTE 6: Applicable to UEs supporting PC2 with 1Tx NOTE 7: Applicable to UEs supporting PC2 with 2Tx NOTE 8: Not applicable to UEs indicating support of low NR band aggregation via switching supportedLowBandSwitching-r19 for this band combination |  |  |  |  |  |  |  |  |  |

# References

- R4-2514596 WF on R19 spectrum enhancement, 3GPP TSG-RAN WG4 Meeting # 116bis, Prague, Czech Republic, Skyworks Solutions, Inc., Murata Manufacturing Corp, Qualcomm France.