3GPP TSG-RAN WG4 Meeting # 117	 R4-2520947

Dallas, US, 17th November – 21st November, 2025

Agenda item:	6.12.2.2

Source: 	Huawei, HiSilicon

Title: 	Discussion on performance part of other RRM requirements

Document for:	Discussion

- Introduction

The Rel-19 MIMO WI is approved in [1]. In this contribution, we provide our views regarding the RAN4 impacts on the performance part of CJTC and asymmetric DL sTRP/UL mTRP.

- Discussion

UE reporting enhancement for CJT calibration

Background

During the last meeting, the RAN4 agreed to further discuss two alternatives on the accuracy requirement. The content of the agreements is captured from the WF[6] and shown as follows:

Issue 1-1: Accuracy requirements for delay and frequency offset reporting for CJT calibration Way forward: Alternatives for further discussion: Alt 1: Define one set of [tighten] accuracy requirements based on SNR >= 0 or 3dB Encourage further input on the exact performance requirement Note: “tighten” means it is tighten compared to -3dB side condition Alt 2: Define two sets of requirements with UE capability Encourage input on details of the UE capability

In this paper, we would like to provide our views on how to define the accuracy requirements based on Alt 2. To define two levels of UE capability on the CJTC accuracy requirement. During the last several meetings, the simulation results of companies’ contributions seem to produce two extremes. The reason may be that the way channel and phase estimation are carried out in the simulation is different. For example, with or without collaboration.

On the other hand, the frequency offset and delay offset that UE provide to network are supposed to benefit the TRPs' collaboration. For instance, the FO and DO should match the real difference between the two TRPs. This means low-accuracy DO and FO estimation results can not have been utilised by the network to do the collaboration.

Observation 1: The low-accuracy DO and FO estimation results can not have been utilised by the network to do the collaboration in some cases.

Therefore, we propose defining two accuracy sets to ensure the applicability of CJT collaboration. The details are discussed in the following section. From our perspective, in order to achieve an end‑to‑end implementation of CJTC across the UE and the network, we are also fine to define the accuracy in terms of the 3 dB simulation results from our contribution.

Improvement of advanced level

Here is the improvement we would like to provide from the basic level to the advanced level. Based on the analysis of the simulation method, there are two crucial points that will affect the estimation result significantly: the channel estimation and frequency/delay tracking. And also several aspects will provide the improvement as well, such as the power selection and multiple samples in the Doppler domain.

For the channel estimation, there are several channel estimation algorithms:

- Least Square Method (LS)

- Minimum mean square error (MMSE)

- Linear Minimum Mean Square Error (LMMSE)

For the frequency/delay tracking, there are two types of methods:

- Phase-difference-based. This is to let the UE collect two RS from different moments, and use the phase difference to derive the frequency offset.

- Correlation-based. This is using the Doppler domain sample(s) to do the correlation to find the strongest path.

According to these, we re-evaluated the CJTC-FO and CJTC-DO estimation. The latest simulation result can be found in our paper R4-2520948 [7]. In this paper, we propose to define two levels of accuracy requirement of CJTC, based on the analysis and simulations. Here are the details of these two sets of CJTC accuracy on frequency offset and delay offset.

CJTC accuracy Set 1: The approach is the general channel estimation, such as the LS algorithm, and phase-difference-based frequency tracking.

| Accuracy of Frequency Offset |  |  |
| --- | --- | --- |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 33 | +3 | 15 |
| 68 | +3 | 30 |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 12 | +3 | 15 |
| 14 | +3 | 30 |

CJTC accuracy Set 2: Beyond the basic level, the channel estimation method will use a high complexity algorithm, such as the MMSE, to enhance the estimation quality of the channel, and the frequency tracking was based on the correlation method.

| Accuracy of Frequency Offset |  |  |
| --- | --- | --- |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 3 | +3 | 15 |
| 4 | +3 | 30 |
| Accuracy of Delay Offset |  |  |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 2 | +3 | 15 |
| 3 | +3 | 30 |

Proposal 1: RAN4 Define two sets of requirements with UE capability for the CJTC report.

For Set 1, the accuracy can be defined as:

| Accuracy of Frequency Offset |  |  |
| --- | --- | --- |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 33 | +3 | 15 |
| 68 | +3 | 30 |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 12 | +3 | 15 |
| 14 | +3 | 30 |

For Set 2, the accuracy can be defined as:

| Accuracy of Frequency Offset |  |  |
| --- | --- | --- |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 3 | +3 | 15 |
| 4 | +3 | 30 |
| Accuracy of Delay Offset |  |  |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 2 | +3 | 15 |
| 3 | +3 | 30 |

UE capability

To discuss UE capability, we need to consider three aspects. First, the UE capability should cooperate with the RAN‑1 CJTC‑report capability. Second, the capabilities for frequency‑offset accuracy and delay‑offset accuracy ought to be defined as separate items. Finally, we must clarify that the new capability is intended to inform the network of the UE’s performance on the CJTC report; it is conditionally mandatory for any UE that supports providing a CJTC report.

Proposal 2: RAN4 introduces two conditional mandatory UE capabilities for the CJTC-FO and CJTC-DO report. These capabilities indicate which of the two accuracy sets defined for the CJTC report the UE supports. (The draft UE feature can be found in the Annex).

To answer the question about how the network manages the CJT report results if multiple UEs with different accuracy capabilities are in one cell or a certain zone. This will be the network implementation, but one simple principle may be applicable that the network assigns the high-performance capability UE’s reports as high priority to do the offset compensation, for example.

Observation 2: The network can assign the high-performance capability UE’s reports as a high priority to do the offset compensation.

The purpose of this feature is to enable the UE to assist the network in calibrating two TRPs. Once the two TRPs are synchronised, all UEs within the coverage area will benefit—provided there are UEs capable of delivering high‑accuracy results to the network.

Observation 3: Defining two accuracy levels enables the UEs that receive a downlink signal from these two TRPs to benefit.

- Conclusions

Observation 1: The low-accuracy DO and FO estimation results can not have been utilised by the network to do the collaboration in some cases.

Proposal 1: RAN4 Define two sets of requirements with UE capability for the CJTC report.

For Set 1, the accuracy can be defined as:

| Accuracy of Frequency Offset |  |  |
| --- | --- | --- |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 33 | +3 | 15 |
| 68 | +3 | 30 |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 12 | +3 | 15 |
| 14 | +3 | 30 |

For Set 2, the accuracy can be defined as:

| Accuracy of Frequency Offset |  |  |
| --- | --- | --- |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 3 | +3 | 15 |
| 4 | +3 | 30 |
| Accuracy of Delay Offset |  |  |
| Max Estimation error | SNR | SCS |
| level | dB | KHz |
| 2 | +3 | 15 |
| 3 | +3 | 30 |

Proposal 2: RAN4 introduces two conditional mandatory UE capabilities for the CJTC-FO and CJTC-DO report. These capabilities indicate which of the two accuracy sets defined for the CJTC report the UE supports. (The draft UE feature can be found in the Annex).

Observation 2: The network can assign the high-performance capability UE’s reports as a high priority to do the offset compensation.

Observation 3: Defining two accuracy levels enables the UEs that receive a downlink signal from these two TRPs to benefit.

4. References

[1] RP-242394 WID: NR MIMO Phase 5

[2] R4-2502596 WF on RRM requirements for NR_MIMO_Ph5_Part2

[3] R4-2508272 WF on RRM requirements for NR_MIMO_Ph5_Part2

[4] R4-2512130 WF on RRM requirements for NR_MIMO_Ph5_Part1

[5] R4-2512131 WF on RRM requirements for NR_MIMO_Ph5_Part2

[6] R4-2514804 WF on NR_MIMO_Ph5_RRM_Part2

[7] R4-2520948 Simulation results for CJT calibration in NR_MIMO_Ph5_Part2

5. Annex

Table 5-1 Draft UE capability of CJT collaboration accuracy

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (V2X WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| NR_MIMO_Ph5 | x-1 | UE accuracy performance of the CJTC frequency offset. | Indicates UE supports which set of accuracy requirements on the CJTC frequency offset. The set 1 values defined in TS 38.133 clause 10.x.x. The set 2 values defined in TS 38.133 clause 10.x.x. | N/A | YES | N/A | If UE does not support this capability, network does not know UE’s performance of CJTC-FO report accuracy. | Per UE | NO | NO | N/A |  | Conditional mandatory for UE support RAN1 Rel19 UE capability 59-2-3-2/59-2-3-2a or RAN1 Rel-19 UE capability 59-2-3-5 |
| NR_MIMO_Ph5 | x-2 | UE accuracy performance of the CJTC delay offset. | Indicates UE supports which set of accuracy requirements on the CJTC delay offset. The set 1 values defined in TS 38.133 clause 10.x.x. The set 2 values defined in TS 38.133 clause 10.x.x. | N/A | YES | N/A | If UE does not support the capability, network does not know UE’s performance of CJTC-DO report accuracy. | Per UE | NO | NO | N/A |  | Conditional mandatory for UE support RAN1 Rel19 UE capability 59-2-3-1/59-2-3-1a or RAN1 Rel-19 UE capability 59-2-3-5 |