TSG-RAN Working Group 4 (Radio) meeting #117	  R4-2521136

Dallas, TX, USA, 17 – 21 November 2025

Source:	Ericsson

Title:	On the guard bands for intra-band contiguous CA and the allowed MPR

Agenda item:	10.2.1

Document for:	Approval

# Background

In this contribution we consider the potential issues on the guard bands and MPR raised in [1]:

“one is related to the equations that define the Intra-Band contiguous Carrier Aggregation (CA) configuration parameters; the second is related to the equations that define the non-contiguous RB allocation MPR. We would like to collect the views of interested companies on these issues and we offer some initial solutions to address each issue.”

A most relevant contribution and important for operation of CA in the field. Firstly, we consider the difference between the BS and UE specifications.

# The guard bands for intra-band contiguous CA for the BS and UE

The internal GB and unwanted emissions requirements are different for the BS and UE. For the BS the emissions outside aggregated carriers are defined w r t the outermost carrier with its internal GB, whereas the UE unwanted emissions are defined w r t the aggregated channel bandwidth with symmetric internal GBs facilitated by a nominal CA spacing that is smaller than the channel spacing (BW1 + BW2)/2 assumed for minimum requirements as shown in in Figure 1. For the BS the outer GBs of the aggregated CA bandwidth can therefore be asymmetric.

Figure 1: asymmetric (BS) and symmetric (UE) internal GBs for CA.

The nominal carrier spacing (NCS) is specified such that the guard bands for the UE become symmetric when the channel bandwidths of the aggregated carriers are unequal, e.g. for the 100k raster

the channel spacing reduced for CA by using part of the carrier gap.

The specification of the nominal CA spacing, aggregated CA bandwidth and guard bands was complicated by the support of multiple numerologies and is now also subject to different interpretations in the field. The CA spacing and the aggregated channel bandwidth are determined by the maximum SCS supported by both carriers but applies for all numerologies configured.

For both the 100k- and SCS-based channel raster, the nominal channel spacing (NCS) is

NCS = FC,high – FC,low ≤ (BW1 + BW2)/2 - |GB1,0 - GB2,0)|

where FC,high and FC,low are the upper and lower centre frequencies of the carriers as specified in clause 5.3A.5 of 38.101-1 and similarly in 38.104, while GB1,0 and GB2,0 are the respective minimum guard bands for the lower and upper carrier as specified in clause 5.3.3 of 38.101-1 for the largest subcarrier configuration µ0 supported by both carriers.

The frequency offsets for the lower and upper carriers are also determined by sub-carrier spacing SCS0 of the largest subcarrier configuration µ0 supported by both carriers and applies for any sub-carrier configuration µ ≤ µ0 configured for the carriers. For the UE

Foffset,low = BW1/2 + max(GB2,0 – GB1,0,0)

Foffset,high = BW2/2 – SCS0 + max(GB1,0 – GB2,0,0)

the upper offset slightly smaller than the lower for equal carrier bandwidths since BW/2 = (12*NRB – 1)*SCS/2 + GBmin + SCS at the upper edge of a single carrier with sub-carrier spacing SCS and minimum guard band GBmin as given by that at the lower edge.

Hence the resulting aggregated bandwidth for the UE

BWChannel_CA = NCS + Foffset,low + Foffset,high ≤ BW1 + BW2 – SCS0

for any sub-carrier configuration µ ≤ µ0 configured on a CC.

The actual internal GB between the aggregated bandwidth edge and the outer PRB of the resource grid then varies with the numerology µ – but there are different interpretations of this in the field. The aggregated channel bandwidth BWChannel_CA is always less than the sum of the channel bandwidths, the same holds for more than two CCs.

For the BS

BWChannel_CA = NCS + Foffset,low + Foffset,high ≤ BW1 + BW2 - |GB1,0 - GB2,0)| – SCS0

smaller than that for the UE when the channel bandwidths are unequal.

# The observations and proposals on the GB and offsets in R4-2514496

The first issue is the guard band size and the frequency offsets that determine the aggregated channel bandwidth raised in [1]. For the combination of a 35 MHz SCC with PCCs of carrying bandwidths, the following results were obtained:

Based on an interpretation in accordance with section 2 above, then for the UE in case of a PCC bandwidth of 5 MHz:

BWChannel_CA = 39.89 MHz for based on µ0 = 1 with 100k raster (NCS = 19.5 MHz)

BWChannel_CA = 39.95 MHz for based on µ0 = 1 with SCS-based raster (NCS = 19.56 MHz)

not in accordance with the table above (39.98 MHz) that may be based on a different interpretation. The minGBlow and minGBhigh quoted above for the carriers are not the guard band sizes applicable for µ0 = 1, the largest sub-carrier configuration supported by both carriers, that should be used for determining the frequency offsets and thus the BWChannel_CA.

The actual guard band sizes of the aggregated carriers for the sub-carrier configurations µ configured are (notation as per [1])

GBlow = Foffset,low – (12*NRB1 + 1)*SCS1/2 = GB1 + max(GB2,0 – GB1,0,0)

GBhigh = Foffset,high – (12*NRB2 – 1)*SCS2/2 = GB2 + SCS2 – SCS0 + max(GB1,0 – GB2,0,0)

with GB1 and GB2  the respective guard band for the configured SCS on the lower and upper carrier, SCS2 sub-carrier spacing for the upper carrier and the configured sub-carrier configuration and using Foffset,low and Foffset,high as determined in section 2. We observe that

Observation 1: following 38.101-1 clauses 5 and 6, the lower GB of the aggregated carriers is always greater than or equal to the corresponding GB for the lower carrier for the configured SCS, whereas the upper GB can be smaller, but the maximum difference is (SCS0 - SCS) with SCS0 the largest sub-carrier configuration supported by both carriers.

Regarding Observation 1 and Proposal 1 in [1],

Observation 1: Based on the results collected in the Annex 1 and 2, it seems that:

- The lower edge CC “minGBCC,low“ is always met using either equation set (TS 38.101-1 or TS 38.104). The worst case being CA configurations where “GB,low” is equal to “minGBCC,low”, the CA configuration provides 0Hz margin, i.e. GB,low ≥  minGBCC,low.

- The equations from both TS often fail to meet the upper edge CC “minGBCC,high”, i.e. GB,high < minGBCC,high . The failures do not seem to exceed more than one sub-carrier (SC) of the largest common µ.

Proposal 1: Interested companies are invited to check and confirm if the issues identified in the Observation 1 based on whether or not the small CA configuration sets of Annex 1 / Annex 2 are valid.

Assuming the above issue is confirmed valid, we see two options to correct “GB,high”

- Option 1: Increase “GB,high” by decreasing the nominal channel spacing AND by increasing Foffset,high, where:

  - Fedge,low, GB,low, Foffset,low, FC,low, BWChannel_CA and Fedge,high are unchanged;

- NCS, FC,high and BWgap are reduced; and

- Foffset,high and GB,high are increased.

- Option 2: Increase “GB,high” by increasing Foffset,high, where:

  - Fedge,low, GB,low, Foffset,low, FC,low, NCS, BWgap and FC,high are unchanged; and

  - BWChannel_CA , Foffset,high and GB,high are increased by 1 SC of the largest common µ.

Observation 1 can be confirmed notwithstanding any difference in the interpretation of the specifications. For Option 1 we note that this would be a NBC, changing the CA spacing is often used in the field but not the NSC. Option 2 is not impossible, it would imply that

BWChannel_CA = NCS + Foffset,low + Foffset,high ≤ BW1 + BW2

for the UE. However, it is a standard change and should be motivated by a need from the standpoint of compliance with unwanted emissions limits. The MPR is large for edge allocations within the aggregated bandwidth.

Proposal 1: regarding the upper GB of the aggregated channel BW, do not modify the Foffset,high in 38.101-1 (the ‘Option 2’ in R4-2514496) unless necessary for compliance with unwanted emissions requirements; the allowed MPR is large for edge allocations within the aggregated BW.

# MPR and inconsistent outer PRB allocations

The next issue discussed in [1] is inconsistent outer allocations for intra-band contiguous CA with non-contiguous RB allocations, the proposed changes in red highlight

Proposal 3: To correct the inconsistent outer 1/outer 2 MPR requirements for intra-band contiguous CA configurations where both CCs are configured with the same CBW and same SCS. consider adopting the following changes to RBStart,low and RBStart,high for non-contiguous RB allocations (clause 6.2A.2.1):

Change RBStart,low

from	RBStart,Low = max(1,  2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz))

to 	RBStart,Low = max(0,  2 ∙ NRB_alloc – ceiling( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz))

Change RBStart,high

from	RBStart,High = floor((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

to 	RBStart,High = ceiling((2 ∙ BWChannel_CA – 3 ∙ BWgap – BWGB,low) / 0.18MHz – 3 ∙ NRB_alloc)

We note that the lower and upper limits of RBstart are not symmetric, an attempt at deriving these below.

Inner non-contiguous allocations

An allocation is a non-contiguous inner allocation if the 3rd order IM products are strictly limited to within the aggregated channel bandwidth BWchannel_CA. Introducing the frequency span of the allocated RBs as

fspan_RB = NRBalloc * SCSµ=0 + BWgap

where

NRB_alloc = (NRB1 - RBStart1)∙ 2^µ1 + (RBStart2 + LCRB2 ) ∙ 2^µ2,

the frequency span of the lower edge of the lowest PRB and upper edge of the highest allocated PRB including the gap between the carriers. The exiting limits RBStart,Low and RBStart,High of the RBStart_CA (the RBstart1) are such that for fspan_RB ≤ BWchannel_CA/3, three of these frequency spans fit in between the lower edge of BWchannel_CA and the upper edge of (BWchannel_CA – BWGB,low) as shown in Figure 2, hence slightly asymmetric.

Outer1 non-contiguous allocations

An allocation is a non-contiguous inner allocation if the 5th order IM products are strictly limited to within the adjacent aggregated channel bandwidths of the same BWchannel_CA. The existing limits RBStart,Low and RBStart,High of the RBStart_CA are such that for fspan_RB ≤ 3*BWchannel_CA/3, five of these frequency spans fit in between the lower edge of BWchannel_CA and the upper edge of (BWchannel_CA – BWGB,low) as shown in Figure 2, hence these limits are also slightly asymmetric.

Figure 2: the limits of inner- and outer1 non-contiguous allocations. The carrier gap is shown in dark grey.

The limits of outer 2 non-contiguous allocations are also asymmetric, the lowest RB of the lower carrier is always an outer 2 allocation:

RBStart,Low = max(1, 2 ∙ NRB_alloc – floor( (BWChannel_CA – 2 ∙ BWgap + BWGB,low)/0.18MHz))

which is not always the case for the highest RB of the upper carrier. This would be changed by the Proposal 3 in [1] but would imply a tighter requirement. The case of a single RB at the edge of one carrier with the other carrier allocated is rare: it could be a common PUCCH but this would not be transmitted if there is a PUSCH allocation on the other carrier.

The asymmetric outer 2 would be removed by Proposal 3 in [1]. This implies a slightly tighter MPR requirement for edge allocations and may have to be confirmed for legacy devices. At any rate, the following clarification should be made to avoid different interpretation of GB and frequency offset by UEs in the field:

Proposal 2: clarify in 38.101-1 by an informative note that the BWchannel_CA is the same for all configured SCS ≤ SCS0 with SCS0 the sub-carrier spacing of the largest sub-carrier configuration supported by both carriers, but that the actual GB depends on the configured SCS on the carrier,. In the MPR section clarify that the Foffset is determined by the said SCS0.

# Proposal

Concerning the observations and proposals in R4-2514496 we propose that

Proposal 1: regarding the upper GB of the aggregated channel BW, do not modify the Foffset,high in 38.101-1 (the ‘Option 2’ in R4-2514496) unless necessary for compliance with unwanted emissions requirements; the allowed MPR is large for edge allocations within the aggregated BW.

based on

Observation 1: following 38.101-1 clauses 5 and 6, the lower GB of the aggregated carriers is always greater than or equal to the corresponding GB for the lower carrier for the configured SCS, whereas the upper GB can be smaller, but the maximum difference is (SCS0 - SCS) with SCS0 the largest sub-carrier configuration supported by both carriers.

The following clarification should be made to avoid different interpretation of GB and frequency offset by UEs in the field:

Proposal 2: clarify in 38.101-1 by an informative note that the BWchannel_CA is the same for all configured SCS ≤ SCS0 with SCS0 the sub-carrier spacing of the largest sub-carrier configuration supported by both carriers, but that the actual GB depends on the configured SCS on the carrier,. In the MPR section clarify that the Foffset is determined by the said SCS0.

A CR against 38.101-1 is provided in [2]. No changes are needed in the 38.104.

# References

1.    R4-2514496, “On Intra-Band Contiguous CA GB and Outer1,2”, Skyworks Solution, Inc.

2.    R4-2521137, “Clarification of the bandwidth of guard bands for intra-band contiguous CA”, Ericsson