3GPP TSG-RAN WG4 Meeting #117	R4-2521143

Dallas, US, 17th – 21st November 2025

Source:	Nokia

Title:	Discussion on NR LBCA enhancements Core Requirements

Agenda item:	7.8.4

Document for:	Discussion

Discussion

In the last meeting, RAN4 agreed with work scope for Rel-20 LBCA. The majority of agreed work scope are similar to LBCA Rel-19 due to recommendation of RAN Plenary to reuse Rel-19 agreements as much as possible.

General

From the perspective of Rel-19 LBCA switching with one DL RX, the expected behavior has been well-established: aside from the switching gap, there is no impact on the PCell DL reception. This behavior is illustrated in Figure 1 and serves as a benchmark for evaluating subsequent releases.

Figure 1 Rel-19 LBCA behavior

However, in the context of Rel-20 LBCA switching, the requirements for carrier switching between FDD PCell and SDL remain ambiguous. This lack of clarity necessitates a careful examination of the potential impacts on FDD PCell DL reception. Two possible scenarios can be anticipated:

- No impact on PCell DL reception 
This scenario aligns with the ideal outcome, where the addition of SDL does not impose any interruptions on PCell DL reception. Under this assumption, the overall transmission throughput is expected to improve, delivering the intended benefits of SDL integration. Achieving this behavior, however, requires that the UE RF design be capable of accommodating seamless switching without degradation to PCell DL performance. This option represents the optimal path forward, ensuring both technical feasibility and performance gains.

Figure 2 non impacted PCell DL by Rel-20 LBCA switching

- Switching gap impact on PCell DL

In this scenario, hardware and RF design constraints may necessitate interruptions in PCell DL reception during switching from PCell UL to SDL. While the details of this behavior will be further explored in the RF session, current assumptions from the RRM perspective suggest that such interruptions could occur. More critically, frequent switching events resulting in PCell DL could undermine the anticipated benefits of SDL integration, raising concerns about the overall efficiency and effectiveness of this approach.

Kindly note that until the RF session provides clarity on the expected behavior of PCell DL during switching gaps, our proposals are formulated based on the behavior depicted in Figure 2 unless otherwise stated.

Figure 3 PCell DL impact by Rel-20 LBCA switching

Two types of behavior for Rel-20 LBCA could be expected during switching gap i.e.,

  - No impact on PCell DL reception

  - Switching gap impact on PCell DL. RAN4 to clarify the impact of switching between UL and SDL on PCell DL.

RAN4 to base the assumption that the PCell DL is not impacted during switching gap until the RF session provides clarity on the expected behavior of PCell DL during switching gaps.

#### Issue 1-1-2: MRTD assumption - Proposal 1 (ZTE Corporation, Sanechips, Samsung): Reuse the MRTD requirement specified in R19 low band CA via switching - Proposal 1-1 (Samsung): The 3us maximum receive timing difference for collocated FDD PCell and SDL SCell CA via switching

This refers to the assumed maximum difference in downlink timing between a collocated FDD PCell and SDL SCell. In Rel-19 LBCA switching, RAN4 specified that the MRTD for a collocated low-band FDD+SDL pair is 3 µs. The question for Rel-20 is whether to reuse the same 3 µs MRTD assumption? Since Rel-20 aims to “reuse Rel-19 requirements as much as possible,” maintaining the same MRTD assumption would provide continuity.

Reuse the MRTD requirement specified in R19 low band CA via switching.

#### Issue 1-1-4: SSB assumption Proposal 1 (Samsung): To define Rel-20 LB CA RRM requirement, the following generic assumptions defined in Rel-19 can be reused: SSB based SDL SCell operation

In Rel-19 LBCA switching, the SDL SCell was assumed to transmit an SSB periodically, which UEs use for synchronization measurements. For Rel-20, the question is whether the same assumption holds? In our view SSB-based operation should be the baseline. While SSB-less operation would simplify many of the RAN4 requirements, we do not see that this scenario alone can ensure the feasibility of the feature. Hence, RAN4 should at least define RRM requirements for SSB-based SCell operation. If there is time, SSB-less operation can be supported if the specification impact is not major.

Rel-19 SSB based SDL SCell operation can be reused as a baseline.

Carrier switching between CC/Switching pattern applicability

#### Issue 1-2-2: Interruption or scheduling restriction requirement for CC switching - Proposal 1 (ZTE Corporation, Sanechips, LG Electronics Inc., Apple, Samsung): For Rel-20 LB CA via switching, no interruption or scheduling restriction requirements for carrier switching between FDD CC and SDL CC

In our view CC switching is not different to Rel-19, however the impact of switching on PCell is not clear. For an ideal performance of Rel-20 LBCA no interruption or scheduling restriction should be allowed on PCell DL due to carrier switching if the behavior of Figure 2 is expected.

No interruption or scheduling restriction is expected in Rel-20 LBCA via switching work item, i.e.

  - No impact on PCell DL reception is expected during switching gap.

  - The impact of switching time is captured by the switching gap configuration for SDL and FDD UL

Applicability of Rel-19 LB-CA RRM requirements

In the previous meeting RAN4 agreed on the following aspects of SCell requirements. The below mentioned requirements are adopted from Rel-19 and can be reused for Rel-20.

Issue 1-3-2: SDL SCell related requirement agreements - For L3 measurement and L1 measurement on activated SDL SCell, reuse the Rel-19 requirement as starting point. - For TCI state switch delay requirements, reuse the Rel-19 requirement as starting point. - For SDL SCell activation/deactivation delay requirements, reuse the Rel-19 requirement as starting point. - No new RRM requirement is expected for deactivated SCell measurement period requirements

Requirement design for gap-less based L3 measurement and L1 measurement of activated SDL SCC

- In our view the same approach as in Rel-19 regarding gap-less measurements (L3 and L1 measurements) could be adopted for Rel-20 LBCA.

#### Issue 1-3-1: Requirement design for gap-less based L3 measurement and L1 measurement of activated SDL SCC - Proposal 1 (Apple, LG Electronics Inc., Samsung, Qualcomm Incorporated [Proposal 3]): In Rel-20 LB-CA via switching, for gap-less based L3 measurement and L1 measurement of activated SDL SCC: - a. If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed. - b. If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions. - c. If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Proposal 1 in Issue 1-3-1 follows the agreements that we reached in Rel 19 LBCA switching. We think it is natural to simply extend the same assumption for the activated SDL cell in Rel 20.

In Rel-20 LB-CA via switching, for gap-less based L3 measurement and L1 measurement of activated SDL SCC:

- a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed.

- b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

- c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Potential RRM requirement impact for Rel-20 LB-CA

SDL SCell related RRM requirement

In the last meeting SDL SCell related RRM requirements briefly discussed and most of the discussion postponed to after RF progress on case 3 architecture. In the meantime, if it is concluded by RAN4 that the PCell DL is impacted by the switching gap, for deactivated SCell measurements an interruption (a short tuning/re-tuning time) to PCell DL is expected. In particular for, deactivated SCell measurements a [tuning + retuning] interruption to PCell DL and [tuning + retuning+ SMTC duration] interruption to UL PCell is expected. Accordingly a missed ACK/NACK rate of [2x[tuning + retuning] + SMTC duration]/ measCycleSCell will impose to system.

For deactivated SCell measurements a [tuning + retuning] interruption to PCell DL (if it is concluded that the PCell DL is impacted by the switching gap) and [tuning + retuning+ SMTC duration] interruption to UL PCell is expected. Accordingly, a missed ACK/NACK rate of [2x[tuning + retuning] + SMTC duration]/ measCycleSCell will impose to system.

FDD PCell related RRM requirement

As mentioned in section ‎1.1 the RF structure has an impact on the PCell RRM requirements. Thus, two cases can be expected. In case of no impact, legacy PCell requirements from Rel-19 (requirements without LBCA) can be reused. Otherwise, the impact of switching gap on PCell DL should be discussed. In this case, if any SSB or CSI-RS on the PCell DL overlaps with switching gap, a delay extension should be considered for the procedures that relies on these reference signals such as intra-frequency L3 measurement (gap-less only), RLM, BFD/CBD, L1-RSRP/SINR, PL-RS switch delay, and TCI state switch delay.

If it is concluded by RAN4 that the PCell DL is impacted by the switching gap:

  - The impact of switching gap on PCell DL and in particular on reference signal-based measurements such as intra-frequency L3 measurement (gap-less only), RLM, BFD/CBD, L1-RSRP/SINR, PL-RS switch delay, and TCI state switch delay should be considered.

Active BWP switching for PCell and SCell

A BWP switch causes an interruption on the carrier being switched. As seen in Figure 4, performing BWP switching as per legacy will also create an interruption on the PCell DL due to required hardware tuning of BWP switching. So, in addition to switching gap the UE will face an extra BWP switching delay which in short term increase the amount of interruptions. In our view BWP switching for both PCell and SCell could be considered so that no additional interruptions imposed to the active cells.

Figure 4: Illustration of problem of BWP switching during LBCA operation.

RAN4 to discuss how to efficiently handle interruptions due to PCell and SCell BWP switching.

Potential RAN1 impacts

#### Issue 1-5-1: Interpretation on switching pattern - Proposal 1 (Ericsson): From RAN4 perspective, the following would need to be accounted for by RAN1 for completing the LBCA via switching feature with switching between Case 1 and Case 3: - A ‘1’ in the switching pattern bitmap configured to the UE indicates that the UE is receiving on both FDD DL and SDL carriers instead of only on the SDL carriers. - N1 represents the switching gap from FDD to dual DL reception - N2 represents the switching gap from dual DL reception to FDD - Case 1: Tx/Rx on FDD carrier 1 and no Rx on SDL carrier 2. - Case 3: Rx on both FDD carrier 1 and SDL carrier 2 and no Tx on FDD carrier 1 - RAN4 to discuss how to involve RAN1 in the work to complete the feature - Proposal 2 (vivo): LS to RAN1 and RAN2 on switching pattern to support switching between case 1 and case 3

Due to continuous operation of FDD PCell DL in Rel-20 LBCA-sw, the interpretation of bits in LBCA switching pattern is clearly different than Rel-19. In contrast to Rel-19 switching pattern when a bit ‘1” indicates to UE of receiving from SDL only, in Rel-20 it indicates receiving on both FDD PCell and SDL. Accordingly, an LS to RAN1 for further clarification and involvement in LBCA WI is required.

RAN4 to send an LS to RAN1 and RAN2 on switching pattern interpretation of Rel-20 LBCA.

Draft LS on switching pattern to support switching between case 1 and case 3

3GPP TSG RAN WG 4 Meeting #117			 R4-25xxxxx

Dallas, USA, 17th – 21st November 2025

Title:	Draft LS on switching pattern to support switching between case 1 and case 3

Release:	20

Work Item:	NR_LBCA_Sw_enh

Source:	3GPP TSG-RAN WG 4

To:	3GPP TSG-RAN WG 1, 3GPP TSG-RAN WG 2

Cc:

Contact person:	Parham Kazemi

parham.kazemi@nokia.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

# Overall description

Due to continuous operation of FDD PCell DL in Rel-20 LBCA-sw, the interpretation of bits in LBCA switching pattern is clearly different than Rel-19. In contrast to Rel-19 switching pattern when a bit ‘1” indicates to UE of receiving from SDL only, in Rel-20 it indicates receiving on both FDD PCell and SDL.

Additionally, RAN4 has reached the conclusion that there is no impact on the PCell DL operation during the switching gap. Therefore, the UE Rx/Tx transmissions are only interrupted for the PCell UL and SDL during gapDurationPCelltoSCell and gapDurationSCelltoPCell.

# 2 	Actions

To TSG RAN WG 1, TSG RAN WG 2

ACTION 1: 3GPP TSG-RAN WG 4 asks TSG-RAN WG1 and TSG-RAN WG 2 for guidance and to see if any specification action is required on switching pattern to support switching between case 1 and case 3.

# 3 	Dates of next TSG RAN WG 4 meetings

TSG RAN4 Meeting #118		9 – 13 February 2026			Gothenburg, Sweden

TSG RAN4 Meeting #118-bis	13 – 17 April 2026			EU

TSG RAN4 Meeting #119		18 – 22 May 2026			China

TSG RAN4 Meeting #120		24 – 28 August 2026			Maastricht, EU

TSG RAN4 Meeting #120-bis	12 – 16 October 2026			Korea

TSG RAN4 Meeting #121		16 – 20 November 2026			Calgary, Canada

Conclusion

Proposal 1: Two types of behavior for Rel-20 LBCA could be expected during switching gap i.e.,

a.	No impact on PCell DL reception

b.	Switching gap impact on PCell DL. RAN4 to clarify the impact of switching between UL and SDL on PCell DL.

Proposal 2: RAN4 to base the assumption that the PCell DL is not impacted during switching gap until the RF session provides clarity on the expected behavior of PCell DL during switching gaps.

Proposal 3: Reuse the MRTD requirement specified in R19 low band CA via switching.

Proposal 4: Rel-19 SSB based SDL SCell operation can be reused as a baseline.

Proposal 5: No interruption or scheduling restriction is expected in Rel-20 LBCA via switching work item, i.e.

a.	No impact on PCell DL reception is expected during switching gap.

b.	The impact of switching time is captured by the switching gap configuration for SDL and FDD UL

Proposal 6: In Rel-20 LB-CA via switching, for gap-less based L3 measurement and L1 measurement of activated SDL SCC:

a.   If all {the SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, no delay extension is allowed.

b.   If a fraction of {the SSB occasions in SMTC window } or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are not available based on the LB-CA switching pattern, the delay is extended in accordance to the fraction of available occasions.

c.   If no {SSB occasions in SMTC window} or {SSB occasions or CSI-RS for L1 measurement} of SDL SCC are available based on the LB-CA switching pattern, the corresponding requirements do not apply.

Proposal 7: For deactivated SCell measurements a [tuning + retuning] interruption to PCell DL (if it is concluded that the PCell DL is impacted by the switching gap) and [tuning + retuning+ SMTC duration] interruption to UL PCell is expected. Accordingly, a missed ACK/NACK rate of [2x[tuning + retuning] + SMTC duration]/ measCycleSCell will impose to system.

Proposal 8: If it is concluded by RAN4 that the PCell DL is impacted by the switching gap:

a.	The impact of switching gap on PCell DL and in particular on reference signal-based measurements such as intra-frequency L3 measurement (gap-less only), RLM, BFD/CBD, L1-RSRP/SINR, PL-RS switch delay, and TCI state switch delay should be considered.

Proposal 9: RAN4 to discuss how to efficiently handle interruptions due to PCell and SCell BWP switching.

Proposal 10: RAN4 to send an LS to RAN1 and RAN2 on switching pattern interpretation of Rel-20 LBCA.

- References

- R4-2514824 WF for [116bis][224] NR_LBCA_Sw_enh_RRM