3GPP TSG-RAN WG4 Meeting # 117	R4-2521210

Dallas, US, November 17 – 21 2025

Agenda Item:	8.8.2

Source:	Ericsson

Title:	Discussion on use cases for AI/ML study for 6G

Document for:	Approval

# Introduction

In the last meeting AI/ML use cases for 6G were discussed. In this contribution, further analysis of those use cases is presented and Ericsson’s view on prioritization of use cases to be studied in RAN4 are highlighted.

# Discussion

## RF Use Cases

### AI/ML for DPD/DPoD

In RAN4#116bis meeting AI/ML techniques to compensate the non-linear characteristics of UE RF front-end behavior was significantly discussed, thereby adjusting the Tx EVM limits and increasing UL high data-rate coverage. As per the WF [1], this can be achieved using UE sided digital pre-distortion (DPD) correction or BS-sided digital post-distortion (DPoD) compensation in the uplink.

Agreement from RAN4#116bis [1]:

- In 6G AI/ML study, the detailed RAN4 scope shall include

  - Identify Use Case(s) of interest (either existing or new) with compelling trade-off between e.g., performance, complexity, etc.

    - RAN4-driven use cases: RAN4 shall identify, select and study only those AI/ML use cases that fall within RAN4 core responsibility. The detailed principles are captured under 3-1.

    - Other WG-driven use cases: RAN4 shall assist other working groups in these use cases by studying RAN4 impacts with RAN4 analysis and evaluation relevant to implementation and performance.

- Principles for AI/ML RAN4-driven use cases selection

  - RAN4 shall closely coordinate with other WG and strive not to duplicate the efforts

    - RAN4 should work with other WG to clarify the split between RAN4 driven AI/ML work and RAN1/2/3 driven work.

    - Coordinated discussions with RAN1/2/3 are required to avoid overlapping scope.

Agreement from RAN4#116bis [1]:

- The scope of potential RAN4-led AI use case, AI-DPD/DPoD, will be further discussed in the next meeting. The candidate sub-use cases include

- UL:

  - UE: AI-DPD

  - NW: AI-DPoD

- DL:

  - NW: AI-DPD

  - UE: AI-DPoD

As per above agreement, our understanding is that this is a potential use-case for RAN4 study item, as it may not have any impact on other working groups and it can be independently driven by RAN4.

RAN4 should study AI based UL-Post distortion compensation at BS receiver and understand the feasibility of creating RAN4 core and performance requirements.

We focus on NW-sided DPoD, considering that UEs typically have limited computational resources and power budgets, which may make the implementation of advanced DPD less practical at the UE side. In addition, AI-based DPD at the UE transmitter could lead to dynamically varying RF performance, which might pose challenges in consistently meeting regulatory emission requirements. Furthermore, AI-based DPD would require LCM aspects to be evaluated by the network, which is not needed in case of NW-sided DPoD.

Advanced DPD can be energy- and computation-intensive for UEs, making it a potentially challenging solution for UL transmissions to avoid power back-off.

Although the implementation may be complex, DPD can offers several significant advantages for the UE. First, the UE has direct knowledge of its own PA and RF front-end, whereas the BS lacks information about the diverse RF front-end characteristics of all connected UEs. Additionally, DPD can enhance UE coverage even at lower data rates and SNR levels, where emission requirements, rather than EVM, are the dominant factor. Therefore, while evaluating NW-sided DPoD, potential spread of DPD in UEs should be taken into account.

RAN4 to clarify the extent to which 6G UEs may support digital pre-distortion, which could further help to evaluate UE transmitter model in the study.

The DL scenario, i.e., UE-side DPoD, is considered less relevant in this study. This is mainly because BS already employ advanced DPD and CFR techniques, leaving little for the UE to further compensate. Moreover, there can be substantial variation in DPD and CFR implementations across different BS. Another important aspect is that BS typically operates multiple transmitters, transmits multiple layers, and supports MU-MIMO, which makes any form of UE-side compensation potentially challenging. In addition, the presence of multi-carrier, multi-RAT, and multi-band PAs on the BS side adds further complexity.

In the candidate sub-use cases, we propose prioritizing “NW: AI-DPoD” for the RAN4 study.

Agreement from RAN4#116bis [1]:

The scope of potential RAN4-led AI use case, AI-DPD/DPoD, will be further discussed in the next meeting.

The candidate sub-use cases include

UL:

  - UE: AI-DPD

  - NW: AI-DPoD

DL:

  - NW: AI-DPD

  - UE: AI-DPoD

To facilitate the 6G AI scope discussion, the interested companies are encouraged to timely provide more analysis, including the performance benefit, spec impact, the assumption behind their analysis, on the specific sub-use case(s).

Table 1. Details of NW: AI-DPoD use case.

| AI model input | Input in training | The model was trained using labeled IQ samples obtained after the IFFT in DFT-s-OFDM |
| --- | --- | --- |
|  | Input in inference | The estimated IQ samples after IFFT in DFT-s-OFDM |
| AI model output | Label in training (if applicable) | Known transmitted coded bits available at the receiver side |
|  | Output in inference | Soft bits |
| Assumption on training | Training type | Offline Training |
|  | Label construction (if applicable) | Based on transmitted bits, which can be obtained at the receiver by relying on - bits decoded after successful cyclic redundancy check (CRCs), - transmission of bit sequences known to both transmitter and receiver. |
| model location for inference |  | NW-sided model |
| Collaboration/interaction between UE and NW |  | The NW may request the UE to - adjust PA back-off associated with relaxed EVM requirements, - generate and provide data to support training and refinement of AI/ML models at the NW side, - provide measurement reports and receive updated configuration from the NW. |
| Evaluation methodology |  | Link level simulation |
| Evaluation assumption |  | Parameters; Value. Carrier frequency; 30 GHz. # of TX layers; 1/2. # of RX antennas; (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1). Waveform; DFT-S-OFDM. # of PRB; 60. # of symbol per slot; 12. # of DMRS per slot; 2. # of DMRS re-used for training; 0. DMRS subcarrier position; -. FFT size; 1024. CP length; -. Modulation; 64, 256 QAM. MCS used in training; 17, 22. Channel model; CDL-A 30-150 ns. Subcarrier spacing; 120 kHz. Evaluation metric; BLER, throughput. Benchmark; Conventional receiver (w/wo non-linearity compensation and w/wo PA ). |
| Evaluation KPI |  | BLER, throughput |
| Evaluation benchmark |  | Legacy receiver without DPoD with PA distortion Legacy receiver without DPoD without PA (upper bound performance) Receiver with AI-DPoD |
| High level potential specification impact |  | - Adjusted in-band distortion (e.g. EVM) and impact to other requirements (e.g. MPR, A-MPR), - UE capability exchange, - Data collection using know bit sequences generated at the UE side by the mean of a shared seed between the UE and the BS, under different hardware operating conditions. |
| Feasibility issues, including complexity, and other aspects related to implementation |  | Performance benefit: 100% link-level throughput gain at mid/high SNR levels at BLER = 10-1 Complexity: 100 MFLOPs |

#### Evaluation for AI/ML for DPoD

Agreement from RAN4#116bis [1]:

- Evaluation Procedures and some detailed discussion points

  - Step-1: Non-linearity model(s) of transmission signals is determined to be used in the evaluation.

##### UE transmitter model

For UL post-distortion compensation at the BS receiver to function effectively, accurate modeling of the UE front-end transmitter is essential, with emphasis on the PA model and other non-linear RF elements such as oscillators, mixers etc. The UE RF front-end model can be trained using AI/ML algorithms; however, since numerous UEs with diverse front-end architectures exist in the market, this leads to a very large dataset of RF models that must be learned by the AI/ML algorithm. Furthermore, RF characteristics vary across the frequency spectrum, and the non-linear behavior is further influenced by additional real-world impairments such as phase noise, local oscillator leakage, IQ imbalance, and variations in EVM across subcarriers. Inter-symbol interference introduced by the PA itself may also affect its nonlinear response. In addition, PA nonlinearity is sensitive to temperature, operating frequency, and load mismatch conditions. Envelope tracking (ET) adds further complexity because the PA supply voltage is continuously adjusted in real time. This means the PA’s behavior is not fixed but changes with the signal envelope and power management circuit response, making accurate modeling even more challenging. Therefore, developing a robust and accurate UE model is critical for the effectiveness of AI/ML-based UE front-end modeling.

​

Accurate modeling of UEs PA and RF-front end is necessary for this feature. However, transmitter characteristics have many variations due to other RF impairments, frequency range, RB allocation etc. Therefore, UE RF front-end needs to be evaluated by RAN4, so that reliable and representative data from the RF front-end can be obtained to effectively train the AI/ML model.

RAN4 needs to study feasibility on how to train AI/ML algorithms with variety or subsets of UE transmitter models.

Agreement from RAN4#116bis [1]:

- Evaluation Procedures and some detailed discussion points

  - Step-2: Evaluation methodology by applying non-linearity model(s) of transmission signals to baseband evaluation with the following procedure

    - FFS a joint session or separate session of AI and non-AI non-linearity compensation.

    - Detailed assumptions for evaluation (including data collection, training and inference) may include, but be not limited to

      - FFS the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA impairment including PA and other RF components non-linearity.

      - FFS suitable AI/ML model, type of training.

      - FFS different data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region: e.g., simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs).

      - FFS how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations

      - Focusing on higher modulation orders focusing on high-SNR conditions

      - Other details are not precluded

##### Discussion thread for AI and non-AI non-linearity compensation

Based on the discussion in the previous meeting [1], it would be required to model the UE Tx front-end transmitter initially for NW-based DPoD, and then consider the AI and non-AI based approaches to compensate the non-linearity in BS. Regarding this, it seems feasible to have a consolidated discussion in UE RF thread for UE transmitter model which could be common for both AI and non-AI discussions. Further, to compensate the UE RF non-linearity, separate discussions maybe feasible in the AI thread (for AI-based approach) and BS Demodulation thread (for non-AI approach). In our opinion, both AI and non-AI approaches would be quite different in terms of performance gain and complexity analysis. In the initial stage, it may be difficult for RAN4 to prioritize any approach because there is no reference study on DPoD for comparison and therefore parallel studies (both AI and non-AI) in different sessions would result in quicker development.

There is no reference study on NW-sided DPoD, therefore both AI and non-AI approach would be needed for performance and complexity evaluation.

We propose to study UE transmitter model in the UE RF session initially and further evaluate the non-linearity compensation in both AI thread (for AI-based approach) and BS Demodulation thread (for non-AI approach).

##### Extent of UE EVM adjustment

A simple solution would be to adjust the EVM requirements for the UE on in-band distortion in the presence of DPoD, thereby reducing the needed UE MPR. However, RAN4 should evaluate not only the limit to which EVM can be adjusted, but also whether there are dependencies affecting this limit, such as resource block (RB) allocation or operating frequency, and whether the extent to which EVM can be adjusted varies depending on the UE implementation. Additionally, if there are additional UE requirements related to the UE transmit signal, while ensuring that the UE remains compliant with Adjacent Channel Leakage Ratio (ACLR) and Spectrum Emission Mask (SEM) specifications. Further, RAN4 should avoid adding additional margins in the BS test to incorporate non-linearities or impairments in the UE. This would ensure strict RAN4 performance requirements while keeping the baseline of creating minimum requirements that can be passed by both UE and BS.

RAN4 could consider the extent of adjusting the UE EVM requirements but also evaluate any additional constraints or dependencies affecting this limit such as compensating the increased distortion at the receiver.

Further, the usefulness of DPoD depends on what limits the UE’s non-linearity. If the primary limitation comes from in-channel distortion, then DPoD could help mitigate the issue by compensating for the distortion at the receiver side. However, DPoD cannot correct the unwanted emissions, such as out-of-band radiation and spurious emissions that impact neighboring channels.

The practical gain of DPoD is limited to in-channel distortion and not unwanted emissions.

##### Deployment scenario (High-SNR conditions with High Modulation order)

In the following, preliminary evaluation results of AI/ML-based uplink PA DPoD are presented as compared to the baseline in an uplink DFT-s-OFDM scenario. Table 1 and Table 2 in the appendix present the parameters used for training and evaluation.

Figure 1 shows the receiver architecture with AI/ML de-mapper to handle UE RF non-linearities.

Figure 1 Schematic of AI based UL Post-Distortion.

Figure 2. Uplink throughput vs. SNR for i) an ideal PA (“no ML, no PA”), and a PA with 5 dB back-off ii) when using an AI/ML-empowered receiver that compensates for PA non-linearities (“ML, 5 dB backoff”), and iii) a non-AI/ML receiver not attempting to perform.

In Figure 2, the “ML, 5 dB backoff” curve refers to a AI/ML receiver compensating for the distortions introduced by a UE PA (see [5]) operating with a 5 dB back-off. The PA operation still allows the UE to comply with ACLR requirements but is assumed not to be required to satisfy the current error vector magnitude (EVM) requirements. In contrast, "no ML, 5 dB backoff" represents a traditional (non-ML) receiver operating with a 5 dB back-off (without DPoD capability), and "no ML, no PA" corresponds to an ideal case with no distortion. The simulation includes link adaptation targeting a 10% BLER, reflecting realistic adaptive modulation and coding behavior. Notably, the key performance gains from the AI/ML DPoD are observed in the high SNR region (above 10 dB), where 256-QAM is typically used. This highlights the AI/ML de-mapper’s effectiveness in handling high-order modulation under non-ideal transmitter conditions, while no noticeable gain is seen for lower-order modulation schemes (16QAM and below) at lower SNRs. In our initial evaluation, we have only considered UE’s PA non-linearity and no other RF components in the UE front-end.

AI/ML-based uplink PA digital post-distortion (DPoD) could in certain conditions achieve performance similar to that of ideal PA without power backoff, resulting in a potentially higher UE output power and/or more power efficient operation for higher order modulations.

For AI-based NW-sided DPoD, we observe reasonable gain under high-SNR and high modulation order conditions with a simple UE PA model and complete investigation with realistic UE-Tx model is needed.

##### AI/ML model training

There can be two approaches to train AI/ML algorithms with different UE RF front-end models: offline and online.

In offline training, UE transmitter characteristics are collected through measurements or simulations and used to build generalized models in advance. This allows the use of large datasets and complex AI/ML architectures without adding real-time processing burden on the UE, but it may not capture device-specific variations or adapt to changes in transmitter behavior over time due to factors such as temperature, frequency band shifts, or device aging etc. Further, offline trained models may need to be updated or fine-tuned during certain intervals to account for new transmitter models being implemented by the UEs.

Online training, on the other hand, enables adaptive learning at the BS receiver, thereby capturing dynamic variations in transmitter characteristics and improving accuracy for individual UEs. However, this approach introduces higher computational complexity, potential latency issues at the BS, and scalability concerns for the UE in multi-user scenarios. Another issue is the training time; if it takes too long to train the AI/ML model, the data may already be delivered from the UE to the BS.

To characterize multiple UE transmitters, either a fine-tuned offline model or less precise and adaptive online approach can be feasible, considering the UE power consumption needed for model training and potential latency issues.

Offline trained AI/ML models may need to be updated in a certain time period to include new UE transmitter models being implemented by UEs.

RAN4 may study which AI/ML algorithms are feasible for modeling UE transmitter characteristics, and which type of model training would be feasible.

##### Data Collection and Feedback

To effectively train AI models, data should be collected when UEs operate in the compressed region, i.e., the non-linear operating range of the transmitter, where post-compensation techniques are most beneficial. Training is not feasible if UEs remain in the linear region, since little or no distortion is present. For data collection, it is important for RAN4 to evaluate suitable approaches. A key question is whether simulation-based data, generated using realistic UE Tx front-end models and diverse PA characteristics, would be sufficient, or whether data gathered directly from multiple UEs would provide a more reliable basis. Another challenge with the dataset is that it must capture UE PA performance under conditions where the UE is driven to the point of “failing” the usual EVM requirement. Such behavior is unlikely to occur naturally in the field unless a specific mechanism is introduced to intentionally push the UE into this operating region.

RAN4 to study data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region. Both simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs) could be evaluated. Other RF impairments should be considered and included.

##### UE RF Front-End Variation

- For UL post-distortion studies, it is important to recognize the UE dependency of RF front-end characteristics and DPD implementations. A UE’s performance is shaped by multiple non-linear components such as the PA, oscillator, and IQ modulator, and in practice, different UEs on the market exhibit varying front-end designs and impairment levels. Also, CFR, when applied to limit PAPR, can also introduce in-band distortion. This variability should be handled by the AI/ML model that is both representative and practical. RAN4 should therefore account for this diversity when developing study assumptions and evaluating compensation techniques.

RAN4 to evaluate RF front-end variation from multiple UEs in the market into account and define suitable AI/ML model for UL post-distortion studies.

The AI/ML model used for DPoD should be trained effectively to handle the UE front-end variation.

- This can be handled by introducing new EVM measurement procedure with updated equalizer taking to account the DPoD receiver.

##### Reference AI/ML model selection

To support consistent evaluation of BS performance, it is necessary to define a reference AI-model with reasonable complexity level. This reference model can serve as a common framework for agreement on key parameters, given that different companies may implement AI/ML solutions differently.

Agreement from RAN4#116bis [1]:

- Evaluation Procedures and some detailed discussion points

  - Step-3: For a given relaxed EVM value (e.g., 6% for a certain modulation order), conduct TX MPR simulation.

    - The potential MPR reduction enabled by AI-DPoD is evaluated.

      - The requirements for other gating factors that impact MPR including ACLR, SEM and SE remain unchanged

    - The impact on UL coverage and UE transmit energy efficiency is evaluated.

RAN4 to evaluate on how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations.

As part of the study, RAN4 should decide the appropriate complexity level of the reference AI/ML model.

##### UE MPR reduction and EVM adjustment

Agreement from RAN4#116bis [1]:

- Other design principle to be considered:

  - UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

Agreement from RAN4#116bis [1]:

- Non-linearity model(s) of transmission signals:

  - Discussed in other 6G agenda (e.g., system parameter agenda and UE RF agenda), which is intended to discuss the common model as much as possible.

    - Using separate models for AI and non-AI DPoD is not precluded.

With the AI-based UL post-distortion (AI-DPoD) feature, UEs can compensate for PA non-linearities more effectively, improving distortion tolerance. As a result, the required UE MPR (Maximum Power Reduction) can be reduced, allowing UEs to transmit at higher effective power without violating linearity requirements. This has potentially two key benefits:

- Increased UL coverage: Higher transmit power extends the uplink range for higher order modulations.

- Improved UE energy efficiency: Lower MPR reduces the back-off from PA saturation, enabling the UE to operate more efficiently, possibly reducing power consumption.

RAN4 to evaluate the potential MPR reduction enabled by AI-DPoD and its  impact on UL coverage and UE transmit energy efficiency.

In order to properly evaluate the potential MPR reduction, an up-to-date UE and MPR modelling approach may be needed to capture the true benefits.

Current UL EVM requirements are specified assuming that the UE transmitter operates within the linear region of the PA, where distortions are minimal. With AI-based UL post-distortion (AI-DPoD) applied at the BS, much of the PA non-linearity can be compensated in the network. This could enable the adjustment of UE EVM requirements but would require changes in EVM measurement procedure and equalizer by inclusion of AI-DPoD receiver.

RAN4 to evaluate the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA and other RF components non-linearity at the base station.

The current UE EVM requirements and EVM measurement procedure including equalizer may need updating to account for factors such as the UE front-end characteristics and the interference environment, in order to more accurately reflect real-world performance.

Furthermore, the proposed uplink post-distortion solution should be strictly network controlled. In other words, the UE may only adjust its EVM requirement and apply MPR reduction when explicitly signaled by the network. In the absence of such signaling, the UE must continue to comply with the existing RF requirements.

UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

As per our understanding, since UE transmitter model maybe discussed in UE-RF session, it would be feasible for UE-RF to define a model which is common for both AI and non-AI non-linearity compensation methods, considering there are multiple UE Tx implementations in the market. This reference UE Tx model would help to evaluate the comparison of performance and complexity between AI and non-AI approaches. Further, it would be feasible to consider separate models for AI and non-AI DPoD since different algorithms, evaluation procedure, performance and complexity KPIs would be required to study the non-linearity compensation in the BS.

A common UE transmitter model with non-linearities should be evaluated in UE RF session, which could further be used for different AI and non-AI DPoD model.

## Demod Use Cases

### AI-SRS-assisted channel reconstruction

It’s been discussed in the last meeting that SRS IL imbalance has been becoming a critical issue that needs to be treated carefully in 6G. As the number of antenna ports and the operating bandwidth increases, Insertion loss differences among UE transmit RF chains become non-negligible. These discrepancies cause SRS IL imbalance, meaning that the actual SRS transmit power across antenna ports is uneven.

We agree that such imbalance will lead to several well-recognized issues such as:

- Inaccurate CSI estimation: the network-derived UL channel differs from the true DL channel

- MU-MIMO performance degradation: imperfect precoding and interference suppression

- Power-control inconsistency: reported SRS power no longer reflects the real transmit power

- Cross-vendor variability: different RF implementations yield inconsistent test results, complicating RAN4 performance evaluations

In NR, RAN4 had already identified this problem, and requirements were defined but never verified. In 6G, this topic is expected to surface at an early stage, with explicit test coverage for 2Rx, 4Rx, 6Rx, and 8Rx handheld UEs. As one of the proposed solutions, AI-based SRS IL loss compensation has been brought up to the table during the last meeting’s discussion.

The target of this method will be if the UE has no available power headroom, it cannot compensate the loss itself. According to the details of this method, the idea is that an AI model implemented at the network side learns to recover the “ideal” CSI from SRS measurements affected by IL imbalance. During inference, CSI-RS is not needed; during model training, CSI-RS-based CSI feedback (or high-SNR SRS data) can be used as ground truth labels.

It’s been proposed that UE may optionally report a coarse “SRS IL range” (for example, a few dB intervals) as assistant information. This information would help the AI model converge faster without requiring the UE to report exact IL values, thereby reducing signalling overhead and protecting UE implementation details.

Except for the potential benefit above, we would like to express the following technical points that may cause further thinking on the feasibility in implementing this in 6G of this use case.

Key technical points that need further investigation:

- Training-data consistency:
Without a unified data generation method, AI performance claims are not comparable across vendors. RAN4 should define baseline channel models and SNR ranges for AI evaluation.

- Reporting strategy:
Whether the UE should provide any IL-related information—and at what granularity—needs further study. The “coarse range” concept must be justified by quantitative gain.

- Performance vs. complexity trade-off:
The AI approach centralizes compensation at the gNB, reducing UE burden but increasing network processing cost. How to quantify this trade-off remains open.

Further investigation could be on “training-data consistency”, “reporting strategy” and “performance vs complexity trade-off”

There are also some unclear points that may need further clarification:

- Purpose and effectiveness of IL-range reporting: A coarse IL-range assists AI convergence has been claimed, but the quantitative benefit versus zero-reporting case is not demonstrated. RAN4 needs evidence showing measurable performance improvement relative to signalling cost.

- UE transparency and privacy: Although range reporting hides detailed RF design, any IL information may still reveal partial implementation characteristics. The trade-off between transparency and AI accuracy should be analyzed.

- Model generalization: The AI model’s performance under different UE vendors, frequency bands, and antenna configurations remains unverified. A single-vendor simulation cannot represent RAN4 consensus.

- Evaluation methodology: Clear test configurations, channel conditions, and comparison baselines (AI vs. non-AI self-compensation) must be defined to ensure reproducibility across companies.

Further clarification is needed on:

- Benefit of IL-range reporting method vs zero-reporting method

- The level of UE implementation transparency for IL-range reporting method

- How to align the AI model performance

- Detailed parameter assumptions

To summarize, companies now share a clear understanding that SRS IL imbalance is a genuine and urgent challenge for 6G RAN performance evaluation. The AI-based compensation proposal provides an interesting network-intelligence-centric approach, potentially reducing UE burden and signalling overhead. However, its practical value depends on how much accuracy can be gained from minimal UE assistance and how consistently the model performs across vendors and bands.
Further evidence and harmonized evaluation are needed before RAN4 can proceed toward any formal requirement or study item.

### AI-enabled PRACH receiver

Agreement from RAN4#116bis [1]

- Principles for AI/ML RAN4-driven use cases selection

  - RAN4 shall closely coordinate with other WG and strive not to duplicate the efforts

    - RAN4 should work with other WG to clarify the split between RAN4 driven AI/ML work and RAN1/2/3 driven work.

    - Coordinated discussions with RAN1/2/3 are required to avoid overlapping scope.

Agreement from RAN4#116bis [1]:

The scope of potential RAN4-led AI use case of AI-enabled PRACH will be further discussed in the next meeting.

The proposed AI/ML-enabled PRACH receiver aims to improve random access by enabling early contention resolution when multiple UEs transmit the same preamble sequence. Using learned models at the gNB, the receiver would not only detect the presence of a preamble but also estimate how many UEs sent that same preamble and provide refined timing advance estimates. With that information the gNB could include multiple PUSCH grants in the Random-Access Response so contending UEs can select appropriate grants and timing values for Msg3, thereby resolving many collisions at the Msg1/Msg2 stage instead of relying on repeated power-ramped retransmissions. Expected benefits include higher detection reliability in low SNR and near-far scenarios, reduced access delay, lower UE energy consumption, and more efficient uplink resource use under massive or bursty access loads anticipated in future 6G networks.

However, this topic is not ready for RAN4 study at this time because it primarily affects the random-access procedure and the fundamental PRACH signal design, which are within RAN1’s scope. Key novel aspects raised by the use-case include: (1) detection of multiple users using the same preamble sequence, (2) procedural consequences for Msg2 through Msg4 if multi-user detection is supported, and (3) lifecycle management, data collection, and performance monitoring for ML models. For example, it is unclear whether current 5G PRACH preamble formats would be suitable if 6G were to permit multi-user detection on the same sequence, and any changes to preamble, sequence allocation, or timing search windows should be studied in RAN1. Procedural changes to Msg2–Msg4 that follow from multi-user detection would eventually be reflected in RRM specifications during a work item, but they do not need to be examined in RAN4 during the initial study item. Likewise, the design of LCM procedures and the specifics of data collection and monitoring for ML performance should be defined in RAN1 before RAN4 considers conformance and measurement requirements. Consequently, while the use case is promising and technically valuable, it is more appropriate for RAN1 to lead the study, with RAN4 to engage later once the foundational signaling and preamble format decisions are settled.

Based on the agreement in RAN4#116bis meeting for RAN4 driven use-case selection, RAN4 may prioritize those use-cases which have minimum to no impact on other WGs.

The AI/ML-enabled PRACH receiver use case depends on new random-access procedures, multi-user detection mechanisms, and potential updates to PRACH preamble design, which are within RAN1’s scope. Current RAN4 requirements only address single-user PRACH detection and cannot assess performance for multi-user collision scenarios.

RAN4 may not prioritize this use-case at this stage and should be driven by RAN1, and its study outcome could be further considered for creating RAN4 performance requirements.

## RRM Use Cases

During the last meeting, approximately 22 use cases were proposed for study. It is evident that RAN4 cannot feasibly address all of these RRM use cases. To support the prioritization process, a set of principles for selecting AI/ML-related use cases was agreed upon. In alignment with these principles, we present our assessment of the proposed use cases and their relevance to the scope of RAN4 studies.

Agreement from RAN4#116bis on principle for AI/ML use case selection [1]:

RAN4 shall closely coordinate with other WG and strive not to duplicate the efforts

  - RAN4 should work with other WG to clarify the split between RAN4 driven AI/ML work and RAN1/2/3 driven work.

  - Coordinated discussions with RAN1/2/3 are required to avoid overlapping scope.

Typical RAN4-driven use case shall

  - ensure performance gain or gain from other aspects, by focusing on some RF/Demod/RRM topics

Prioritize the use case by taking into account:

  - use cases with gain over non-AI counterpart

    - gain may include performance gain or gain from other aspects, like overhead reduction.

    - trade-off between performance and complexity/overhead should also considered.

  - use cases with potential standard impact.

  - Note: the prioritization principle is the general principle and no intention to have implication on RAN4 6G AI work plan.

Agreement from RAN4#116bis [1]:

Additional principle/method for AI-RRM use case selection

  - Whether the proposed AI-RRM use case should be driven by RAN4:

    - Existing 5G-A use cases included in RAN2 and RAN1 WIs should be precluded in RAN4 6G study.

    - Coordination with RAN1 and RAN2 when potential AI-RRM use cases are agreed in RAN4 to avoid overlapping scope.

Agreement from RAN4#116bis [1]:

The scope of potential RAN4-led AI-RRM use case(s) will be further discussed in the next meeting.

From the agreements above following observations can be made.

RAN4 should avoid overlap between the use cases being studied in RAN4 and in other working groups.

Use cases with expected gains over non-AI counterpart shall be prioritized for RAN4 study.

RAN4 should preferably avoid parallel studies for AI/ML and non-AI/ML enhancements for the same RRM procedure or feature. Non-AI/ML Rel-20 6G enhancements (over 5G) can be prioritized over AI/ML based enhancements, since the former will be the reference and widely implemented.

### L1 measurement prediction

Use cases for L1 beam measurement prediction in time, frequency, and spatial domain were proposed in the last meeting to be studied in RAN4 for 6G AI/ML. We understand that the motivation to study L1 measurement prediction is for beam management where the potential goal would be to further enhance the L1 measurement accuracy by employing AI/ML model and/or to reduce the L1 measurement overhead by exploiting AI/ML model for L1 measurement prediction. This in our understanding is similar to the discussion RAN1 is also having [2]. As per the agreement from RAN4#116bis RAN4 should therefore avoid having parallel discussion on L1 measurement prediction and wait until RAN1 concludes whether or not to study AI/ML based beam management for 6G.

AI/ML for beam management is being discussed in RAN1. RAN4 to avoid having parallel discussion on AI/ML for L1 measurement prediction that is relevant for beam management in 6G.

In addition to L1 measurement prediction, use cases such as L3 measurement prediction, adaptation of measurement procedure was also proposed for RAN4 led AI/ML study for 6G.

### L3 measurement prediction

AI/ML for prediction of L3 beam level and L3 cell level RRM measurements were also discussed to be considered for 6G AI/ML study in the last meeting. To evaluate and identify the gain AI/ML can bring in these use cases, a reference measurement model and framework for 6G mobility are required. Since the corresponding non-AI/ML baseline is yet to be defined, though we understand the motivation to study these use cases in RAN4, we think RAN4 should wait before starting any work on L3 measurement prediction.

To quantify the benefits AI/ML can provide for L3 measurement prediction (both cell-level and beam-level), a reference 6G measurement model and a mobility framework for comparison are essential.

### Adaptation of measurement procedure

Use of AI/ML for dynamic adaptation of measurement procedure, spatial domain Rx beam sweeping factor reduction and enhancement to SCell related measurements were also discussed in the last meeting. To evaluate and identify benefits AI/ML can bring to adaptation of measurement procedures at UE in 6G, corresponding non-AI counterpart first needs to be agreed or at least clarified. We do not think 5G can be considered as baseline for evaluation purposes as the measurement procedure for 6G may not be as defined for 5G. Without such details being discussed or clarified, it is hard to assess whether RAN4 can already start a study on these use cases or RAN4 should wait for progress in other working group before a study can eventually start in RAN4.

To identify benefits AI/ML can bring to measurement procedures at UE in 6G, corresponding non-AI/ML counterpart needs to be agreed first.

### Other use cases

AI/ML based RLM/BFD/CBD use cases were also discussed in the last RAN4 meeting. The advantage with these procedures is that they are rather stable, and not so many enhancements have been proposed for them in 6G (compared to 5G), hence such AI/ML based use cases would not be competing in gains with non-AI/ML based enhancements, which can simplify the AI/ML studies a lot. However, the objectives and candidate enhancement details need to be further discussed to agree on the scope.

Further discuss the details for candidate AI/ML based RLM/CBD/BFD enhancements.

AI/ML based RLM, BFD, CBD, enhancements for s-measurement mechanism were also discussed in the last meeting. Since reference signals for these use cases are yet to be agreed upon for 6G and given the lack of defined study objectives and evaluation baselines, we believe it is premature to start discussing AI/ML-based methods for these use cases, even though they fall within RAN4’s scope.

Reference signals for 6G are yet to be agreed upon.

Study objectives and evaluation baselines for s-measure mechanism should be thoroughly discussed before study on these use cases can be agreed.

# Summary

In this contribution AI/ML use cases for 6G are discussed. The observations and proposals below summarize the discussion presented in this paper.

AI/ML for DPoD

- RAN4 should study AI based UL-Post distortion compensation at BS receiver and understand the feasibility of creating RAN4 core and performance requirements.

- Advanced DPD can be energy- and computation-intensive for UEs, making it a potentially challenging solution for UL transmissions to avoid power back-off.

RAN4 to clarify the extent to which 6G UEs may support digital pre-distortion, which could further help to evaluate UE transmitter model in the study.

In the candidate sub-use cases, we propose prioritizing “NW: AI-DPoD” for the RAN4 study.

Accurate modeling of UEs PA and RF-front end is necessary for this feature. However, transmitter characteristics have many variations due to other RF impairments, frequency range, RB allocation etc. Therefore, UE RF front-end needs to be evaluated by RAN4, so that reliable and representative data from the RF front-end can be obtained to effectively train the AI/ML model.

RAN4 needs to study feasibility on how to train AI/ML algorithms with variety or subsets of UE transmitter models.

There is no reference study on NW-sided DPoD, therefore both AI and non-AI approach would be needed for performance and complexity evaluation.

We propose to study UE transmitter model in the UE RF session initially and further evaluate the non-linearity compensation in both AI thread (for AI-based approach) and BS Demodulation thread (for non-AI approach).

RAN4 could consider the extent of adjusting the UE EVM requirements but also evaluate any additional constraints or dependencies affecting this limit such as compensating the increased distortion at the receiver.

The practical gain of DPoD is limited to in-channel distortion and not unwanted emissions.

AI/ML-based uplink PA digital post-distortion (DPoD) could in certain conditions achieve performance similar to that of ideal PA without power backoff, resulting in a potentially higher UE output power and/or more power efficient operation for higher order modulations.

For AI-based NW-sided DPoD, we observe reasonable gain under high-SNR and high modulation order conditions with a simple UE PA model and complete investigation with realistic UE-Tx model is needed.

To characterize multiple UE transmitters, either a fine-tuned offline model or less precise and adaptive online approach can be feasible, considering the UE power consumption needed for model training and potential latency issues.

Offline trained AI/ML models may need to be updated in a certain time period to include new UE transmitter models being implemented by UEs.

RAN4 may study which AI/ML algorithms are feasible for modeling UE transmitter characteristics, and which type of model training would be feasible.

RAN4 to study data collection methods for AI/ML model training, focusing on UE operation in the compressed (non-linear) PA region. Both simulation-based datasets (with realistic Tx front-end and PA models) and measurement-based datasets (from multiple UEs) could be evaluated. Other RF impairments should be considered and included.

RAN4 to evaluate RF front-end variation from multiple UEs in the market into account and define suitable AI/ML model for UL post-distortion studies.

The AI/ML model used for DPoD should be trained effectively to handle the UE front-end variation.

RAN4 to evaluate on how to define a reference AI/ML model for evaluating AI/ML DPoD performance and ensuring comparability across different implementations.

As part of the study, RAN4 should decide the appropriate complexity level of the reference AI/ML model.

RAN4 to evaluate the potential MPR reduction enabled by AI-DPoD and its  impact on UL coverage and UE transmit energy efficiency.

In order to properly evaluate the potential MPR reduction, an up-to-date UE and MPR modelling approach may be needed to capture the true benefits.

RAN4 to evaluate the extent to which UE EVM requirements can be adjusted when AI-DPoD compensates for PA and other RF components non-linearity at the base station.

The current UE EVM requirements and EVM measurement procedure including equalizer may need updating to account for factors such as the UE front-end characteristics and the interference environment, in order to more accurately reflect real-world performance.

UE may adjust EVM and reduce MPR only under explicit network control; otherwise, existing RF requirements apply.

A common UE transmitter model with non-linearities should be evaluated in UE RF session, which could further be used for different AI and non-AI DPoD model.

AI-SRS-assisted channel reconstruction

Further investigation could be on “training-data consistency”, “reporting strategy” and “performance vs complexity trade-off”

Further clarification is needed on:

- Benefit of IL-range reporting method vs zero-reporting method

- The level of UE implementation transparency for IL-range reporting method

- How to align the AI model performance

- Detailed parameter assumptions

AI-enabled PRACH receiver

Based on the agreement in RAN4#116bis meeting for RAN4 driven use-case selection, RAN4 may prioritize those use-cases which have minimum to no impact on other WGs.

The AI/ML-enabled PRACH receiver use case depends on new random-access procedures, multi-user detection mechanisms, and potential updates to PRACH preamble design, which are within RAN1’s scope. Current RAN4 requirements only address single-user PRACH detection and cannot assess performance for multi-user collision scenarios.

RAN4 may not prioritize this use-case at this stage and should be driven by RAN1, and its study outcome could be further considered for creating RAN4 performance requirements.

RRM use cases

RAN4 should avoid overlap between the use cases being studied in RAN4 and in other working groups.

Use cases with expected gains over non-AI counterpart shall be prioritized for RAN4 study.

RAN4 should preferably avoid parallel studies for AI/ML and non-AI/ML enhancements for the same RRM procedure or feature. Non-AI/ML Rel-20 6G enhancements (over 5G) can be prioritized over AI/ML based enhancements, since the former will be the reference and widely implemented.

AI/ML for beam management is being discussed in RAN1. RAN4 to avoid having parallel discussion on AI/ML for L1 measurement prediction that is relevant for beam management in 6G.

To quantify the benefits AI/ML can provide for L3 measurement prediction (both cell-level and beam-level), a reference 6G measurement model and a mobility framework for comparison are essential.

To identify benefits AI/ML can bring to measurement procedures at UE in 6G, corresponding non-AI/ML counterpart needs to be agreed first.

Further discuss the details for candidate AI/ML based RLM/CBD/BFD enhancements.

Reference signals for 6G are yet to be agreed upon.

Study objectives and evaluation baselines for s-measure mechanism should be thoroughly discussed before study on these use cases can be agreed.

# Appendix

Table 2: Common training and evaluation assumptions for AI-DPoD.

| Parameters | Value |
| --- | --- |
| Waveform | DFT-S-OFDM |
| Carrier frequency | 30 GHz |
| PA backoff values | 4 dB, 6 dB, 8 dB |
| PA type | CMOS (memoryless) [3] |
| Number of RBs | 60 |
| Subcarrier spacing (kHz) | 120 |
| Link adaptation | Active with MCS index table 2 in TS 38.214 |
| BS antenna configuration | [4,8,2] |
| FFT size | 1024 |

Table 3: Varying parameters used for evaluation and training dataset generation of AI-DPoD.

| Parameters | Evaluation dataset | Training and validation dataset |
| --- | --- | --- |
| Number of slots | 10000 | 200000 |
| Channel model | CDL-B, C, D | CDL-A |
| Delay spread (ns) | 100 | 30-150 |
| UE speed (km/h) | 3-30 | 3-30 |
| MCS | LA (10% target BLER) | MCS-17 (64QAM) MCS-22 (256QAM) |
| SNR (dB) | -6:2:20 | -7:5 (64QAM) 1:20 (256QAM) |

# References

R4-2514637, WF on 6G AI, Samsung.

R1-2508118, Moderator summary #5 on AI/ML for 6GR, Samsung (Moderator).